MODAL DECOMPOSITION OF INSTABILITIES AND ITS APPLICATIONS IN ANALYSIS AND DESIGN OF COLD-FORMED STEEL STRUCTURES

A thesis submitted for the degree of

Doctor of Philosophy

In the Faculty of Engineering of

The University of Sheffield

by

XILIN LI

Department of Civil and Structural Engineering

The University of Sheffield

August, 2017

# ABSTRACT

The construction industry has witnessed a widespread of cold-formed steel structures (CFS) in the recent years. However, despite its advantages due to the thin-walled nature of such kind of structures they are highly susceptible to instabilities including coupled instabilities. Currently the modal decomposition method used to analyse these coupled instabilities still has space of improvement. For example, the method generates overly rigid distortional and global modes, and an increased computational effort is needed when dealing with complex cross-sections. Regarding the design of CFS, the existing design code is cumbersome to implement since different empiral formulas are proposed for different types of buckling. In addition, an overly conservative prediction of the ultimate load is obtained for sections where there is significant disparity between the slenderness values of different parts.

The aim of the project is to improve the modal decomposition methods and to exploit its application aspects in the analysis and design of CFS undergoing coupled instabilities.

First, two new modal decomposition methods are developed, namely the polarisation method and the nodal force method. Both of the methods are based on the finite strip method (FSM), and are aimed at overcoming the disadvantages of the existing method.

Second, the nodal force method is applied to analyse the deformed shape of CFS columns in the post-buckling stage, in which the contributions of buckling modes are traced until the column fails. This lays a foundation for the development of design approaches dealing with coupled instabilities. In addition, the modal decomposition method is applied to the statistical analysis of geometric imperfections of CFS specimens, which leads to a scientific charactorisation of geometric imperfections.

Finally, a design approach is developed based on the concept of the direct strength method (DSM). In the proposed design approach, the initial post-buckling stiffness is considered as an input parameter in addition to the critical load. These input parameters are then used to establish a multi-linear model approximating the elastic post-buckling behaviour, which is linked to the ultimate load using empirical formulas. In the proposed design approach, two improvements are made. First, a unified design formula is used for different types of buckling, which are distinguished by the difference in post-buckling stiffness. Second, the overly conservative prediction of the ultimate load, when one part of the section is significantly slender, is avoided. This is due to the fact that in these cases, the reduction of the stiffness when buckling occurs is comparably minor.

# PUBLICATIONS

Conference papers:

Becque J & Li X (2016) Modal decomposition of buckled shapes: the method of the equivalent forces. SEMC 2016: 6th International Conference on Structural Engineering, Mechanics and Computation, Cape T, 5 September 2016 - 7 September 2016.

Li X & Becque J (2016) Modal decomposition of instabilities in thin-walled structural elements. The Annual Postgraduate Research Conference, 10 May 2016 - 10 May 2016.

Becque J & Li X (2014) The polarization method for modal decomposition of buckled shapes. Proceedings of the 7th International Conference on Thin-Walled Structures. South-Korea, 28 September 2014 - 2 October 2014.

# ACKNOWLEDGMENTS

This work would not have been possible without the financial support of the university scholarship at the University of Sheffield and this is also gratefully acknowledged.

I have been staying in Sheffield for five years since the start of my MSc study in 2013. My family has a history with Sheffield and the university. Xun Li, the brother of my grandfather, got a PhD degree in material science and engineering in this university in the 1940s. After graduation he returned to china and devoted his life to the development of Chinese industry. He had set up a role model in my family, which was part of the reason that I decided to do an engineering PhD in Sheffield. I really enjoyed the life in Sheffield, for it is a safe and beautiful city. Besides, the advanced university facilities and warm-hearted staff have offered convenience to my study.

I would like to express my deepest gratitude to my supervisor Dr. Jurgen Becque and Prof. Buick Davison. Your guidance, inspiration and continuous encouragement made this thesis possible.

Thank you to all my colleagues in the research group of steel structures. It was a pleasure to work with your guys. Thank you to Jun Ye and Francisco Meza Ortiz for the support and help provided.

To my family: thank you for your support and encouragement.

# TABLE OF CONTENTS

ABSTRACT	I

PUBLICATIONS	II

ACKNOWLEDGMENTS	III

TABLE OF CONTENTS	IV

LIST OF FIGURES	IX

LIST OF TABLES	XV

CHAPTER 1. Introduction	1

1.1. Background	1

1.2. Necessity of research	3

1.3. Aims and objectives	4

1.4. Thesis layout	4

CHAPTER 2. Literature Review	7

2.1. Instability types involved in cold-formed steel structures	7

2.2. Local buckling	8

2.3. Global buckling	9

2.4. Distortional buckling	10

2.5. Finite strip method	11

2.6. Interaction among buckling modes	13

2.7. Linear interaction	13

2.7.1. Generalised beam theory (GBT)	13

2.7.2. Constrained finite strip method (cFSM)	15

2.8. Non-linear interaction	16

2.9. Initial post-buckling stiffness	18

2.10. Secondary bifurcation	19

2.11. Geometric imperfections	21

2.12. Design approaches for coupled instabilities	23

2.12.1. Effective width method	23

2.12.2. Direct strength method	24

2.13. Experimental studies	27

CHAPTER 3. The polarisation method of modal decomposition	34

3.1. Introduction	34

3.2. Introduction to the polarisation method	37

3.2.1. Number of modes	38

3.2.2. Orthogonality of modal shapes	40

3.3. Modal identification	40

3.3.1. Modal identification step 1	40

3.3.2. Modal identification step 2	42

3.3.3. Modal identification step 3	45

3.4. Modal decomposition	46

3.5. Illustrative examples	46

CHAPTER 4. The nodal force method of modal decomposition	58

4.1. Introduction to the nodal force method	58

4.2. Modal identification	59

4.2.1. Local modes	59

4.2.2. Distortional modes	62

4.2.3. Global modes	65

4.2.4. Transverse extension and shear modes	66

4.3. Illustrative examples	69

CHAPTER 5. Comparison between the modal decomposition methods	77

5.1. Introduction	77

5.2. Comparison between the newly proposed methods	77

5.2.1. Similarities	77

5.2.2. Differences	78

5.3. Comparison between the newly proposed and existing methods	80

5.3.1. Mode generation	80

5.3.2. Orthogonality	80

5.3.3. Scope of applicability to various cross-sections	82

5.3.4. Constitutive relations	82

5.3.5. Membrane strains and stresses	82

5.4. Case studies	83

5.4.1. Lipped channel section	83

5.4.2. Single-cell box section	88

5.4.3. Hollow-flange channel section	92

5.4.4. Summary	96

CHAPTER 6. Modal decomposition of post-buckling deformed shapes	97

6.1. Introduction	97

6.2. Transformation from FSM to FE modes	98

6.2.1. Determination of line loads in the FE model	99

6.2.2. Determination of the nodal loads in the FE model	103

6.2.3. Generating FE modes	105

6.2.4. Orthogonalisation and normalisation	107

6.3. Modal decomposition of post-buckling deformed shapes	108

6.3.1. Determination of modal participation	108

6.3.2. Boundary conditions	109

6.4. Illustrative examples	111

6.4.1. Local buckling under simple-simple boundary condition	111

6.4.2. Local-distortional interactive buckling under fixed-fixed boundary condition	114

6.4.3. Illustrative examples with general boundary conditions	118

6.5. Evaluation of the error	123

6.6. Effect of imperfection	129

6.6.1. Introduction	129

6.6.2. Local buckling	131

6.6.3. Local-distortional interactive buckling	133

6.6.4. Conclusion	135

CHAPTER 7. Modal decomposition of geometric imperfections	136

7.1. Introduction	136

7.2. Imperfection measurements	136

7.2.1. Equipment	136

7.2.2. Specimen dimensions	137

7.2.3. Imperfection measurements	138

7.3. Post-processing of the data	140

7.3.1. Assembly of the imperfect geometry	140

7.3.2. Calculation of imperfection magnitudes	144

7.3.3. Results	148

CHAPTER 8. Calculation of initial post-buckling stiffness	156

8.1. Introduction	156

8.2. Local buckling	158

8.2.1. Governing equations	159

8.2.2. Displacement functions	160

8.2.3. Potential energy	165

8.2.4. Equilibrium equation	165

8.2.5. Compressive force	166

8.3. Distortional buckling	167

8.3.1. Governing equations	167

8.3.2. Displacement functions	168

8.3.3. Potential energy	170

8.3.4. Equilibrium equation	171

8.3.5. Compressive force	171

8.4. End boundary conditions	171

8.5. Programme development	173

CHAPTER 9. Elastic post-buckling behaviour	177

9.1. Introduction	177

9.2. Local-local interactive buckling	177

9.2.1. Approximating model	178

9.2.2. Verification against numerical results	180

9.3. Local-distortional interactive buckling	188

9.3.1. Introduction	188

9.3.2. Approximating model	189

9.3.3. Verification against numerical results	194

9.3.4. Verification against experimental results	201

CHAPTER 10. DSM equations for CFS column	202

10.1. Introduction	202

10.2. Design approach	202

10.2.1. Design formula	202

10.2.2. Determination of the function g	205

10.2.3. Predicting ultimate loads	210

CHAPTER 11. Summary and recommendations for future work	221

11.1. Summary	221

11.2. Recommendations for future work	224

REFERENCES	226

APPENDIX A. Python Script generating FEM buckling modes	234

APPENDIX B. Imperfection modal magnitudes	250

APPENDIX C. Total potential energy expression for the initial post-buckling analysis	257

APPENDIX D. Modal shapes	267

APPENDIX E. Verification of the multi-linear model	278

# LIST OF FIGURES

Fig. 1.1 Cold-formed steel structural members	1

Fig. 1.2 Effective width method	2

Fig. 2.1 Buckling modes	8

Fig. 2.2 Definition of node types	11

Fig. 2.3 Coordinate systems and displacements	11

Fig. 2.4 The van der Neut Curve (excerpt of [94])	16

Fig. 2.5 Load-end shortening curve of a buckled column	18

Fig. 2.6 Secondary bifurcation	20

Fig. 2.7 Illustration of the Kutanova/Mennink model (excerpt of [42])	21

Fig. 2.8 Points representative of imperfection magnitudes	22

Fig. 2.9 Bending resistance of a lipped channel section	24

Fig. 2.10 Sections tested by Tomasson	27

Fig. 2.11 Sections tested by Lau et al.	28

Fig. 2.12 Sections tested by Kwon et al.	29

Fig. 2.13 Sections tested by Kwon and Hancock	31

Fig. 2.14 Sections tested by Yang and Hancock	31

Fig. 2.15 Sections tested by Young and Yan	32

Fig. 3.1 Linear mode interaction	35

Fig. 3.2 Membrane and bending components	35

Fig. 3.3 Shear modes obtained from cFSM	37

Fig. 3.4 Transverse extension modes obtained from cFSM	37

Fig. 3.5 Cross square section and its analogous truss	39

Fig. 3.6 Transformation from global to local displacements	41

Fig. 3.7 Lipped channel section	44

Fig. 3.8 Critical stress and modal participation	48

Fig. 3.9 Lipped channel section with rounded corners	48

Fig. 3.10 Critical stress and modal participation	50

Fig. 3.11 Double-cell box section	50

Fig. 3.12 Critical stress and modal participation	52

Fig. 3.13 Hollow-flange channel section	52

Fig. 3.14 Critical stress and modal participation	54

Fig. 3.15 Single-cell box section	54

Fig. 3.16 Critical stress and modal participation	55

Fig. 3.17 Multi-cell square section	55

Fig. 3.18 Critical stress and modal participation	57

Fig. 4.1 Nodal line loads	58

Fig. 4.2 Transformation from local to global nodal forces	61

Fig. 4.3 Lipped channel section	61

Fig. 4.4 Local modal shapes and nodal forces	62

Fig. 4.5 Distortional modal shapes and nodal forces	64

Fig. 4.6 Global modal shapes and nodal forces	66

Fig. 4.7 Transverse extension modal shapes and nodal forces	68

Fig. 4.8 Shear modal shapes and nodal forces	68

Fig. 4.9 Critical stress and modal participation	70

Fig. 4.10 Double-cell box section	70

Fig. 4.11 Critical stress and modal participation	71

Fig. 4.12 Hollow-flange channel section	72

Fig. 4.13 Critical stress and modal participation	73

Fig. 4.14 Single-cell box section	73

Fig. 4.15 Critical stress and modal participation	74

Fig. 4.16 Cross square section	74

Fig. 4.17 Critical stress and modal participation	76

Fig. 5.1 Flexural global mode	79

Fig. 5.2 Shear flow and equivalent nodal force configuration	80

Fig. 5.3 Orthogonality among buckling modes in the cFSM	81

Fig. 5.4 Lipped channel section	84

Fig. 5.5 Distortional buckling critical stress of lipped channel section	87

Fig. 5.6 Global buckling critical stress of lipped channel section	88

Fig. 5.7 Single-cell box section	89

Fig. 5.8 Distortional buckling critical stress of single-cell box section	91

Fig. 5.9 Global buckling critical stress of single-cell box section	92

Fig. 5.10 Hollow-flange channel section	92

Fig. 5.11 Distortional buckling critical stress of hollow-flange channel section	93

Fig. 5.12 Global buckling critical stress of hollow-flange channel section	94

Fig. 5.13 GBT modes	94

Fig. 5.14 Switch of dominant modal shape	96

Fig. 6.1 DOFs involved in FS and FE models	98

Fig. 6.2 Calculation of local rotations	99

Fig. 6.3 Nodal line loads	99

Fig. 6.4 Equilibrium at strip junction	102

Fig. 6.5 Transformation from line loads to nodal forces	104

Fig. 6.6 Boundary conditions applied to the FE model	105

Fig. 6.7 Flow chart of the mode generation process	106

Fig. 6.8 FE buckling mode	107

Fig. 6.9 Boundary conditions applied to the FE model	111

Fig. 6.10 Single-cell box section	111

Fig. 6.11 Cross-sectional deformations of selected buckling modes	112

Fig. 6.12 Modal participations	113

Fig. 6.13 Lipped channel section	115

Fig. 6.14 Cross-sectional deformations of selected buckling modes	115

Fig. 6.15 Modal participations (fixed-fixed)	117

Fig. 6.16 Modal participations (fixed-fixed)	119

Fig. 6.17 Shifting of the deformed shape	120

Fig. 6.18 Boundary conditions applied to the FE model	120

Fig. 6.19 Modal participations (fixed-free without end plate)	121

Fig. 6.20 Modal participations (fixed-free with end plate)	122

Fig. 6.21 Final deformed shapes	123

Fig. 6.22 Error when only conventional modes are involved	125

Fig. 6.23 Comparison between the total displacement magnitudes	125

Fig. 6.24 Comparison between the warping displacements	126

Fig. 6.25 Comparison between the total displacement magnitudes	126

Fig. 6.26 Comparison between the warping displacements	127

Fig. 6.27 Error when transverse extension and shear modes are included	128

Fig. 6.28 Error of the examples with general boundary conditions	129

Fig. 6.29 Imperfection shapes as combinations of buckling modes	130

Fig. 6.30 Longitudinal shapes of modes constituting imperfection shapes	130

Fig. 6.31 Modal participation factors subject to imperfection	132

Fig. 6.32 Modal participation factors subject to imperfection	134

Fig. 7.1 Imperfection measurement set-up	137

Fig. 7.2 Definition of notations	138

Fig. 7.3 Imperfection measurements	139

Fig. 7.4 Imperfection measurement locations along cross-section	140

Fig. 7.5 Assembly of the imperfect geometries	144

Fig. 7.6 Calculation of node rotations	144

Fig. 7.7 Sampling of imperfection and nodal forces	147

Fig. 7.8 Effectiveness of the pre-processing process	147

Fig. 7.9 Modes involved in the modal decomposition of imperfection shape	148

Fig. 7.10 Critical half-wavelengths determined from the signature curve	149

Fig. 7.11 Comparable imperfection data	150

Fig. 7.12 Imperfection modal magnitudes of section T12012	151

Fig. 7.13 Imperfection modal magnitudes of various sections	152

Fig. 7.14 Mean and variance of imperfection modal magnitudes	154

Fig. 7.15 Statistical distribution of the phase angle	154

Fig. 7.16 ‘Folded’ Statistical distribution of the phase angle	155

Fig. 8.1 Coordinate systems and displacements	158

Fig. 8.2 Boundary conditions for distortional buckling	170

Fig. 8.3 Boundary conditions	172

Fig. 8.4 Shift of centroid	173

Fig. 8.5 Main flow chart of the programme	175

Fig. 8.6 Flow chart of the m-file IniPost.m	176

Fig. 9.1 Multi-linear model with involvement of two modes	178

Fig. 9.2 Critical half-wavelengths of local modes	179

Fig. 9.3 Verification of the multi-linear model	186

Figure 9.4 Multi-linear model with involvement of three modes	190

Fig. 9.5 Critical half-wavelengths of local and distortional modes	191

Fig. 9.6 Simple lipped channel (CH1)	192

Fig. 9.7 Cross-sectional modal shapes of buckling modes selected	193

Fig. 9.8 ABAQUS model of distortional buckling	194

Fig. 9.9 Verification of the multi-linear model	194

Fig. 9.10 Stiffened lipped channel (CH2)	196

Fig. 9.11 Section geometry notations	197

Fig. 9.12 Verification of the multi-linear model	203

Fig. 10.1 Average post-buckling stiffness	205

Fig. 10.2 Illustration of the design formula	206

Fig. 10.3 Relation between kp and g	212

Fig. 11.1 Out-of-plane displacements of plates in contact	225

# LIST OF TABLES

Table 3.1 Number of modes	38

Table 3.2 Modal shapes	44

Table 3.3 Modal shapes	45

Table 3.4 Modal shapes	48

Table 3.5 Modal shapes	51

Table 3.6 Modal shapes	52

Table 3.7 Modal shapes	54

Table 3.8 Modal shapes	56

Table 4.1 Assumptions for different mode types	69

Table 4.2 Modal shapes	70

Table 4.3 Modal shapes	72

Table 4.4 Modal shapes	73

Table 4.5 Modal shapes	75

Table 5.1 Number of modes	78

Table 5.2 Relevant membrane strains and stresses	83

Table 6.1 Boundary conditions	109

Table 6.2 Cross-sectional deformed shapes with increasing load.	114

Table 6.3 Change of displacement pattern with increasing load	116

Table 6.4 Post-buckling deformed shapes subject to imperfection	133

Table 6.5 Post-buckling deformed shapes subject to imperfection	135

Table 7.1 Nominal dimensions of specimens	138

Table 7.2 Errors of the assembly of the imperfect geometry	142

Table 7.3 Critical half-wavelength for local and distortional modes	149

Table 8.1 Constraints on DOFs of a lipped channel section	164

Table 8.2 Constraints on DOFs of an I-section	164

Table 9.1 Buckling parameters	180

Table 9.2 Buckling modal shapes	182

Table 9.3 Verification of the multi-linear model	183

Table 9.4 Buckling parameters	184

Table 9.5 Buckling modal shapes	186

Table 9.6 Verification of the multi-linear model	188

Table 9.7 Section geometry	193

Table 9.8 Buckling parameters (independent of column length)	193

Table 9.9 Critical stresses subject to column length (fixed ends)	195

Table 9.10 Verification of the multi-linear model	195

Table 9.11 CH1 section geometry	197

Table 9.12 CH2 section geometry	197

Table 9.13 Hat section geometry	197

Table 9.14 Z-section geometry	198

Table 9.15 Rack section geometry	198

Table 9.16 Buckling parameters	199

Table 9.17 Buckling modal shapes	200

Table 9.18 Verification of the multi-linear model	201

Table 10.1 kp and g for local buckling of box-section	207

Table 10.2 kp and g for distortional buckling	209

Table 10.3 Predictions of the ultimate load of box-sections in local buckling	214

Table 10.4 Predictions of the ultimate load in distortional buckling	216

Table 10.5 Comparion between the design approaches	221

# Introduction

## Background

Cold-formed steel (CFS) structural elements are steel structural products made by bending thin flat sheets of steel at ambient temperature (Fig. 1.1). CFS structures have a history of over a century, and recently it has become widespread throughout the world thanks to the more economic production of steel coils particularly in coated form with zinc or aluminium/zinc coatings. Nowadays, CFS structures are employed in a wide range of applications, such as roof purlins, stud walls, storage racks, wall and roof cladding, and more recently even as primary load-bearing members in low to medium rise buildings.

Fig. 1.1 Cold-formed steel structural members

CFS has many advantages, such as a high strength-to-weight ratio and a relatively straightforward and versatile manufacturing process, leading to a high flexibility in the cross-sectional shapes which can be obtained. However, the manufacturing process limits their wall thickness and, as a result, they are susceptible to various instabilities. These may include coupled instabilities, where buckling modes of different types interact with each other.

In order to gain an in depth understanding of coupled instabilities, it is useful to analyse it using a modal decomposition method. In this method, the deformed shape of a buckled CFS member is decomposed into a linear combination of individual buckling modes, each with a specific mechanical significance. In doing so, it is possible to study the behaviour of each isolated buckling mode, as well as determine their contributions to the overall behaviour of the member when coupled instabilities occur.

Fig. 1.2 Effective width method

The involvement of coupled instabilities adds to the design complexity of CFS member. Current design codes for CFS, including the European Eurocode 3 and the American AISI codes, are typically based on the effective width method, which is based on the fact that after a simply supported plate buckles, the central part of the plate carries a reduced load due to the large out-of-plane displacements (Fig. 1.2). As a result, the load becomes concentrated in strips adjacent to edges of the plate. Therefore, it is assumed in the effective width concept that only a certain width of plate next to the edges plays a role in the load-carrying capacity of the member, while the contribution of the reminder of the plate is neglected. The combined width of the load-bearing parts is named ‘the effective width’. The design codes based on it are inevitably suffering from some disadvantages. First, in these design codes the effective width of each flat element of the cross-section has to be determined. As a result, the computation procedure becomes more tedious for members having complex cross-sections, such as lipped channel sections with additional return lips, and sections with stiffeners. Second, each element is considered separately, ignoring the rotational restraints provided by the adjacent elements.

With the purpose of overcoming the deficiencies, a new design methodology called the direct strength method (DSM) is proposed. In this method, the load carrying capacity of a CFS member is linked to its slenderness by empirical formulas, subject to the type of buckling it undergoes. In this regard, the DSM is deemed to be an extension of the established column design method to new types of instability. The critical stresses used to calculate the slenderness associated with different types of buckling are determined using the signature curve obtained from the FSM.

## Necessity of research

Current modal decomposition methods are suffering from a couple of deficiencies. First, they adopt the idealised assumptions for distortional and global modes, which neglect, for example, membrane transverse strains and shear strains that occur in these modes. This is especially problematic for the cFSM, where these assumptions are implemented in the context of the FSM with sufficient DOFs to account for the neglected strains resulting from the Poisson’s effect and the shear effect. Overly rigid idealised buckling modes are obtained [2] [3] [4]. Second, although these methods work well for simple sections (i.e. open sections without branches), the complexity of implementation is significantly increased when dealing with complex cross-sections (i.e. sections with closed parts or branches) [5] [6] [82]. Third, these method are unable to produce buckling modes strictly orthogonal to each other with respect to the global stiffness matrix K. In light of these, the current modal decomposition methods still have the space of improvement. In addition, the application of the modal decomposition method is currently restricted to the finite strip (FS) numerical model. However, cases exist where the finite element (FE) model rather than FSM model is preferable while still a modal decomposition is required. One of the examples is the modal decomposition of the post-buckling deformed shape. This calls for the extension of the application to the FE model. Furthermore, the modal decomposition method is also found useful for the statistical analysis of the geometric imperfections.

Regarding the design of CFS, the DSM has its limitations. First, it uses different formulas to predict the ultimate load associated with different types of buckling, causing inconvenience to the implementation of the method. Second, the member is treated differently when it undergoes pure instability or coupled instability. In addition, it gives underestimation of the ultimate load when one part of the section is extremely slender compared to the other parts [71] [73]. Therefore, improvement of the method is necessary.

## Aims and objectives

The main aim of the project is to propose a novel modal decomposition method and to extend its applications in both the analysis and design. The detailed objectives are as follows:

- To propose a new method of modal decomposition. In a first stage the new method will be implemented in the context of the FSM. The method should be consistent with the mechanical principle of the FSM, without being tied to the limiting assumptions of GBT. A MATLAB code will be developed implementing the new modal decomposition method. The software should have the capability to plot the critical stresses associated with different types of buckling against the half-wavelength, as well as visually present the modal shapes at a specific half-wavelength. The software should be able to deal with both simple and complex cross-sections with straight or rounded corners.

- To develop a framework in which the modal decomposition method can be applied to FE output. This is particularly useful in the modal decomposition of post-buckling deformed shapes.

- To apply the modal decomposition method in the statistical analysis of initial geometric imperfections.

- To determine the initial post-buckling stiffness of CFS columns using the perturbation method, and to implement this in two separate software programmes. The first software code will focus on open sections, I-sections and box sections undergoing local buckling, while the second software code is specialised in distortional buckling of open sections only.

- To develop a framework in which the signature parameters regarding the buckling and initial post-buckling behaviour obtained from the software developed in (4) are used in a multi-linear model approximating the load versus end shortening curve of a CFS column undergoing elastic buckling.

- To propose a design approach predicting the ultimate load of a CFS column using the approximating model of the elastic buckling behaviour developed in (5). The approach will be able to deal with local, distortional and global buckling as well as local-distortional interactive buckling.

## Thesis layout

The thesis consists of eleven chapters, of which a brief overview is given as follows:

Chapter 1 is introduction.

Chapter 2 presents a literature review. The first section reviews studies on the three major types of buckling involved in CFS structures, namely local, distortional and global buckling. The second section reviews the FSM, which is essential in the numerical analysis of CFS structures, followed by some basics about coupled instabilities and studies on the linear and non-linear coupled instabilities. The third section reviews studies on the calculation of the initial post-buckling stiffness, the secondary bifurcation, the geometric imperfections and the considerations of current design approaches on coupled instabilities. The last section reviews the experimental investigations on CFS.

Chapter 3 proposes the polarisation method of modal decomposition. The method is derived, implemented and validated against a variety of sections. Results obtained from the method are presented including critical stresses and modal shapes.

Chapter 4 proposes the nodal force method of modal decomposition. The method is derived, implemented and validated against a variety of sections. Results obtained from the method are presented including critical stresses and modal shapes.

Chapter 5 makes a comparison between the newly proposed and existing methods. The differences on the characteristic features and the modal decomposition results are unveiled and explained.

Chapter 6 discusses the further application of the modal decomposition method to post-buckling deformed shapes of CFS columns, whereby the participation factors of the buckling modes are traced beyond the critical point during the loading process. In addition, the sensitivity of the participation factors to the imperfection is studied.

Chapter 7 discusses the application of the modal decomposition method in analysing the measured geometric imperfections, leading to a more scientific modelling of the imperfections based on statistical data.

Chapter 8 discusses the calculation of the initial post-buckling stiffness of CFS columns using the perturbation method. The whole derivation of the method is given. Initially in the first part of the chapter the method focuses on local buckling but its application is then extended to distortional buckling in the second part. Two boundary conditions are discussed where the shift-of-centroid effect can either be considered or neglected.

Chapter 9 develops a multi-linear model approximating the load-end shortening curve of a CFS column undergoing elastic buckling. The signature parameters used in the model are obtained from the buckling and initial post-buckling analysis discussed in the previous chapters. The model is then verified against box sections and I-sections undergoing local buckling, and a variety of simple sections undergoing distortional buckling or local-distortional interactive buckling.

Chapter 10 develops a design approach predicting the ultimate load of a CFS column using the approximating model of the elastic buckling behavior. The relation is found between the average post-buckling stiffness and the residual load carrying capacity after buckling, which is applied in the design approach to bond the plastic buckling behaviour to elastic buckling behaviour. Finally the effectiveness of the approach is assessed using the ultimate load results obtained from both numerical modelling and experimental investigation.

Chapter 11 summarises the whole work and gives recommendations for future work.

# Literature Review

## Instability types involved in cold-formed steel structures

Due to the thin-walled nature of CFS structures they are highly susceptible to instabilities (buckling), which are commonly categorised into different types based on their mechanical features. Early research distinguished between three types of buckling, namely local, distortional and global. Although there exists no full consensus on the definition of these types of buckling, commonly cited ones are given by Schafer [68]:

- Local buckling is normally defined as the mode which involves plate-like deformations alone, without the translation of the intersection lines of adjacent plate segments.

- Distortional buckling is a mode with cross-sectional distortion that involves the translation of some of the intersection lines of adjacent plate segments.

- Global buckling is a mode where the member deforms with no deformation in its cross-sectional shape, consistent with classical beam theory.

In the above definition the term ‘plate segments’ refers to the flat plates constituting CFS structural cross-section. For instance, for a CFS member with a lipped channel cross-section, the cross-sectional displacements (see section 2.5 for definition) of different buckling modes are illustrated in Fig. 2.1. The original and deformed cross-sections are presented in blue and red lines, respectively.

| Local | Distortional | Global |  |
| --- | --- | --- | --- |
|  |  | Torsional | Flexural |

Fig. 2.1 Buckling modes

Among the three types of buckling, local and global buckling also occur in traditional hot-rolled or welded steel structures, and have more extensively been studied than distortional buckling.

## Local buckling

Local buckling of CFS structures is closely related to the buckling problem of individual plates. The differential equation describing elastic buckling of a plate subject to in-plane loading was first derived by St. Venant [66]. When no transverse load is imposed on the plate, the equation reads:

where D is the flexural rigidity of the plate, w is the out-of-plane deflection, t is the thickness, σx, σy, τxy are the load membrane stresses. This equation forms the basis of the eigenvalue problem describing the elastic buckling of the plate. It was then noticed that the load membrane stresses were varying at the post-buckling stage due to the stretching of the plate. In light of this, Föppl [28] introduced the stress function Φ which paved the way for von Kámán [95] to derive the governing equation for perfect flat plates:

His theory was then generalised by Maguerre by considering initial imperfections and curved plates. The St. Venant’s equation was solved for the first time by Bryan [18] in close form for simply supported plates and then by Timoshenko [91] for generalised boundary conditions. The von Kámán’s equation was solved by Yamiki [102] for generalised boundary conditions. The equation was also solved for simply supported plates by Rhodes and Harvey [64] based on the energy methods, and by Walker [99] using the perturbation method.

Local buckling is characterised by a large residual capacity after buckling especially when buckling occurs in the purely elastic range, less so for plates buckling close to the yield load. The ultimate capacity can be determined on the basis of von Kámán’s effective width method. Winter [101] proposed a formula to calculate the effective width be of a compressed plate simply supported along the unloaded edges:

where b is the plate width, t is the plate thickness, E is the Young’s modulus, fmax is the ultimate compressional stress, The formula was later modified by the same researcher [100] to improve the accuracy by slightly changing the factor 0.475 to 0.415:

## Global buckling

As cited by Timoshenko and Gere [90], Euler pioneered solving the flexural global buckling problem of column using the equilibrium differential equation:

where EI is the flexural rigidity, w is the lateral deflection, and P is the concentric load. The critical load is calculated as:

where Le is the effective length, which is a function of the column length and the boundary conditions. Thereafter, it was noticed that columns with open thin-walled sections due to their small torsional rigidities tend to bend and twist simultaneously under axial compression. As cited by Chai H Yoo [19] and Wai-Fah Chen [98], in 1929 Wagner did a pioneer work on torsional buckling of open thin-walled sections, followed by Wagner and Pretschner [96] and Kappus [40] focusing on flexural-torsional buckling. Goodier [32] then extended the governing differential equations to include beam-columns under biaxial bending with identical loading conditions at each end. All of these works are based on the assumption that while warping is allowed, the geometric of the cross-section is unchanged during buckling, which is consistent with the definition of global buckling. These theories were summarised by Bleich [17] and Timoshenko [90].

To take into account the plasticity of the material, in 1889 Engesser proposed the tangent modulus approach, where Eq.  is modified to Eq. :

where Et is the tangent modulus of the material. In this approach Engesser assumed that inelastic buckling occurs with no increase in load. However, the second-order moment of the load will produce incremental strains that will vary linearly and have a zero value at the centroid. The linear strain variation will have compressive and tensile values. The tangent modulus of the incremental compressive strain is equal to Et and that for the tensile strain is E. In 1898 Engesser corrected his original theory by accounting for the different tangent modulus of the tensile increment, leading to the new reduced modulus method. For decades engineers were faced with the dilemma that the tangent modulus method is closer to the experimental results while the reduced modulus method is correct. This dilemma was eventually resolved by Shanley [78].  He found that lateral deflection starts at a load very close to the critical load given by the tangent modulus method. However, as the lateral deflection develops, the load is increasing proportionally, approaching eventually (but never reach) the critical load given by the reduced modulus method.

## Distortional buckling

As cited by Schafer and Hancock [69], distortional buckling was first encountered by Thomasson [89], Desmond and Mulligan in the 1970s when studying local buckling of cold-formed steel members. However, most of them regarded distortional buckling as undesirable and failed to recognise its importance as a buckling type distinctive from local buckling. Thus it was not until the 1980s that researchers began focusing on distortional buckling. Lau and Hancock [46] studied the distortional buckling of storage rack columns, for which the optimised nature leads to the dominance of distortional buckling over local buckling. Hancock also contributed to the popularisation of the finite strip method as a numerical tool to understand buckling behaviour, especially distortional buckling behaviour, of CFS members. In addition, prediction methods for the elastic distortional critical stress based on hand calculations were proposed by Lau and Hancock [45].

Kwon and Hancock [43] performed more experimental research on distortional buckling and discovered that its post-buckling residual capacity is lower than for local buckling. They also proposed new column strength curves for distortional buckling [35], which later led to the development of the direct strength method (DSM). In parallel, Sean and Rhodes [77], according to their own experimental results, proposed a different design approach for distortional buckling based on the effective width method.

## Finite strip method

Fig. 2.2 Definition of node types

The finite strip method (FSM) can be seen as a specialised finite element method (FEM), specifically developed to study buckling of prismatic thin-walled structural members. According to the summary by Cheung [21], the method differs from the FEM in that only a cross-sectional discretisation is required, leading to the emergence of ‘strips’ rather than elements. As a result, the number of degrees of freedom (DOF) involved in the analysis is significantly reduced, thus improving computational efficiency. The strips are separated by nodal lines, which are often simply referred to as ‘nodes’. At the very minimum, a cross-section needs to be discretised into a number of flat plate segments connected at main nodes, although each plate segment can be further discretised into smaller strips at sub-nodes. Main nodes associated with only one strip are termed external main nodes, as opposed to internal main nodes which connect multiple strips. The definition of the various types of nodes is illustrated in Fig. 2.2.

(a)                                                                          (b)

Fig. 2.3 Coordinate systems and displacements

In the FSM, a local coordinate system is defined for each strip, as well as one global coordinate system shown in Fig. 2.3(a). In the global coordinate system, each of the nodal lines is associated with the following four DOFs:  the translations U in the X -direction measured at the specimen ends (which are assumed to be equal and opposite), in addition to the translations V and W in the global Y - and Z -directions and the rotation Θ about the X -axis, all measured at mid-length. In the local coordinate system associated with each strip, the corresponding DOF are the translations u in the x -direction measured at the specimen ends, as well as the translations v and w in the local y - and z -directions and the rotation θ about the x -axis, all measured at mid-length. In what follows, the displacements u, v, w will also be referred to as the warping, transverse and out-of-plane displacements, respectively. In addition, the displacements v and w are often termed ‘cross-sectional displacements’. The notations regarding local and global displacements are shown in Fig. 2.3(b) for a strip with length L and width b.

In the FSM, the longitudinal shape functions are assumed to be trigonometric. Thus for a thin-walled member with pinned boundary conditions, the global displacements of the i th nodal line (subscript i) can be expressed as:

where m is the number of half-waves within the member length L. ,  and  are the amplitudes of the displacement values, and  is the amplitude of the rotation.

Similarly, the local displacements within the i th strip (superscript i) can be expressed as:

where, and are functions which allow interpolation between the local DOFs at the nodal lines adjacent to the strip. The functions and  take on a linear form, while  is a third degree polynomial.

In the FSM, the buckling modes are obtained as the solutions of an eigenvalue problem:

where d is the deformed (buckled) shape, and K and G are the elastic and geometric stiffness matrices, respectively.

## Interaction among buckling modes

When two or more buckling modes contribute simultaneously to the overall buckling behaviour of a member, interaction occurs among these buckling modes. There are two types of interactions [37], being:

- Linear interaction where the interaction among buckling modes occurs at the same half-wavelength.

- Non-linear interaction where short half-wavelength modes occur over multiple half-wavelength and interact with long half-wavelength modes.

Linear and non-linear interactions occur at the critical point among buckling modes having close critical loads. In addition, non-linear interaction could also occur at the post-buckling stage due to secondary bifurcation, where a buckling mode different from the original one emerges and start participating in the overall buckling behaviour. In this case, the closeness among the critical loads is not necessary for the interaction [26].

## Linear interaction

Linear interaction among buckling modes can be analysed by a modal decomposition method, which defines the buckling modes of different types and then decomposes the deformed shape into a combination of these buckling modes. A number of such methods have been developed, including primarily the generalised beam theory (GBT) and the constrained finite strip method (cFSM).

### Generalised beam theory (GBT)

The generalised beam theory (GBT) is a semi-numerical method specialised in analysing prismatic thin-walled structural members and was invented by Schardt [76] and introduced to the English-speaking world by Davies [23]. As suggested by its name, in its original form it is an extension of the classic beam theories where Vlasov’s assumptions are implemented:

- The membrane transverse strain εyM is neglected.

- The membrane shear strain γxyM is neglected.

Based on these assumptions, the cross-sectional displacements and rotations are uniquely linked to the warping displacements, which are consequently identified as the only independent DOFs.

Each independent DOF is associated with a displacement function notated as kV for the k th DOF. The total internal virtual work of the system is expressed in terms of kV and its second and fourth derivatives with respect to the longitudinal coordinate, kV ″ and  kV ‴′. Finally, by setting the variation of the virtual work equal to zero, the fundamental equation of GBT is obtained:

where V = [1V, …, kV]T, q is virtual work done by the external loads and C, D and B are matrices which depend on the cross-sectional geometry of the member. In Eq.  a coupled system of differential equations is defined, which cannot be solved until a decoupling process is carried out, where matrices C, D and B are diagonalised.  In this process V is converted to , which contains the re-orthogonalised displacement functions or modal shapes. Thus Eq.  is transformed to a group of uncoupled differential equations:

where nDOF is the number of independent DOFs, which equals the number of GBT modes.

As a result of the two initial assumptions, the application of the early GBT was limited to open sections and only Vlasov modes (i.e. modes satisfying the Vlasov’s assumption, also referred to as conventional modes) were included. However, more recently work by Gonçalves et al. [31], and Simão and Simões [84] has extended its application to closed sections and to complement the set of modes by unconventional modes such as the torsional mode of sections with closed parts, the transverse extension modes and the shear modes. In addition, early GBT was only able to deal with elastic prismatic members, while more recently GBT has been applied by Gonçalves and Camotim [29][30] to the analysis of stainless steel and aluminium columns accounting for plasticity, and by Silvestre [83] to the analysis of curved-wall members.

Among the applications of GBT in the analysis of thin-walled structures, modal decomposition is one of great significance. The first approach was developed by Schardt himself [75], and is based on the fact that when an external load corresponding to one buckling mode is imposed on the modal shape of another bucking mode, it will generate work as a result of second-order effects. This leads to an additional term on the left-hand side of Eq.  as the second-order effect of the external load, which, along with the first-order effect q, constitutes the total contribution of the external load.

where iW is the warping stress resultant in the i th mode, and ijkκ are terms reflecting the interactions between buckling modes. iW are the only unknowns in the equation, and the solution would be the critical load when multiple modes are involved in the buckling behaviour. More recently, an energy-based approach was developed by Simão and Simões [84]. In this method, the total potential energy function is expressed as a polynomial with up to fourth-degree terms. The critical load of the member is determined by setting the determinant of the Hessian Matrix of the total potential energy function, evaluated along the fundamental equilibrium path equal to zero.

where W is the total potential energy function, and iq is the amplitude of the i th mode.

### Constrained finite strip method (cFSM)

The idea of integrating the basic assumptions of GBT into the framework of FSM resulted in the development of the constrained finite strip method (cFSM) by Ádány and Schafer [2] [3] [4]. In this method, the assumptions are used to define the Vlasov modes. The modes which violate the assumptions (named as ‘other’ modes) are defined by finding the orthogonal space to the Vlasov modes with respect to the global stiffness matrix K. Once the modal shapes belonging to one modal type (local, distortional, global or other) are defined, the critical load is determined by solving the eigenvalue problem:

where H is the constraint matrix containing in its columns the modal shapes of the concerning modal type, γ is the load factor, and a is a vector of the linear combination factors indicating the contribution of each mode to the compound modal shapes (i.e. amplification factors), among which the one with the lowest load factor would be critical. In the cFSM, the member deformation are then ‘constrained’ to the set of basic vectors of a given modal type, hence its name.

Although the application of this method was at first limited to open sections due to the Vlasov’s assumptions, more recently it was extended to arbitrary sections by its original inventors [5] [6]. In this updated method, new mechanical criteria regarding the membrane strains and curvatures in the plate segments are applied to produce modal shapes of various types. This makes it possible to define shear modes and transverse extension modes for closed sections. Meanwhile, the definition of Vlasov modes remains unchanged and transverse extension and shear are not allowed to participate in these modes. Moreover, an extension of the cFSM to general boundary conditions was presented by Li and Schafer [48]

## Non-linear interaction

The interaction between local and global buckling has been extensively studied. Since global buckling has no post-buckling residual capacity, researchers have focused mainly on the determination of the global secondary bifurcation stress after local buckling occurs. This critical stress is reduced compared to pure global buckling due to the reduction of the stiffness of the cross-section as a result of local buckling.

Fig. 2.4 The van der Neut Curve (excerpt of [94])

Using a simplified column model, Van der Neut [94] studied the effect of the interaction of local and global buckling on the load carrying capacity of an elastic column with varying length. As shown in Fig. 2.4, only the two flanges of the simplified column are load-carrying. If the column length L>L1, the column will buckle globally and its load carrying capacity will take the value of the critical load of elastic global buckling. If L0<L<L1, the column is at unstable equilibrium when local buckling critical load is reached and will fail abruptly; if L2<L<L0, the column is at stable equilibrium when the local buckling critical load is reached. If L<L2, the column will buckle first locally and eventually globally and the critical load of global buckling is determined by the tangent modulus method as ηKE, where η is the ratio between the compressive stiffness of the flange after and before local buckling.

Graves Smith [34] investigated a problem similar to that of Van der Neut, but for a practical rectangular section and while considering material plasticity. The Rayleigh-Ritz method was used to solve von Kámán’s equations, while trigonometric displacement functions were assumed. The yield stress of the material was assumed to be considerably higher than the local critical stress to avoid the complexity incurred by the interaction between local buckling and plasticity.

Rhodes and Harvey [62] studied the flexural overall buckling behaviour of a plain channel section column under either concentric or eccentric loading. The cross-section after local-buckling was considered as a beam-column element under a combination of axial force and bending moment. The deformations resulting from the load were described in terms of the average axial strain and curvature and the relation between the load and deformations was established through post-local-buckling analysis. This relation was then substituted into the differential equation governing the equilibrium of an eccentrically loaded column to analyse its flexural overall buckling behaviour.

Pignataro [60] studied true local-global interaction of simply supported channel columns on the basis of Koiter’s theory [41] of elastic stability. The perturbation method was used in the analysis. Similar research was also conducted by Benito and Sridharan [15] , who widened the scope of research to include not only local-global interaction of I-section and channel section columns, but also the interaction between local and lateral-torsional buckling of T-section beams.

Rasmussen [61] and Young [105] developed a method to study the global secondary bifurcation of columns after local buckling. In this method, similarly to the finite strip method, the cross-section is discretised to an assembly of strips. Buckling causes a reduction to the various stiffness of the strips, including the compressional, bending and torsional stiffness, dependent on the extent of the out-of-plane displacements. Thus, the locally buckled section is replaced by an unbuckled section with a varying stiffness along the section. In this way, the secondary bifurcation problem is transformed into a much simpler primary bifurcation problem. This analysis is applicable to both elastic and inelastic materials, since yielding merely causes a further reduction of the stiffness.

The research team led by Camotim [25] [51] [52] [82] conducted numerous numerical studies regarding local-distortional interactive buckling of common sections using the extended GBT. They initially focused on sections with similar local and distortional critical stresses before progressing further to sections with a greater range of the ratio between local and distortional critical stresses. Valuable conclusions were obtained about the buckling and post-buckling behaviour, including the elastic post-buckling equilibrium paths when different buckling modes are involved, the plastic equilibrium paths as a function of the slenderness of the column, the contribution of different bucking modes at the post-buckling stage subject to various imperfections, and the redistribution of the membrane longitudinal stress in the plates after buckling.

Another straightforward approach to investigate the non-linear interaction problem is to decompose the post-buckling deformed shapes obtained from finite element (FE) analysis using the FE buckling modes generated from cFSM output by extending the cross-sectional buckling deformation in a sinusoidal way in the longitudinal direction. Relevant research was carried out by Li et al. [47] involving the cFSM and by Nedelcu [57] involving GBT.

## Initial post-buckling stiffness

The load-end shortening curve of a cold-formed column under compression is shown in Fig. 2.5, where the horizontal axis represents the end shortening λ while the vertical axis represents the load P. It can be seen that the stiffness (slope of the curve) is reduced from K to Kp at the buckling point Pcr, and is further reduced gradually thereafter as the load increases. The stiffness Kp right after buckling occurs is defined as the initial post-buckling stiffness and is representative of the initial post-buckling behaviour of the column.

Fig. 2.5 Load-end shortening curve of a buckled column

As previously addressed, early research conducted by Yamiki [102], Rhodes and Harvey [64], and Walker [99] concerned post-local buckling of a single plate and determined the initial post-buckling stiffness. However, as these methods accounted for the change of the deformed shape in both the transverse and longitudinal directions with increasing load, their complexity becomes a deterrent factor if they are used in the analysis of plate assemblies. For plate assemblies, Graves Smith and Sridharan [85] [86] and Hancock [36] developed a semi-analytical FSM to investigate the post-buckling behaviour of plate assemblies under either uniform or linear varying end shortening. Graves Smith and Sridharan focused on channel and rectangular box section columns, while Hancock focused on square box and I-section columns. In their methods, both out-of-plane and transverse displacements were considered as unknowns. At a later stage, Sridharan [87] extended the application of his method to distortional buckling. He also used the perturbation technique to approximate the initial post-buckling behaviour neighbouring to the critical point. In parallel, Rhodes and Harvey [63] developed a semi-energy method for the same purpose. In this method, only out-of-plane displacements were considered as unknowns, while transverse displacements were determined by invoking the principle of minimum potential energy. Another noticeable feature of this method is that cross-sectional discretisation is performed at the minimum level by only introducing nodes at the junctions between the plate segments, thus reducing the complexity of the method. However, change of the deformed shape at the post-buckling stage was not considered. Finally, by incorporating the FSM into the semi-energy method, Ovesy [58] proposed a novel method combining the features of the aforementioned methods.

## Secondary bifurcation

The phenomenon of secondary bifurcation can be explained by the system with two DOFs shown in Fig. 2.6, where the deformed shape can be decomposed into a linear combination of two buckling modes:

where Q1, Q2, a1, a2 are buckling modes and their amplification factors. Stable equilibrium paths are plotted in solid line while unstable equilibrium paths are plotted in dashed line. It can be seen that when the load is below the critical load (λ < λcr), only one solution can be found for the equilibrium equations, which corresponds to the unbuckled state, and is shown in Fig. 2.6 as the unbuckled equilibrium path. The natural loading path (i.e. the path with the load increasing from zero) follows initially this path. However, as load is increased to the critical load (λ = λcr), an additional solution (or multiple solutions) emerges, which corresponds to the buckled state, and is shown in Fig. 2.6 as the primary buckling path. This path bifurcates from the unbuckled equilibrium path at the critical point associated with the critical load (also termed the bifurcation point). Since the unbuckled equilibrium path becomes unstable starting from the critical point, while the primary post-buckling path is stable, the natural loading path will follow the primary buckling path after the critical point, and the deformed shape consists purely of the first mode. However, this primary buckling path becomes unstable again after passing the secondary bifurcation point. This results in the switching of the natural loading pass from the unstable primary buckling path to the stable secondary buckling path through a transition path, incurring a sudden change of the deformed shape from pure first mode to pure second mode.

Fig. 2.6 Secondary bifurcation

Secondary bifurcation has been studied in sandwich struts [38], which is important because the neutral primary buckling path of such type of structural component is followed by an unstable secondary buckling path. This makes it unsafe to merely keep the load below the critical value without considering the effect of secondary bifurcation.

Secondary bifurcation is also important for local buckling of plates due to the considerable residual capacity after buckling, which makes it necessary to study the effect of secondary bifurcation in order to determine the ultimate capacity. Researcher has been conducted on local buckling of simply supported plates. Supple [88] developed a simplified model to investigate the problem, which considered only two purely local modes and one interactive mode, assuming that the number of half-waves stays unchanged under progressive loading. He then studied the effect of imperfections on the occurrence of the secondary bifurcation. Nakamura [56] also studied the problem using a more complicated but more accurate method, where he allowed the involvement of buckling modes with a different number of half-waves at the post-buckling stage. He calculated secondary buckling loads for plates with various aspect ratios, and compared the results when different levels of approximation were applied. Similar research was conducted by Uemura [92] [93].

Mennink [53] developed a prediction model for local buckling, where secondary bifurcation was considered. In this model, the plate segments constituting the cross-section can be divided into two groups, namely plate groups 1 (pg1) and 2 (pg2), with critical stresses lower and higher than the critical stress of the whole cross-section, respectively. Therefore, two local modes are relevant, being the primary mode and the secondary mode, where only the plates of pg1 and pg2 buckle, respectively. As shown in Fig. 2.7, at the critical strain εcr1, as only the plates of pg1 buckle, the cross-section buckles in a primary mode (the path L between εcr1 and εcr2 in the figure). Then at the critical strain εcr2, as the plates of pg2 also buckle, the cross-section buckles in an interactive mode between the primary and secondary modes (the path LL between εcr2 and εp in the figure). The cross-section finally fails at the strain εp due to material yielding. The application of this model was then extended by Kutanova [42] to distortional bucking of C-shaped specimen, where local-distortional (LD), distortional-local (DL) or distortional-local (DD/DL) interactive buckling occurs, as shown in Fig. 2.7. It can be seen that the stiffness of the specimen is reduced after primary buckling, and further reduced after secondary buckling. As a safe approach, the stiffness after secondary buckling is neglected, apart from the case where local-local interactive buckling occurs.

Fig. 2.7 Illustration of the Kutanova/Mennink model (excerpt of [42])

## Geometric imperfections

Geometric imperfections are the deviations from the perfect nominal geometry. Since the buckling behaviour of CFS members is dependent on the geometric imperfections, especially when coupled instabilities are involved, they play an important role in the design and analysis of CFS structures. Therefore, much work has been done on assessing the magnitude of imperfection and simulating then in numerical modelling. As cited by Zeinoddini [111], two types of imperfections are relevant, namely global imperfections and cross-sectional imperfections (local and distortional imperfections). Generally, more research has been done studying the former than the latter, including Dat [22], Mulligan [55], Young [109], Shifferaw et al. [79], Peterman and Schafer [59]. As for cross-sectional imperfections, two approaches have been developed. Early work was based on the simple approach that the imperfections at two specific points on the cross-section are used as representative of the type 1 (local) and type 2 (distortional) imperfection magnitudes respectively, as shown in Fig. 2.8, including Dat [22], Mulligan [55], Ingvarsson [39], Thomasson [89], Lau [45], Kwon [43] and Bernard [16]. Later on, a more advanced alternative approach was used by Young [109], Shifferaw et al. [79], and Peterman and Schafer [59] where the imperfection magnitudes were determined by seeking an optimal fit between the combined buckling modal shapes and the measured imperfection shape.

Fig. 2.8 Points representative of imperfection magnitudes

The global and cross-sectional imperfection magnitudes are commonly expressed as fractions of the specimen length and the plate thickness, respectively. In addition to these conventional expressions, Dowson and Walker [27] proposed two other methods to consider the magnitude of imperfections for plates: imperfection magnitude as a constant coefficient multiplied by the element slenderness, or a constant coefficient multiplied by the square of the element slenderness. Furthermore, Schafer and Peköz [70] performed a simple linear regression based on the plate width w for the type 1 imperfection (as shown in Fig. 2.8), yielding an approximate expression:

They also proposed an alternative rule based on an exponential curve fit to the thickness t:

Once the statistical imperfection magnitudes are determined, they are applied in imperfection simulation in numerical modelling. In [111], three approaches for this purpose are compared, including the traditional modal approach, the random field (2D) spectra approach and the 1D modal spectra approach.

In the traditional modal approach, imperfections are modelled as a combination of five buckling modes, including one local mode, one distortional mode, two flexural global modes and one torsional global mode. For each type of buckling modes, the mode with the lowest critical load is selected. The magnitudes of each mode is assumed to be a random value having mean and variance determined from available statistics of the measurements.

In the random field (2D) spectra approach, it is assumed that if the section is unfolded, the imperfection shape becomes a 2D random field superposed on the nominal geometry of the specimen. The random field is represented by a 2D power spectrum, which is determined also from available date about imperfections.

The traditional modal approach is easy to use but not so accurate while the random field (2D) spectra approach is accurate but difficult to implement. In order to combine the advantages of these two methods, the 1D modal spectra approach is proposed. In this method, the cross-sectional imperfection shape is assumed to be a combination of five buckling modes, while the longitudinal variation of the imperfection is modelled using a 1D power spectrum.

## Design approaches for coupled instabilities

The erosion of the ultimate load due to coupled instabilities is considered in the existing design approaches, including the effective width method (EWM) and the direct strength method (DSM), through different strategies.

### Effective width method

The effective width method, as the conventional design approach for CFS, works on the principle of load redistribution in the post-buckling range. Its consideration of coupled instabilities can be illustrated by the example of a lipped channel section under bending shown in Fig. 2.9.

The determination of the ultimate strength involves the following steps: First, the main nodes of the section are kept in place, and the effective widths are calculated in pure local buckling. Second, the reduction effect of distortional buckling on the design strength of the beam is considered by releasing the above restriction and introducing a spring at the centroid of the effective area of the edge stiffener, which also includes the adjacent effective area of the flange. As the design strength of the stiffener is limited by the critical stress of its flexural buckling, the effective width for the stiffener is recalculated, which in turn affects the design strength. After several such iterations are performed, the effective width of the edge stiffener is obtained as be2,n. Rather than reducing the effective width, distortional buckling is accounted for by reducing the effective thickness from t to tred. Once the effective portion of the section in load-bearing is determined, so is the ultimate strength. It can be seen that in this procedure, the coupled instabilities regarding local and distortional buckling have been considered.

Original section                       Step1                              Step2                              Step3

Fig. 2.9 Bending resistance of a lipped channel section

### Direct strength method

The direct strength method is a more recently developed design method for CFS members aiming at overcoming some of the shortcomings of the traditional effective width method. The history of this method can be dated back to the research on distortional buckling of rack post sections at the University of Sydney [35] [43] [45]. Later on, the method is further developed by Schafer [72] [74] to be applied to local, distortional and global modes, as well as interactions between the modes.

In this method, the ultimate capacity of a CFS member is evaluated by linking it to the critical load obtained from elastic buckling analysis via empirical formulas. As the post-buckling residual capacity differs greatly subject to the buckling type (local, distortional or global), different formulas are given for different instabilities. The formulas for members under compression are given as follows:

- Global Buckling

The nominal axial strength Pne for flexural, torsional or torsional- flexural buckling is:

For             		 
For

where the slenderness ,		 
the yield load Py = Agfy, with Ag being the gross area of the cross-section,
Pcre is the minimum of the critical elastic column buckling load in flexural, torsional, or torsional-flexural buckling.

- Local Buckling

The nominal axial strengthfor local buckling is:

For             		 
For

where the slenderness ,
Pcrl is the critical elastic local column buckling load,
Pne is the predicted ultimate load of global buckling given in Eq.  and Eq. .

- Distortional Buckling

The nominal axial strengthfor distortional buckling is:

For             		 
For

where the slenderness ,
Pcrd is the critical elastic distortional column buckling load,
Py is the previously defined yield load.

It is noted that in Eq.  and Eq. , local-global interaction is considered by replacing the yield load Py by the predicted ultimate load of global buckling Pne. Although the current DSM cannot deal with local-distortional interactive buckling, its effect on the ultimate load can be considered using a similar strategy. Two approaches for this purpose have been proposed by Schafer [74]:

- The nominal strength against local-distortional (NLD) interaction approach

The predicted ultimate load of global buckling Pne in Eq.  and Eq.  when local-global interactive buckling occurs is replaced by the predicted ultimate load of distortional buckling Pnd.

For             		 
For

where the slenderness .

- The nominal strength against distortional-local (NDL) interaction approach

The yield load Py in Eq.  and Eq.  when pure distortional buckling occurs is replaced by the predicted ultimate load of local buckling Pnl.

For             		 
For

where the slenderness .

According to the assessment by Dinis [25] and Silvestre [80], the performances of these two methods are basically equivalent, although the NDL approach provides marginally better results. Nevertheless, it was found by the same researchers that these approaches give unduly conservative predictions of the ultimate load at high levels of slenderness. Thus in order to achieve accurate predictions over the whole slenderness range, they proposed a refined DSM termed the modified NDL (MNDL) approach, which takes into account the intensity of the interaction between local and distortional buckling, reflected in the ratio between the half-wavelengths of the local and distortional modes. The approach adopts the current DSM distortional strength curve for stocky columns with λd < 1.5, while for slender column with  
λd > 1.5, it prescribes an ultimate load equal to Pndl in Eq.  with Pnl replaced by Pnl*:

For

For

For

It is noted that as Lcrd /Lcrl increases, the ultimate strength decreases since the interaction effect becomes more pronounced. Conversely, as Lcrd /Lcrl decreases, the interaction effect diminishes and the buckling becomes progressively ‘more distortional’. This novel DSM approach was extensively tested against lipped channel sections, as well as a variety of simple sections including Z-sections, hat sections and rack sections. Satisfactory results were achieved.

## Experimental studies

Experimental studies have been conducted extensively on CFS columns involving a variety of sections and buckling types.

Tomasson [89] studied local-global interactive buckling by testing 46 columns with lipped channel sections. The sections were categorised into type A, B and C, among which sections of type A had no stiffener, while sections of type B and C had one and two web stiffeners respectively, as shown in Fig. 2.10. Three different sizes of stiffeners were used, where the nominal values of dr equal 9, 14 and 17 respectively. The measured yield stress of the material ranging from 288 to 464 MPa. The columns were consistently 2500 mm long, and were tested between pin ends.

Fig. 2.10 Sections tested by Tomasson

Loughlan [50], Rhodes and Loughlan [65] studied local-global interactive buckling by testing 33 columns with lipped channel sections. The sections had four different web widths (102, 127, 152 and 187 mm), two different flange widths (51 and 64 mm), and two different lip widths (19 and 25 mm). The average measured yield stress of the material was 242 MPa for the 0.79 mm thick plate, and 233 MPa for the 1.6 mm thick plate. The columns had three different nominal lengths (1219, 1524 and 1829 mm), and were tested between pin ends.

As cited by Loughlan et al.[49], Adetoro [10] studied local buckling by testing 20 columns with lipped channel sections. The sections had four different web widths (102, 127, 152 and 187 mm), three different flange widths (51, 56 and 64 mm), and three different lip widths (9, 11 and 12 mm). The average measured yield stress of the material was 209 MPa. The columns had five different nominal lengths ranging from 1000 to 1800 mm in 200 mm increments, and were tested between fixed ends.

Mulligan and Peköz [54] studied local buckling by testing 22 columns with lipped channel sections. The sections had three different web widths (90, 120 and 180 times the plate thickness) and three different flange widths (30, 60 and 90 times the plate thickness), while the plate thickness was 1.2 mm. The lip width was determined in proportional to the web and flange width, so that it was adequate to prevent distortional buckling. The measured yield stress of the material was varying around 220 MPa among the specimens. The columns had several different measured lengths ranging from 507 to 3073 mm, with the longitudinal ends pinned about the weak axis and fixed about the strong axis.

Lau et al. [46] studied distortional buckling by testing 68 columns with various sections, including lipped channel sections (CH), rack column upright sections (RA), rack column upright sections with additional lip stiffeners (RL) and hat sections (HA). The measured dimensions of the sections are tabulated in Table 2.1, where the definitions of the notations are shown in Fig. 2.11. The plates with a thickness of 1.6, 2.0 and 2.4 mm were made of HR 340, HR2 and G450 steel respectively. For each of the sections, columns with six different nominal lengths (300, 700, 1100, 1370, 1640 and 1900 mm) were tested between fixed ends.

Fig. 2.11 Sections tested by Lau et al.

Table 2.1 Dimensions of the sections tested by Lau et al.

| Test Section | t (mm) | bw (mm) | bf or b1 (mm) | b2 (mm) | d or d1 (mm) | d2 (mm) | r (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CH17 | 1.670 | 83.0 | 61.8 | - | 10.2 | - | 2.8 |
| CH20 | 1.996 | 82.7 | 62.1 | - | 10.1 | - | 2.5 |
| CH24 | 2.394 | 81.4 | 58.8 | - | 10.5 | - | 2.1 |
| RA17 | 1.652 | 76.2 | 32.5 | 25.4 | 13.4 | - | 2.8 |
| RA20 | 1.982 | 76.1 | 32.8 | 25.8 | 13.9 | - | 2.5 |
| RA24 | 2.395 | 80.3 | 29.3 | 28.2 | 12.8 | - | 2.1 |
| RL17 | 1.658 | 85.1 | 36.7 | 22.1 | 11.9 | 3.7 | 2.8 |
| RL20 | 2.015 | 81.3 | 38.2 | 21.0 | 13.2 | 3.5 | 2.5 |
| RL24 | 2.386 | 81.2 | 28.3 | 26.4 | 13.2 | 4.2 | 2.1 |
| HA17 | 1.666 | 83.8 | 71.6 | - | 10.1 | - | 2.8 |
| HA20 | 1.976 | 84.2 | 72.2 | - | 10.1 | - | 2.5 |
| HA24 | 2.381 | 78.4 | 81.5 | - | 10.7 | - | 2.1 |

Young and Rasmussen [106] studied distortional buckling by testing 42 columns with plain and lipped channel sections. The nominal web width was 96 mm for all sections. The nominal lip width was 12 mm for all lipped channel sections. The nominal flange width was either 36 or 48 mm. The nominal yield stress of the material was 450 MPa. The columns had several different nominal lengths ranging from 280 to 3500 mm. The columns were grouped, and were tested between fixed ends and pin ends respectively.

Kwon et al. [44] studied interactive buckling by testing 28 columns with lipped channel sections. The sections were categorised into type A, B and C, among which sections of type A had no stiffener, while sections of type B and C had one web stiffener and two flange stiffeners respectively. The nominal dimensions of the sections are tabulated in Table 2.2, where the definitions of the notations are shown in Fig. 2.12. The nominal yield stress of the material was 560 MPa. The columns had four different lengths (400, 800, 1000, 1200 mm), and were tested between fixed ends.

Fig. 2.12 Sections tested by Kwon et al.

Table 2.2 Dimensions of the sections tested by Kwon et al.

| Specimens | t (mm) | b (mm) | h (mm) | s1 (mm) | d1 (mm) | s2 (mm) | d2 (mm) | ls (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A-6-1 | 0.6 | 40 | 40 | - | - | - | - | 10 |
| A-6-2 | 0.6 | 50 | 50 | - | - | - | - | 8 |
| B-6-1 | 0.6 | 40 | 40 | 12 | 6 | - | - | 10 |
| B-6-2 | 0.6 | 80 | 40 | 14 | 7 | - | - | 10 |
| C-6-1 | 0.6 | 90 | 40 | - | - | 10 | 5 | 10 |
| A-8-1 | 0.8 | 50 | 40 | - | - | - | - | 10 |
| A-8-2 | 0.8 | 40 | 40 | - | - | - | - | 10 |
| A-8-3 | 0.8 | 80 | 40 | - | - | - | - | 10 |
| A-8-4 | 0.8 | 80 | 40 | - | - | - | - | 5 |
| B-8-1 | 0.8 | 50.0 | 50.0 | 12.0 | 6.0 | - | - | 10.0 |

Santos et al. [67] studied interactive buckling by testing 12 columns with lipped channel sections. The nominal dimensions of the columns are tabulated in Table 2.3, where bw, bf and bs are the widths of the web, flange and lip respectively, L is the column length, and t is the plate thickness. The measured yield stresses for specimens C1-10 and C21-22 were 342 MPa and 407 MPa respectively. The columns were tested between fixed ends.

Table 2.3 Dimensions of the columns tested by Santos et al.

| Specimens | bw (mm) | bf (mm) | bs (mm) | L (mm) | t (mm) |
| --- | --- | --- | --- | --- | --- |
| C1-3 | 81 | 78 | 12 | 2850 | 1.11 |
| C4-6 | 75 | 65 | 11 | 2350 | 1.11 |
| C7-8 | 71 | 60 | 11 | 2100 | 1.11 |
| C9-10 | 62 | 55 | 11 | 1650 | 1.11 |
| C21-22 | 76 | 60 | 11 | 2350 | 1.11 |

Kwon and Hancock [43] studied local-distortional interactive buckling by testing 22 columns with lipped channel sections. The sections were categorised into two types named CH1 and CH2. CH1 sections had no stiffener, while CH2 sections had one web stiffener. The measured dimensions of the columns are tabulated in Table 2.4, where the definitions of the notations are shown in Fig. 2.13, and L is the column length. The columns were formed from AS 1397 grade 500 steel with a specified minimum yield stress of 500 MPa, and were tested between fixed ends.

Yang and Hancock [103] studied local-distortional interactive buckling by testing 21 columns with lipped channel sections. The sections had stiffeners on both the web and the flanges, as shown in Fig. 2.14. The nominal cross-sectional dimensions of the columns were consistent: b = 120, h = 90, sw = 20, sd = 10, and d = 12.5 (all in mm), leaving column length the only variable being one of the four values: 360, 800, 1300 and 2000 mm. The nominal yield stress of the material was 550 MPa. The columns were tested between fixed ends.

Fig. 2.13 Sections tested by Kwon and Hancock

Table 2.4 Dimensions of the columns tested by Kwon and Hancock

| Specimen | t (mm) | L (mm) | bw (mm) | bf (mm) | d (mm) | r (mm) | s1 (mm) | s2 (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH1-5-800 | 1.085 | 800 | 119.55 | 89.65 | 4.80 | 1.30 | - | - |
| CH1-6-800 | 1.085 | 800 | 119.80 | 89.75 | 6.00 | 1.30 | - | - |
| CH1-7-400 | 1.095 | 400 | 120.80 | 89.70 | 7.00 | 1.30 | - | - |
| CH1-7-600 | 1.095 | 600 | 120.40 | 89.55 | 7.00 | 1.30 | - | - |
| CH1-7-800 | 1.100 | 800 | 120.50 | 89.50 | 7.00 | 1.30 | - | - |
| CH2-7-800 | 1.100 | 800 | 120.50 | 90.15 | 7.00 | 1.30 | 10.00 | 20.00 |
| CH2-7-1000 | 1.100 | 1000 | 120.50 | 89.80 | 7.20 | 1.30 | 10.30 | 19.50 |
| CH2-8-1000 | 1.095 | 1000 | 119.95 | 89.95 | 8.00 | 1.30 | 10.15 | 20.00 |
| CH2-10-1000 | 1.105 | 1000 | 116.00 | 89.95 | 10.00 | 1.30 | 10.30 | 20.00 |
| CH2-12-1000 | 1.100 | 1000 | 120.50 | 89.95 | 12.00 | 1.30 | 10.00 | 19.50 |
| CH2-14-1000 | 1.105 | 1000 | 116.60 | 89.85 | 14.00 | 1.30 | 10.30 | 20.00 |

Fig. 2.14 Sections tested by Yang and Hancock

Young and Yan [108] studied distortional-global interactive buckling by testing 30 columns with lipped channel sections. The sections had complex edge stiffeners. The measured dimensions of the sections are tabulated in Table 2.5, where the definitions of the notations are shown in Fig. 2.15. The nominal yield stress of the material was 450 MPa. The columns had seven different nominal lengths ranging from 500 to 3500 mm in 500 mm increments, and were tested between fixed ends.

Fig. 2.15 Sections tested by Young and Yan

Table 2.5 Dimensions of the sections tested by Young and Yan

| Test series | brl (mm) | bl (mm) | bf (mm) | bw (mm) | t (mm) | ri (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| T1.5F80 | 17.6 | 28.0 | 83.5 | 153.6 | 1.482 | 2.0 |
| T1.5F120 | 17.4 | 28.1 | 123.8 | 153.4 | 1.478 | 2.0 |
| T1.9F80 | 18.8 | 28.4 | 83.4 | 154.7 | 1.884 | 2.0 |
| T1.9F120 | 18.9 | 28.2 | 123.3 | 155.1 | 1.871 | 2.0 |

Charnvarnichborikarn [20] studied distortional buckling by testing 123 columns with Z-sections. The nominal web width of the sections bw was consistently 102 mm, while the nominal flange width bf  had four different values (38, 51, 64 and 178 mm), and the lip width bl had eight different values (0, 4, 6, 13, 19, 25, 38 and 51 mm). The nominal yield stress of the material was 345 MPa. The columns had three different lengths (457, 610 and 1219 mm), and were tested between pin ends.

Young [107] studied local-distortional interactive buckling by testing 26 columns with lipped channel sections. The specimens had nominal web, flange, and lip widths ranging from 95 to 235, 45 to 190, and 15 to 30 mm, respectively. The measured yield stress of the material ranging from 336 to 589 MPa. The columns had length ranging from 616 to 2499 mm, and were tested between fixed ends.

# The polarisation method of modal decomposition

## Introduction

In order to gain a deeper understanding of the buckling behaviour, the deformed shape of a buckled thin-walled member can be expressed as a linear combination of a number of basic components, each having a clear mechanical description. The set of pure (local, distortional, global…) buckling modes, due to their natural orthogonality, are obvious candidates for these basic components. The process of generating these buckling modes and determining their contributions in the overall buckling behaviour is termed modal decomposition.

In this and the following chapters, the application of modal decomposition method is limited to analysing linear interaction between buckling modes at the critical point, although it should be stressed that the method itself has much wider applicability. An example of such kind of interaction is provided by the so-called signature curve shown in Fig. 3.1, where the critical load of a thin-walled member under pure compression is plotted against the half-wavelength. The dark blue line represents the results when all buckling modes are allowed to participate. The light blue line, the green line and the red line represent the curves for a constrained member when only the local, distortional or global modes are included, respectively. It can be seen that interaction between local and distortional buckling occurs at the same half-wavelength of approximately 350mm. The deformed shape of the coupled instability is illustrated in the bottom right corner, where it is clearly seen that both local and distortional modes play a role in the deformed shape.

Fig. 3.1 Linear mode interaction

Before proceeding with a full explanation of the proposed modal decomposition method, definitions of the membrane and bending components of strains and stresses which occur in a thin-walled member need to be provided. The longitudinal strain εx, the transverse strain εy, the shear strain γxy and their corresponding stresses σx, σy, τxy can be expressed as the summation of two components related to (1) bending and twisting and (2) membrane deformations. For example: σx = σxB + σxM. While the bending component varies linearly along the z -axis, the membrane component is constant (Fig. 3.2).

Fig. 3.2 Membrane and bending components

The basic categories of buckling types include local, distortional and global buckling. In the cFSM the following criteria are used to identify the distortional and global modes:

- The membrane shear strains are zero.

- The membrane transverse strains are zero.

- The warping displacements are linear within each flat plate segment

In addition, the global modes are characterised by rigid cross-section movement, and the transverse curvature of the plate segments is zero. However, these idealised assumptions are problematic when implemented in the FSM, where sufficient DOFs are present to account for the Poisson’s effect, resulting in membrane transverse strains and the transverse curvature, and the shear effect, resulting in membrane shear strains and non-linear warping displacements within each flat plate segment. Consequently, when maintaining the above assumption within the context of the FSM (as the cFSM does), overly rigid idealised buckling modes are obtained. This is illustrated, for instance, by the inconsistency between the distortional or global modal shapes obtained from the cFSM, and the shapes obtained from the FSM when pure distortional or global buckling is perceived to occur.

In modal decomposition methods, the local, distortional and global modes (shown in Fig. 2.1) alone do not constitute a complete set of basis vectors. In order for an arbitrary deformed shape to be expressed as a linear combination of buckling modes, the number of buckling modes should be equal to the number of degrees of freedom (DOFs) of the discretised section, which outnumber the total number of local, distortional and global modes. Therefore, extra modes need to be added to the set, namely the transverse extension modes and shear modes. Contrary to local, distortional and global modes, these modes are unlikely to be seen occurring independently in practice but are theoretically important. According to the discussion by Ádány [9] and Gonçalves et al. [31], shear modes are uniquely characterised by the presence of membrane shear strain. Two examples of such modes are illustrated in Fig. 3.3, where the one on the left is termed a transverse-only shear mode displaying only cross-sectional displacements, while the one on the right is termed a warping-only shear mode containing only warping displacements. The buckling mode on the very right side is the global mode associated with these two shear modes. It can be seen that the global mode can be expressed as a linear combination of these two shear modes, or in other words, any two of the three modes are linearly independent. Transverse extension modes are characterised by the presence of membrane transverse strain, as shown in Fig. 3.4. It is noted that these shear and transverse extension modes are established in the cFSM at the cost of significant programming effort, which is further increased for complex cross-sections. On the other hand, the aforementioned problems caused by the idealised assumptions for the distortional and global modes in the cFSM can be solved partially by allowing the shear and transverse extension modes to participate so as to re-introduce the neglected effects. However, this solution does not secure that these effects are properly considered and not exaggerated. Furthermore, these assumptions are prohibitive in achieving a complete orthogonal set of buckling modes, an issue which will be discussed later in section 3.2.2. Therefore, a novel modal decomposition method is needed, which does not rely on the idealised assumption of GBT.

In this study, two novel modal decomposition methods are proposed in the framework of the FSM, namely the polarisation method (PM), discussed in this chapter, and the nodal force method (NFM), discussed in the next chapter.

|  | Shear mode |  | Global mode |
| --- | --- | --- | --- |
|  | Transverse-only | Warping-only |  |
| Cross-sectional |  |  |  |
| Warping |  |  |  |

Fig. 3.3 Shear modes obtained from cFSM

Fig. 3.4 Transverse extension modes obtained from cFSM

## Introduction to the polarisation method

In this method, buckling modes are categorised into five types including local, distortional, global, transverse extension and shear. The shear modes are neither transverse-only nor warping-only but have non-zero values for all displacements. In this method, buckling modes are identified according to the proportion of the plate bending strain energy to the total strain energy in the modal shape. The buckling modes are sorted in an order from the highest proportion to the lowest. The different classes of modes can then be delineated based on the known number of modes associated with each type, which is calculated prior to the modal decomposition process. In general, local modes are associated with the highest proportions, while transverse extension and shear modes are associated with the lowest. Since transverse extension and shear modes contain similarly low proportions of the plate bending strain energy to the total strain energy, transverse extension modes are filtered out using the criterion of null membrane transverse stress. This is done in advance before the sorting process of the buckling modes.

### Number of modes

The calculation of the number of buckling modes is important for the polarisation method, which will be discussed in section 3.3.2. In the cFSM, the number of buckling modes associated with each type is calculated using the formulas tabulated in the second column of Table 3.1.

Table 3.1 Number of modes

| Mode type | cFSM | PM |
| --- | --- | --- |
| Local | nm + 2ne + 2ns | nm + 2ne + 2ns |
| Distortional | 2nm + ne − np + D − 3 | 2nm + ne − np + D − 3 |
| Global | 3 | 3 |
| Transverse extension | ns + np − D | ns + np |
| Shear | nm + ne + ns | nm + ne + ns − D |

where nm, ne, and ns are the number of internal main nodes, external main nodes, and sub nodes as previously defined in section 2.5. Moreover, np is the number of plate segments, and D is the static indeterminacy of the 2D truss analogous with the cross-section, as shown in Fig. 3.5 (b). If the truss is statically determinate or a mechanism, then D = 0, and all of the plate segments can extend or contract independently without resulting in any membrane transverse strain in other plate segments. In this case, the number of transverse extension modes equals to the number of strips in the discretised cross-section. Otherwise, if the truss is statically indeterminate and D > 0, as is the case in Fig. 3.5, the extension (or contraction) of some of the plate segments is coupled so that they cannot occur independently. Thus, the number of transverse extension modes is reduced by D. However, in the here proposed polarisation method, the defining feature of the transverse extension modes is altered from non-zero membrane transverse strain to non-zero membrane transverse stress. The membrane transverse stress is dependent on not only the membrane transverse strain but also the membrane longitudinal strain because of the Poisson’s effect. Therefore, a membrane transverse stress can occur independently in any of the plate segments, regardless of the cross-sectional geometry. Thus, the number of transverse extension modes will always equal the number of strips in the section. On the other hand, in the shear modes, the warping displacements are combined with extension or contraction in the plate segments to ensure that the assumption of null membrane transverse stress is obeyed. In the case where D > 0, since the extension or contraction in some of the plate segments cannot occur independently, the warping displacement cannot occur arbitrarily in all nodes without introducing transverse stress. Therefore, the number of shear modes is reduced by D in the proposed method. The number of buckling modes associated with each type in the polarisation method is summarised in the third column of Table 3.1. The difference is noted in the number of shear and transverse extension modes compared to the cFSM.

(a)                                              (b)

Fig. 3.5 Cross square section and its analogous truss

The number of local modes is determined in Table 3.1 based on the defining feature of the local modes that translation of the main nodes is prohibited. However, for sections with rounded corners this definition is too stringent. In this case, all nodes associated with a rounded corner are regarded as internal main nodes since they connect non-aligned plate segments. They are thus obliged to remain in place during local buckling, so that the corner is prohibited from rotating, thus virtually clamping the adjacent plate segments. This is contradictory to our intuitive understanding of the local modes where the corners are free to rotate. Therefore, the definition of main and sub nodes should be altered at the corner, which should be considered to contain only one internal main node, with the others counted as sub-nodes. For example, for the section shown in Fig. 3.9, each rounded corner consists of three nodes, among which one should be counted as an internal main node while the other two are sub nodes. Therefore, the section possesses a total 4 internal main nodes, 8 sub nodes and 2 external main nodes, resulting in 24 local modes according to the formula in Table 3.1.

### Orthogonality of modal shapes

It is necessary that the buckling modes form a set of linearly independent basis vectors of the deformation space. In addition to that, it is desirable that they are orthogonal with respect to the global stiffness matrix K. This orthogonality facilitates the decomposition of a random deformed shape, which can then be achieved by simply projecting the shape onto the basis vectors. The choice that they are orthogonal with respect to K is logical for two reasons [12]: First, the solution of the eigenvectors in Eq.  (i.e. the buckling deformed shapes) are orthogonal with respect to K. Therefore, if the buckling modes are also orthogonal with respect to K, there exists a rotation that transforms the buckling deformed shapes into pure buckling modes, given that the modes are normalised and correctly oriented along their axes (+1 or -1). Second, orthogonality with respect to K is associated with the physical meaning that the stresses resulting from one mode will not do any work in the strains of another mode. However, the assumption of null membrane transverse strain used in the cFSM for the distortional and global modes prohibits orthogonality among the buckling modes. This is attributed to the fact that the condition of zero membrane transverse strain will result in membrane transverse stress in the distortional and global modes. These transverse stresses, as a result of the Poisson’s effect, do work in the membrane transverse strains associated with the transverse extension modes. However, in our method orthogonality is regained.

The orthogonality consists of (1) the orthogonality among modal shapes of different types; (2) the orthogonality among modal shapes of the same type. The deformed shapes satisfying orthogonality (1) but not orthogonality (2) are termed non-orthogonal basis vectors of the mode space. These basis vectors can be further transformed to modal shapes by performing re-orthogonalisation on them, where the following eigenvalue problem is solved [8]:

where H is the matrix containing in its columns the non-orthogonal basis vectors of the mode space; A is the matrix containing in its columns the vectors of the linear combination factors (i.e. amplification factors); Λ = diag[λ1, λ2, …, λn] is the matrix containing the load factors. Therefore, the matrix containing modal shapes  is expressed as:

## Modal identification

### Modal identification step 1

The local, distortional, global and shear modes are distinguished from the transverse extension modes by assuming that the membrane transverse stress σy = 0. However, in the transverse extension modes the membrane transverse strain εy and the membrane shear strain γ are allowed.

According to the constitutive relations, the transverse stress can be expressed as:

Since εx varies linearly along the local y -axis within each strip, εy has to vary linearly as well in order for σy to vanish. This is contradictory to the observation in the FSM that εy is constant, because in the FSM transverse displacements vary linearly along the local y -axis within each strip, thus its derivative is constant. Therefore, an equivalent assumption is used instead stating that within each strip the average value of transverse stress is zero:

where the membrane transverse strain and the average membrane longitudinal strain are expressed as follows:

In the above equations v1 , u1 and v2, u2 are the displacements of the start and end nodes of the strip, respectively. When a transformation of the coordinate systems is carried out:

where α is the inclination angle of the plate segment with respect to the Y -axis, as shown in Fig. 3.6.

Fig. 3.6 Transformation from global to local displacements

Therefore, for a section discretised into np strips, Eq.  after substituting Eq.  and Eq.  becomes:

where the superscript i refers to the i th strip.

Eq.  can be summarised as Ch = 0, where C is a constraint matrix, and h is a basis vector of the space where the null membrane transverse stress assumption is valid .

Therefore, the matrix containing the non-orthogonal basis vectors of the local, distortional, global and shear space Hldgs is obtained by finding the null space of C:

Since the basis vectors of the transverse extension space ht need to be orthogonal to the basis vectors of the local, distortional, global and shear space with respect to K, they must satisfy

Thus, the matrix Ht containing the non-orthogonal basis vectors of the transverse extension space is obtained as follows:

Once the basis vectors of the transverse extension space are obtained, they are re-orthogonalised to obtain the modal shapes using Eq.  and Eq. :

where At is the matrix containing the amplification factors of the basis vectors in forming the transverse extension modal shapes contained in .

### Modal identification step 2

The deformations of a thin-walled member consist of two components: bending and membrane deformations, of which the corresponding strain energies are notated as UB and UM , respectively. For local buckling modes the total strain energy is virtually entirely composed of plate bending energy, while for global buckling modes membrane deformations contribute the most. Consequently, in terms of the difference UB  − UM , maximum values are reached when local buckling happens while the minimum values are witnessed when global buckling occurs. This suggests the possibility of determining the pure buckling modes by finding the extremum values of the function UB  − UM .

In order to make the values of UB  − UM associated with different deformed shapes comparable, the deformed shapes need to be normalised so that the total strain energy U is 1. This is added to the problem as a constraint. The constrained optimisation problem is solved through the Lagrange multiplier method, where the problem is equivalent to finding the optimum values of the Lagrange function

In the finite strip method (FSM), the strain energies associated with the deformed shape d are calculated as:

where KB and KM are the components of K associated with bending and membrane deformations respectively.

After the transverse extension modes have been filtered out, we have:

where aldgs is the matrix containing in its columns the amplification factors of the basis vectors of the local, distortional, global and shear spaces. Substituting Eq.  into Eq.  and the result into Eq.  and then setting the derivative of the function to zero yields:

Thereby the modal decomposition problem is transformed into an eigenvalue problem, where indicates the proportion of bending strain energy in the total strain energy:

It is noted that local modes are associated with UB /UM ratios close to 1 (or λ close to 1) while global and shear modes are characterised by UB /UM ratios close to 0 (or λ close to -1). This ‘polarisation’ of the λ values has given the method its name. Meanwhile, distortional modes possess UB /UM and λ values in between. Therefore, if the deformed shapes obtained from Eq.  are sorted in a descending sequence by their λ values, the local basis vectors will come first, followed by the distortional and global basis vectors. The shear basis vectors come last as their transverse plate bending is negligible. Since this sequence persists regardless of the half-wavelength, different mode spaces can be extracted as long as the number of buckling modes in each space is calculated using Table 3.1. For example, for the lipped channel section with sharp corners shown in Fig. 3.7, the local, distortional, global and shear modal shapes obtained after step 2 are shown in Table 3.2, along with the corresponding values of λ. In this and the following chapter, all illustrative examples of modal shapes are associated with an arbitrarily chosen half-wavelength of 1000 mm without further specification.

(All dimensions are measured in mm from mid-lines)

Fig. 3.7 Lipped channel section

Table 3.2 Modal shapes

| Local |  |  |  |  |  |  |  | Distortional |  | Global |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -0.039 | -0.370 | -0.998 | -1 |
|  | Shear |  |  |  |  |  |  |  |  |  |  |
| -1 | -1 | -1 | -1 | -1 | -1 | -1 |  |  |  |  |  |

### Modal identification step 3

Finally, within each space a re-orthogonalisation is performed according to Eq. , and the orthogonal buckling modes obtained are put in the matrixes , , and  for local, distortional, global and shear respectively. The full set of modal shapes is then given by:

Table 3.3 illustrates the full set of modes for the lipped channel section in Fig. 3.7.

Table 3.3 Modal shapes

| Local |  |  | Distortional | Global |
| --- | --- | --- | --- | --- |
|  | Transverse-extension | Shear |  |  |

## Modal decomposition

As modal shapes are vectors that can be arbitrarily scaled, they need to be normalised before being used in modal decomposition. The chosen normalisation criterion is that the strain energy resulting from each modal shape is equal to unity:

where  is the normalised modal shape. After the full set of normalised modes shapesis obtained, an arbitrary deformed shape d can be decomposed into a linear combination of the buckling modes:

where a is the vector containing the amplification factors of the pure buckling modes in forming the deformed shape, for which the modal participation is determined. Also:

where n is the total number of buckling modes or DOFs. There are two common definitions of modal participation [8], which are based on either displacements or energy. Among the two options the energy-based definition of modal participation was selected in this thesis, which determines the contribution of each mode in the total strain energy associated with the deformed shape. Thus, the participation factor of the i th mode is calculated as follows:

where  is the total number of DOFs.

## Illustrative examples

The method was tested on a variety of sections.

The mechanical properties of the material were assumed to be: E = 210000 MPa, ν = 0.3. The plate thickness was 1mm.

First, the lipped channel section with sharp corners shown in Fig. 3.7 was studied, whose modal shapes were previously established. The critical stress (signature curve) and modal participation are plotted against the half-wavelength in Fig. 3.8.

Second, the lipped channel section with rounded corners shown in Fig. 3.9 was studied. The modal shapes established are shown in Table 3.4. The critical stress and modal participation are plotted against half-wavelength in Fig. 3.10.

Third, the double-cell box section shown in Fig. 3.11 was studied. The modal shapes are shown in Table 3.5. The critical stress and modal participation are plotted against the half-wavelength in Fig. 3.12.

Fourth, the hollow-flange channel section shown in Fig. 3.13 was studied. The modal shapes are shown in Table 3.6. The critical stress and modal participation are plotted against the half-wavelength in Fig. 3.14.

Fifth, the Single-cell box section shown in Fig. 3.15 was studied. The modal shapes are shown in Table 3.7. The critical stress and modal participation are plotted against the half-wavelength in Fig. 3.16.

Finally, the multi-cell square section shown in Fig. 3.17, associated with a statically indeterminate analogous truss was studied. The modal shapes are shown in Table 3.8. It can be seen according to the formulas in Table 3.1 that since the degree of static indeterminacy is 1, there exists no distortional mode but there are four shear modes. It is also noted that the fourth transverse extension modal shape from the left looks more like a shear mode, displaying negligible cross-sectional displacements compared to the warping displacements. However, it is still identified as a transverse extension mode according to our definition since transverse stresses occur in the plate segments. The critical stress and modal participation are plotted against the half-wavelength in Fig. 3.18.

Fig. 3.8 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 3.9 Lipped channel section with rounded corners

Table 3.4 Modal shapes

| Local |  |  |  |  |
| --- | --- | --- | --- | --- |
| Distortional | Global | Transverse-extension |  |  |
|  |  |  | Shear |  |

Fig. 3.10 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 3.11 Double-cell box section

Table 3.5 Modal shapes

| Local |  |  |
| --- | --- | --- |
| Distortional | Global |  |
| Transverse-extension |  |  |
| Shear |  |  |

Fig. 3.12 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 3.13 Hollow-flange channel section

Table 3.6 Modal shapes

| Local |  |  | Distortional |
| --- | --- | --- | --- |
| Global | Transverse-extension |  |  |
| Shear |  |  |  |

Fig. 3.14 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 3.15 Single-cell box section

Table 3.7 Modal shapes

| Local | D | Global |
| --- | --- | --- |
| Transverse-extension | Shear |  |

Fig. 3.16 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 3.17 Multi-cell square section

Table 3.8 Modal shapes

| Local |  | Global |
| --- | --- | --- |
| Transverse-extension |  |  |
| Shear |  |  |

Fig. 3.18 Critical stress and modal participation

# The nodal force method of modal decomposition

## Introduction to the nodal force method

Fig. 4.1 Nodal line loads

According to Eq. , the global displacements and rotation obey trigonometric longitudinal shape functions, with their amplitudes , , ,  defined as the nodal displacements in the FSM. In order to generate such a deformed shape, nodal line loads are imposed on the 3D model equivalent to the 2D model used in the FSM, as shown in Fig. 4.1. These nodal line loads obey the longitudinal shape functions: , ,  and , representing the intensity of the loads per unit length. Therefore, the nodal forces in the FSM are defined as integrations of these nodal line loads over the length L:

In the nodal force method, the buckling modes are generated by applying precisely defined nodal forces f, obeying a definitive set of criteria for each of the buckling types. The corresponding modal shape h is obtained from a first-order elastic equation, which in the context of the FSM is given by:

where K is the global stiffness matrix, and f is the nodal force vector. Similarly to what was adopted for the polarisation method in the previous chapter, in the nodal force method buckling is categorised into five types including local, distortional, global, transverse extension and shear. In summary, the criteria used to achieve modal identification include (1) the criterion of non-zero longitudinal force to extract shear modes from the other modes, (2) the criterion of non-zero membrane transverse stress to identify the transverse extension modes, and (3) the equilibrium of cross-sectional forces and moments to distinguish the distortional modes from the global modes. In addition, the orthogonality with respect to K is consistently maintained during the establishment of buckling modes.

## Modal identification

The different types of modes are established as follows:

### Local modes

It is assumed that the local modes obey two assumptions.

- The longitudinal nodal forces associated with the displacement U are zero.

- Only a subset of DOFs, which includes the rotations of all nodes and the out-of-plane displacements w of sub nodes and external main nodes, takes on non-zero values. These DOFs are here termed local buckling DOFs. The reader is referred to section 2.5 for the definitions of the coordinate systems, displacements and node types.

The first assumption is also valid for the distortional, global and transverse extension modes. It is based on the fact that these modes can be generated by only applying transverse forces, without being accompanied by any longitudinal forces.

The second assumption is rooted in the characteristic feature of local buckling that only transverse plate bending occurs. The assumptions are enforced by converting Eq.  to Eq.  where the matrices and vectors are partitioned. The subscript 2 represents the DOFs associated with the displacements V and W of the internal main nodes, while the subscript 1 represents the remaining DOFs.

With h2 = 0, we obtain:

By imposing on the member, in turn, a unit nodal force (or moment) corresponding to one local buckling DOF while setting all other DOFs to zero, one basis vector of the local space is established. A transformation from the local to the global coordinate system is thereby required so that global nodal force vector f1 can be determined:

where α is again the inclination angle of the plate segment with respect to the Y –axis as shown in Fig. 4.2, fz and m are the nodal force and the moment corresponding to the local displacements w and θ, fY, fZ and Mx are the nodal forces and moment corresponding to the global Y - and Z – displacements and the rotations Θ. It is noted that since the components of f1 corresponding to the longitudinal displacements U are consistently zero, assumption (1) is satisfied. Once f1 is determined, the corresponding basis vector h1 is obtained from Eq. . Subsequently h1 is expanded to h by setting the additional elements, corresponding to h2, equal to zero. The matrix Hl is then defined, containing in its columns all the established non-orthogonal basis vectors of the local space. Finally, the matrix Hl can be transformed to the matrix containing all the orthogonal local modes by performing re-orthogonalisation as described in the previous chapter by Eq.  and Eq. . This procedure is illustrated with an example in which the lipped channel section in Fig. 4.3 is studied. The number of local modes equals the number of local buckling DOFs: Nl = nm + 2ne + 2ns, where nm, ne, ns are the number of internal main nodes, external main nodes, and sub nodes.

Fig. 4.2 Transformation from local to global nodal forces

(All dimensions are measured in mm from mid-lines)

Fig. 4.3 Lipped channel section

In Fig. 4.3, the out-of-plane displacements of the eight basis vectors before re-orthogonalisation are shown in the middle panel; The corresponding cross-sectional nodal forces are shown in the upper panel, and the out-of-plane displacements of the final modal shapes are shown in the lower panel.  In the nodal force diagram, nodal forces corresponding to f1 in Eq.  are presented in red, while nodal forces corresponding to f2 are presented in blue.

(a) Nodal forces of the basis vectors

(b) Cross-sectional displacements of the basis vectors

(c) Cross-sectional displacements of the buckling modes

Fig. 4.4 Local modal shapes and nodal forces

### Distortional modes

It is assumed that the distortional modes obey four assumptions:

- The longitudinal nodal forces are zero.

- The membrane transverse stresses σy = 0.

- The cross-sectional nodal forces (i.e. nodal forces corresponding to the cross-sectional displacements) and moments are in translational and rotational equilibrium.

- They are orthogonal to the previously established local modes with respect to K.

The second assumption replaces the assumption of null membrane transverse strain applied in GBT and the cFSM and instead allows extension and contraction of the strips as a result of the Poisson’s effect. Meanwhile, the third assumption ensures that any overall lateral translation or rotation of the cross-section is prohibited in distortional buckling, which distinguishes it from global buckling.

These assumptions are enforced as follows:

Assumption 1:

This assumption is enforced by performing the following transformation:

where  is the reduced nodal force vector containing all nodal forces except the longitudinal ones. If n is defined as the total number of nodes in the cross-section, then the transformation matrix T contains in its rows a 3n × 3n identity matrix I and an n × 3n zero matrix 0.

Assumption 2:

For the implementation of this assumption the reader can be referred back to section 3.3.1, where it was shown to lead to Ch = 0. While C is constraining the basis vector h, its equivalent matrix C2 constraining the nodal force vector f is defined by:

Assumption 3:

This assumption requires that the resultant forces and torque of the nodal forces vanish:

where fYi and fZi are the nodal force components in the global Y and Z directions at node i, and Yi and Zi are global coordinates of node i with respect to any chosen origin. Eq.  can be summarised in matrix form as C3 f = 0.

Assumption 4:

Given that Hl contains the basis vectors of the local space, the orthogonality is expressed as

.

Therefore, the reduced nodal force vectors generating the distortional modes should satisfy:

Thus, the matrix  containing all the distortional reduced nodal force vectors is calculated by finding the null space of Cd:

Subsequently  is transformed to the matrix Fd containing all the full-size nodal force vectors as follows:

The matrix Hd containing the basis vectors of the distortional space is then calculated as:

The matrix C2 is (np + ns) × 4n in dimensions, where np + ns is the number of strips, and np is the number of plate segments. The matrix C3 has size 3 × 4n, and the matrix HlT has size  
Nl × 4n. Since the matrix T is  in size, the matrix Cd is a (np + ns + Nl + 3) × 3n matrix. If the rows in C2T are linearly independent, the number of distortional modes 
Nd = 2nm + ne – np − 3. However, this is only the case when the analogous truss introduced in section 3.2.1 is statically determinate. Otherwise, some rows in C2T will become linearly dependent, which physically means that the transverse stress cannot occur independently in each of the plate segments assuming that the longitudinal nodal forces are all zero. In this case the number of distortional modes is increased by the degree of static indeterminacy of the truss D. After performing a re-orthogonalisation, the matrix Hd can be transformed to the matrix  containing all the distortional modes, as previously explained by Eq.  and Eq. .

Reverting back to the example introduced in Fig. 4.3, the cross-sectional displacements of the two distortional modal shapes are shown in the middle panel of Fig. 4.5; the warping displacements are shown in the lower panel, and the corresponding cross-sectional nodal forces are shown in the upper panel.

(a) Nodal forces

(b) Cross-sectional displacements

(c) Warping displacements

Fig. 4.5 Distortional modal shapes and nodal forces

### Global modes

It is assumed that the global modes obey four assumptions:

- The longitudinal nodal forces are zero.

- The membrane transverse stresses σy = 0.

- They are orthogonal to the previously established local modes with respect to K.

- They are orthogonal to the previously established distortional modes with respect to K.

Similarly, the reduced nodal force vectors for global modes should satisfy:

Thus, the matrix Hg containing the basis vectors of the global space is calculated as:

The number of global modes is always three. After performing a re-orthogonalisation, the matrix Hg can be transformed to the matrix  containing all the global modes.

In Fig. 4.6, the cross-sectional displacements of the three global modal shapes for the lipped channel in Fig. 4.3 are shown in the middle panel; the warping displacements are shown in the lower panel, and the corresponding cross-sectional nodal forces are shown in the upper panel.

(a) Nodal forces

(b) Cross-sectional displacements

(c) Warping displacements

Fig. 4.6 Global modal shapes and nodal forces

### Transverse extension and shear modes

It is assumed that the transverse extension modes obey four assumptions:

- The longitudinal nodal forces are zero.

- They are orthogonal to the previously established local modes with respect to K.

- They are orthogonal to the previously established distortional modes with respect to K.

- They are orthogonal to the previously established global modes with respect to K.

Similarly, the reduced nodal force vectors for transverse extension modes should satisfy:

Thus, the matrix Ht containing the basis vectors of the transverse extension space is calculated as:

If the analogous truss is statically determinate, the number of transverse extension modes equals the number of strips Nt = np + ns. Otherwise, some combinations of the transverse stresses in the plate segments cannot satisfy the assumption that the longitudinal nodal forces are all zero, and the number of transverse extension modes is reduced by D. After performing re-orthogonalisation, the matrix Ht can be transformed to the matrix  containing all the transverse extension modes.

Finally, the shear modal shapes are established via the assumption that they are orthogonal to all the other modes established previously with respect to K. Thus, the nodal force vectors for the shear modes should satisfy:

The matrix Hs containing the basis vectors of the shear space is thus calculated as:

The number of shear modes is calculated by deducting the total number of local, distortional, global and transverse extension modes from the total number of DOFs, namely
Ns = 4n – Nl – Nd – Ng – Nt = n. After performing a re-orthogonalisation, the matrix Hs can be transformed to the matrix  containing all the shear modes.

In Fig. 4.7, the cross-sectional displacements of the transverse extension modal shapes of the lipped channel in Fig. 4.3 are shown in the middle panel. The warping displacements are shown in the lower panel, and the corresponding cross-sectional nodal forces are shown in the upper panel.

(a) Nodal forces

(b) Cross-sectional displacements

(c) Warping displacements

Fig. 4.7 Transverse extension modal shapes and nodal forces

(a) Cross-sectional nodal forces

(b) Longitudinal nodal forces

(c) Warping displacements

Fig. 4.8 Shear modal shapes and nodal forces

In Fig. 4.8, the warping displacements of the shear modal shapes are shown in the lower panel. The corresponding cross-sectional nodal forces are shown in the upper panel, and the longitudinal nodal forces are shown in the middle panel. It should be noted that the latter, of course, act perpendicular to the plane of the cross-section.

In summary, the validity of the assumptions defined in section 4.2.2 for different types of modes are tabulated in Table 4.1, where the ticks mean that the assumptions must be satisfied, while the crosses mean that the assumptions may or may not be satisfied.

Table 4.1 Assumptions for different mode types

|  | Asm. 1 | Asm. 2 | Asm. 2 |
| --- | --- | --- | --- |
| Local | √ | √ | √ |
| Distortional | √ | √ | √ |
| Global | √ | √ | × |
| Transverse extension | √ | × | × |
| Shear | × | × | × |

## Illustrative examples

The method was tested on the same sections used in the previous chapter.

First, the lipped channel section shown in Fig. 4.3 was studied, whose modal shapes were already shown in the derivation of the method. The critical stress and modal participation are plotted against the half-wavelength in Fig. 4.9.

Second, the double-cell box section shown in Fig. 4.10 was studied. The modal shapes are shown in Table 4.2. The critical stress and modal participation are plotted against the half-wavelength in Fig. 4.11.

Third, the hollow-flange channel section shown in Fig. 4.12 was studied. The modal shapes are shown in Table 4.3. The critical stress and modal participation are plotted against the half-wavelength in Fig. 4.13.

Fourth, the Single-cell box section shown in Fig. 4.14 was studied. The modal shapes are shown in Table 4.4. The critical stress and modal participation are plotted against the half-wavelength in Fig. 4.15.

Finally, the cross square section shown in Fig. 4.16 was studied. The modal shapes are shown in Table 4.5. It is noted that compared to Table 3.8, presenting the results of the polarisation method, this method leads to one more shear mode but one less transverse extension mode. The critical stress and modal participation are plotted against the half-wavelength in Fig. 4.17.

Fig. 4.9 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 4.10 Double-cell box section

Table 4.2 Modal shapes

| Local |  |  |
| --- | --- | --- |
| Distortional | Global |  |
| Transverse-extension |  |  |
| Shear |  |  |

Fig. 4.11 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 4.12 Hollow-flange channel section

Table 4.3 Modal shapes

| Local |  | Distortional |  |
| --- | --- | --- | --- |
| Global | Transverse-extension |  |  |
| Shear |  |  |  |

Fig. 4.13 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 4.14 Single-cell box section

Table 4.4 Modal shapes

| Local | D | Global |
| --- | --- | --- |
| Transverse-extension | Shear |  |

Fig. 4.15 Critical stress and modal participation

(All dimensions are measured in mm from mid-lines)

Fig. 4.16 Cross square section

Table 4.5 Modal shapes

| Local |  | Global |  |
| --- | --- | --- | --- |
| Transverse-extension |  |  |  |
| Shear |  |  |  |

Fig. 4.17 Critical stress and modal participation

# Comparison between the modal decomposition methods

## Introduction

In this chapter, a comparison is made between the available modal decomposition methods, including the polarisation method (PM) and the nodal force method (NFM) as newly proposed methods, together with the generalised beam theory (GBT) and the constrained finite strip method (cFSM) as existing methods. The version of GBT discussed in this chapter is the one implemented in GBTUL 2.0.4.3 [11], and the cFSM is implemented according to [5] [6].

## Comparison between the newly proposed methods

### Similarities

The newly proposed polarisation method and the nodal force method are sharing four common features, among which the first three are advantages over the existing methods:

- Both of the methods are able to deal with complex cross-sections (sections with closed part or branches) without any increase in computational effort. Although these sections are not yet classified to be designed using the DSM, the improved computational efficiency of the proposed methods will facilitate the future extension of the application of the DSM. Moreover, the polarisation method can also deal with sections with rounded corners. This contributes to the improved accuracy of this method compared to other methods, where the influence of rounded corners can only be taken by analysing an otherwise similar cross-section with sharp corners.

- In both of the methods, the Poisson’ s effect and the shear effect are accounted for in the distortional and global modes, thus allowing membrane transverse strains, transverse curvature, membrane shear strain and non-linear warping displacements within each flat plate segment. The distortional or global modal shapes obtained from these modal decomposition methods coincide with the shapes obtained from the FSM when pure distortional or global buckling occurs.

- In both of the methods, all modes are orthogonal to each other with respect to K. This orthogonality simplifies the decomposition of a random deformed shape, as discussed in section 3.2.2.

- In both of the methods, modal shapes vary with the half-wavelength. This is due to the fact that the Poisson’s effect and the shear effect have more relative importance at short half-wavelengths.

### Differences

Table 5.1 Number of modes

| Mode type | PM | NFM |
| --- | --- | --- |
| Local | nm + 2ne + 2ns | nm + 2ne + 2ns |
| Distortional | 2nm + ne − np + D − 3 | 2nm + ne − np + D − 3 |
| Global | 3 | 3 |
| Transverse extension | ns + np | ns + np − D |
| Shear | nm + ne + ns − D | nm + ne + ns |

Note:   nm 		number of internal main nodes

ne		number of external main nodes

ns 		number of sub nodes

np		number of plate segments

D		static indeterminacy of the analogous 2D truss

One major difference between these two methods resides in the treatment of shear modes and transverse extension modes. The criterion of null membrane transverse stress is valid for the shear modes in the PM but not for the same type of modes in the NFM. On the other hand, the criterion of null longitudinal force is valid for the transverse extension modes in the NFM but not for the same type of modes in the PM. This accounts for the difference in the number of buckling modes which emerges when the analogous truss addressed in section 3.2.1 is statically indeterminate. As shown in Table 5.1, the number of transverse extension modes is then reduced by the degree of static indeterminacy D in the NFM but not in the PM. Conversely, the number of shear modes is reduced by D in the PM but not in the NFM. It is also noted that the number of various types of buckling modes in the NFM is consistent with that in the cFSM.

Direction

of translation

(a) Nodal force method                         (b) Polarisation method

Fig. 5.1 Flexural global mode

(a) Shear flow                                     (b) Equivalent nodal force

Fig. 5.2 Shear flow and equivalent nodal force configuration

Another difference is unveiled when the double-cell box section in Fig. 3.11 is studied. For this section, the modal shapes of the same flexural global mode obtained from the two methods are shown in Fig. 5.1, where the cross-section translates upwards as a whole. In both (a) and (b) the tension side of the section (top) contracts while the compressive side (bottom) extends due to the Poisson’s effect. However, the two sections distort differently. The difference is attributed to the shear flow pattern shown in Fig. 5.2 (a) with two shear flows of the same magnitude but opposite direction occurring in the two closed cells of the section. The equivalent nodal force configuration shown in Fig. 5.2 (b) obeys the equilibrium assumption imposed on the distortional modes in the NFM, and this shear flow pattern is thus allowed to participate in the distortional modal shapes obtained from the NFM. Consequently, it is prohibited in the global modal shapes in the NFM due to the orthogonality requirement. On the other hand, this shear flow pattern is allowed in the global modal shapes obtained from the PM, resulting in the modal shape shown in Fig. 5.1 (b) where plate bending is minimised.

In summary, the definitions of various types of buckling modes are slightly different in these two methods. However, it will be proved in the next section that the influence of these differences on the local, distortional and global critical stresses is negligible.

## Comparison between the newly proposed and existing methods

It is also important to compare the newly proposed methods with the existing methods (GBT and cFSM), which leads to some interesting observations.

### Mode generation

In the cFSM and the NFM, the basis vectors associated with a specific type of buckling modes are generated by making assumptions associated with their mechanical features. For example, in the cFSM global modes are characterised by rigid translation or rotation of the section, while in the NFM two assumptions are made to global modes that they have null longitudinal nodal force and membrane transverse stress.

In the PM, first the transverse extension modes are distinguished from other modes through the assumption of non-zero membrane transverse stress. Then other buckling modes are identified according to the proportion of the plate bending strain energy to the total strain energy in the modal shape.

In all the three methods, within each type of buckling, the basis vectors are transformed to buckling modes by performing the re-orthogonalisation process.

In GBT, the conventional buckling modes are generated in a process similar to the re-orthogonalisation process in Eq.  to transform basis vectors to buckling modes. First, initial basis vectors are defined, each with the displacement value associated with one specific DOF set equal to unity while all other displacement values are set equal to zero. During the process of mode generation, the initial basis vectors are transformed to buckling modes, while simultaneously the matrices in Eq.  are diagonalised. The obtained set of conventional modes is subsequently complemented by transverse extension and shear modes generated independently using mechanical assumptions.

### Orthogonality

In the cFSM, a number of different orthogonality criteria are used regarding displacements and strains, including the warping displacements U, the membrane shear strains γxy, the membrane transverse strains εy and the transverse curvatures κy = ∂2 w/∂2 y, as shown in Fig. 5.3. The buckling type labels are coloured according to the orthogonality criterion used within the class between two buckling modes of the same type. The arrows in different colours linking two labels represent the orthogonality criterion used between two buckling modes of different types. For the distortional modes, either the orthogonality criterion regarding U or κy can be used. Alternatively, the buckling modes within each type can also be re-orthogonalised using Eq.  so that they are orthogonal with respect to matrices K and G.

Note:   L		local modes

D		distortional modes

G		global modes

SDw 		warping-only shear modes associated with distortional modes

SGw 		warping-only shear modes associated with global modes

SDt 		transverse-only shear modes associated with distortional modes

SGt 		transverse-only shear modes associated with global modes

Fig. 5.3 Orthogonality among buckling modes in the cFSM

In the GBT, the conventional modes are orthogonal with respect to the matrices C and BB (i.e. the component of B associated with transverse plate bending). While part of the conventional mode space, the global modes are also orthogonal with respect to the matrices C and D1 (i.e. the component of D associated with shear). Within the global mode space, the axial compression mode and the flexural global modes are orthogonal with respect to the matrices C and Χ. Matrix X takes into account the geometrically non-linear effect, and is calculated as

where the integration is performed along the cross-sectional mid-line S, σx is the membrane longitudinal stress,   and  are longitudinal amplitudes of the cross-sectional displacements. In addition, the natural shear modes (main nodes are allowed to move) are orthogonal with respect to the matrices BB and D1, while the local shear modes (only sub nodes are allowed to move) are orthogonal with respect to the matrices CM (the component of C associated with longitudinal membrane deformation) and D1M (the component of D1 associated with shear resulting from membrane deformation). Finally, the transverse extension modes are orthogonal with respect to the matrices BM (the component of B associated with transverse membrane deformation) and D1.

Contrarily to the complicated framework in the existing methods where a number of different orthogonality criteria are used linked to the mechanical features of the buckling modes, in the newly proposed methods a unified orthogonality criterion with respect to K is used among all the buckling modes.

### Scope of applicability to various cross-sections

Although all of the methods are able to deal with arbitrary sections, computational effort is increased for GBT when analysing complex cross-sections. In such cases, the conventional modes obtained from the classic GBT are complemented by independently generated shear modes and transverse extension modes. Moreover, since Vlasov’s assumption is violated, the approximation used in the classical beam theory is abandoned. Consequently, the simplicity of the method is lost. Similar problems occur for the cFSM, where shear modes and transverse extension modes are established at the cost of significant programming effort, especially for complex cross-sections. In the newly proposed methods, there is no increase in computational effort when dealing with complex cross-sections. Finally, the cFSM, GBT and the NFM are able to deal with only cross-sections with sharp corners, while the PM is able to deal with cross-sections with rounded corners.

### Constitutive relations

Concerning the constitutive relations for the membrane deformations, the cFSM, the PM and the NFM adopt the exact equations:

In GBT, the expressions of the membrane longitudinal and transverse stresses are uncoupled so that:

### Membrane strains and stresses

Since different definitions are attached to different types of buckling modes in various modal decomposition methods, membrane strains and stresses can differ. This is summarised in Table 5.2, where the ticked components are allowed (meaning they may or may not occur) while the crossed components are prohibited (meaning they must not occur). In the PM and the NFM, the majority of the strain and stress components are allowed, except for the transverse stress σy , which is prohibited for the conventional modes in both the PM and the NFM, and for the shear modes only in the PM. In the cFSM and GBT, in light of the fact that the shear modes in GBT are equivalent to the warping-only shear modes in the cFSM, identical assumptions are used regarding the strains. First, no transverse extension or shear are allowed for the conventional modes. Second, since no warping displacements are allowed in the transverse extension modes, the longitudinal strain εx is zero. However, in these transverse extension modes as the plates extend or contract, membrane transverse strains εy and shear strains γxy are introduced. Third, for the shear modes, εx and γxy are allowed while εy is prohibited. In particular, for the transverse-only shear modes in the cFSM, which are generated by adopting the cross-sectional displacements of the corresponding conventional modes while eliminating the warping displacements, εx and εy are prohibited while γxy is allowed. Based on these assumptions regarding the strains, the existence or non-existence of stresses is determined using the constitutive relations in Eq.  and Eq. .

Table 5.2 Relevant membrane strains and stresses

| Method | PM |  |  | NFM |  |  | cFSM |  |  |  | GBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mode type | C | TE | S | C | TE | S | C | TE | St | Sw | C | TE | S |
| εx | √ | √ | √ | √ | √ | √ | √ | × | × | √ | √ | × | √ |
| εy | √ | √ | √ | √ | √ | √ | × | √ | × | × | × | √ | × |
| γxy | √ | √ | √ | √ | √ | √ | × | √ | √ | √ | × | √ | √ |
| σx | √ | √ | √ | √ | √ | √ | √ | √ | × | √ | √ | × | √ |
| σy | × | √ | × | × | √ | √ | √ | √ | × | √ | × | √ | × |
| τxy | √ | √ | √ | √ | √ | √ | × | √ | √ | √ | × | √ | √ |

Note:   C 		conventional modes

TE		transverse extension modes

S 		shear modes

St 		transverse-only shear modes

Sw 		warping-only shear modes

εx , σx		membrane longitudinal strain and stress

εy , σy		membrane transverse strain and stress

γxy , τxy		membrane shear strain and stress

## Case studies

### Lipped channel section

First, the lipped channel section shown in Fig. 5.4 was studied, and the critical stresses obtained from various modal decomposition methods are compared. For GBT and the cFSM, the critical stresses of distortional and global modes in two cases are calculated. In the first case, only distortional or global modes are allowed. In the second case, the transverse extension and shear modes are also allowed, which couple with distortional or global modes. Due to the straightforward definition of the local buckling modes, which is consistent across all presented modal decomposition methods, the local critical stresses given by the various methods are perfectly consistent. Therefore, local buckling is not included in the here presented comparison. The critical stresses associated with distortional and global buckling are plotted in Fig. 5.5 and Fig. 5.6 respectively.

(All dimensions are measured in mm from mid-lines)

Fig. 5.4 Lipped channel section

Some remarks can be made on the outcomes. For both distortional and global buckling, as the half-wavelength becomes extremely short, as shown in Fig. 5.5 (a) and Fig. 5.6 (a): (1) The critical stresses obtained from GBT will become infinite, while a very large finite maximum value is reached for the cFSM, which according to [7] is E/(1 − v2) for global buckling. (2) Considerably smaller finite maximum values can be found for the critical stresses obtained from the PM and the NFM. (3) The critical stresses obtained from GBT and the cFSM with coupled shear modes and transverse extension modes are at the lowest. This divergence is attributed to the different involvement of the membrane transverse and shear strains in distortional and global modes obtained from the various methods. Since these strains are increasingly important at short half-wavelengths, the results diverge. In (1), none of these strains are allowed. In (2), these strains can develop, but under the condition that the membrane transverse stress is zero in both the PM and the NFM, and the longitudinal nodal force is zero in the NFM. In (3), these strains are free to develop without constraints of any kind. In the first two cases, the constraints on the membrane transverse and shear strains increase the stiffness of the column, thus increasing the critical stresses. Furthermore, at short half-wavelengths it is evident that the global critical stress obtained from PM is higher than that obtained from the NFM. This suggests that more membrane transverse and shear strains are allowed in the NFM when generating global modes.

In addition, a drop in the critical stresses obtained from PM is noted at extremely short half-wavelengths. This is attributed to the fact that in PM local, distortional, global and shear modes are identified by evaluating the contribution of strain energy resulting from plate bending in the total strain energy. Among the components of the bending strain energy, at extremely short wavelengths the one resulting from longitudinal bending is dominant over that resulting from transverse bending. Thus, bending strain energy plays an increasingly important role not only in local and distortional modes, but also in global modes. Therefore, at extremely short half-wavelengths sometimes changes happen to the ranking of buckling modes by the proportion of bending strain energy, which accounts for the drop of the critical stresses. This is more evident for the next example regarding the single-cell box section. However, it is found that the problem is resolved if the mesh of the cross-section is refined, as shown in Fig. 5.6 (b). The same issue for distortional buckling is shown when comparing Fig. 5.5 (a) and (b).

Although discrepancy exists among the results obtained from various methods at extremely short half-wavelengths, this is of limited importance for the applicability of these methods since these very short half-wavelengths are of very little practical interest.

At longer half-wavelengths for global buckling, as shown in Fig. 5.6 (c), a good correlation is achieved between the NFM and the PM, while the cFSM gives slightly higher results and GBT gives slightly lower results. These differences reside in the previously addressed discrepancies regarding the constitutive relation of the membrane deformations. It is noted that the results obtained from the cFSM are approximately 10% higher than those from GBT, which derives from the ratio 1/(1 − v2) in Eq. . However, when transverse extension and shear modes are allowed to participate in the global modes, the critical stresses obtained from the cFSM are lowered to a level close to that of the NFM and the PM, but is still slightly higher than that of GBT.

It is known from the modal participations shown in Fig. 3.8 and Fig. 4.9 that at half-wavelengths over 5000 mm the column buckles solely in the global mode, thus the critical stress obtained from the FSM equals real global critical stresses. Therefore, it is worthwhile to verify the global critical stresses obtained from various modal decomposition methods against the the critical stress obtained from the FSM at long half-wavelengths, as shown in Fig. 5.6 (d). It can be seen that a better correlation is achieved by the NFM and the PM compared to the cFSM and GBT, which suggests that the proposed methods provide better results of the global critical stresses.

Similar remarks can be made on the distortional critical stresses shown in Fig. 5.5 (c), while the difference between the results obtained from the cFSM and GBT is lessened. This is due to the fact that as opposed to global buckling, in distortional buckling the longitudinal stress plays a less relevant role [7].

(a) Distortional critical stresses over extended range of half-wavelength

(b) Distortional critical stresses (dense discretisation) over extended range of half-wavelength

(c) Distortional critical stresses at intermediate half-wavelengths

Fig. 5.5 Distortional buckling critical stress of lipped channel section

(a) Global critical stresses over extended range of half-wavelength

(b) Global critical stresses (dense discretisation) over extended range of half-wavelength

(c) Global critical stresses at intermediate half-wavelengths

(d) Global critical stresses verified against the FSM

Fig. 5.6 Global buckling critical stress of lipped channel section

### Single-cell box section

Second, the single-cell box section shown in Fig. 5.7 was studied, whose critical stresses for distortional and global buckling are plotted in Fig. 5.8 and Fig. 5.9 respectively. It is important to note that for the cFSM and GBT, the global modes consist of only those modes obeying the null-membrane-shear-strain criterion. Since a shear flow occurs in the torsional mode of the box section, it is not considered to be a global mode.

(All dimensions are measured in mm from mid-lines)

Fig. 5.7 Single-cell box section

At extremely short half-wavelengths, the discrepancy among the critical stresses obtained from various methods is due to the reasons explained for the lipped channel section. The sudden drop of the distortional critical stress obtained from the PM in Fig. 5.8 (a) disappears when the cross-section is discretised, as shown in Fig. 5.8 (b).

Again at intermediate and long half-wavelengths, a good correlation is achieved between the NFM and the PM, while the difference between the distortional and global critical stresses obtained from the cFSM and GBT can be explained similarly as in the previous section. Nevertheless, while for the lipped channel section the global critical stress obtained from GBT was slightly lower than that obtained from the NFM and the PM, this time for the single-cell box section it is found to be slightly higher. This suggests that when the simplified constitutive relation in Eq.  is replaced by the exact relation in Eq. , and the membrane transverse and shear strains are allowed, the global critical stress for the lipped channel section is slightly increased while for the box section it is slightly lowered.

It is known from the modal participations shown in Fig. 3.16 and Fig. 4.15 that at half-wavelengths around 1000 mm the column buckles solely in the distortional mode, thus the critical stress obtained from the FSM equals real distortional critical stresses. Therefore, it is worthwhile to verify the distortional critical stresses obtained from various modal decomposition methods against the the critical stress obtained from the FSM at intermediate half-wavelengths, as shown in Fig. 5.8 (d). It can be seen that a better correlation is achieved by the NFM and the PM compared to the cFSM and GBT, which suggests that the proposed methods provide better results of the distortional critical stresses.

(a) Distortional critical stresses over extended range of half-wavelength

(b) Distortional critical stresses (dense discretisation) over extended range of half-wavelength

(c) Distortional critical stresses at intermediate half-wavelengths

(d) Distortional critical stresses verified against the FSM

Fig. 5.8 Distortional buckling critical stress of single-cell box section

(a) Global critical stresses over extended range of half-wavelength

(b) Global critical stresses (dense discretisation) over extended range of half-wavelength

(c) Global critical stresses at intermediate half-wavelengths

Fig. 5.9 Global buckling critical stress of single-cell box section

### Hollow-flange channel section

Finally, the hollow-flange channel section shown in Fig. 5.10 was studied, whose critical stresses for distortional and global buckling are plotted in Fig. 5.11 and Fig. 5.12 respectively. It is important to note that for the cFSM and GBT, the global and distortional modes consist of only those modes obeying the null-membrane-shear-strain criterion. For example, the torsional mode is not considered to be a global mode, and the GBT modes 5 and 6 shown in Fig. 5.13 are not considered to be distortional modes due to the existence of membrane shear strains.

(All dimensions are measured in mm from mid-lines)

Fig. 5.10 Hollow-flange channel section

In Fig. 5.11 a good correlation is again achieved between the distortional critical stresses obtained from the PM and the NFM despite the expected discrepancy at extremely short half-wavelengths. Nevertheless, in addition to the expected minor differences between the results obtained from the cFSM and GBT, and the results obtained from the PM and the NFM at half-wavelengths shorter than 500 mm, significantly different results are seen for longer half-wavelengths.

(a) Distortional critical stresses over extended range of half-wavelength

(b) Distortional critical stresses at intermediate half-wavelengths

Fig. 5.11 Distortional buckling critical stress of hollow-flange channel section

(a) Global critical stresses over extended range of half-wavelength

(b) Global critical stresses at intermediate half-wavelengths

Fig. 5.12 Global buckling critical stress of hollow-flange channel section

| cross -sectional | warping | cross -sectional | warping | cross-sectional | warping |
| --- | --- | --- | --- | --- | --- |
| GBT mode 4 |  | GBT mode 5 |  | GBT mode 6 |  |
| cross -sectional | warping | cross -sectional | warping |  |  |
| GBT mode 7 |  | GBT mode 8 |  |  |  |

Fig. 5.13 GBT modes

The critical distortional modal shape obtained from the NFM at a half-wavelength of 300 mm is presented in Fig. 5.14 (a), which switches to the modal shape shown in Fig. 5.14 (b) at a half-wavelength of 800 mm. The modal shapes obtained from the PM are not shown here since they are identical to NFM. Modes at the same half-wavelengths are shown for the cFSM in Fig. 5.14 (c) and (d), and for GBT in Fig. 5.14 (e) and (f). It can be seen that the modal shapes obtained from various methods are identical at a half-wavelength of 300 mm, with corresponding coinciding curves. On the other hand, the divergence of the various curves at half-wavelengths longer than 800mm is attributed to the differences in modal shapes. The critical distortional modal shapes obtained from the NFM and the cFSM shown in Fig. 5.14 (b) and (d) consist mainly of GBT modes 4 and 6. Mode 4 is thereby more prominently present in the cFSM mode compared to the NFM mode. The coupling of the GBT torsional global and distortional modes leads to the occurence of the second local minimum on the curves associated with the NFM, the PM and the cFSM in Fig. 5.11, while the difference in relative participation of those modes between the cFSM and the NFM (or the PM) is responsible for the difference between the critical stresses at half-wavelengths longer than 800 mm. On the other hand, the critical distortional modal shape obtained from GBT shown in Fig. 5.14 (f) consists predominantly of GBT mode 8. Since the torsional global mode is completely filtered out of the distortional modal shapes, the second minimum is absent in the curve associated with GBT in Fig. 5.11, which diverges from the curves associated with other methods at longer half-wavelengths.

The previous remarks made in section 5.4.1 for global bucking can also be applied to the global mode results shown in Fig. 5.12.

| cross -sectional | warping | cross -sectional | warping |
| --- | --- | --- | --- |
| (a) NFM, L = 300 |  | (b) NFM, L = 800 |  |
| cross -sectional | warping | cross -sectional | warping |
| (c) cFSM, L = 300 |  | (d) cFSM, L = 800 |  |
| cross -sectional | warping | cross -sectional | warping |
| (e) GBT, L = 300 |  | (f) GBT, L = 800 |  |

Fig. 5.14 Switch of dominant modal shape

### Summary

It can be concluded from the comparisons that the critical stresses obtained from the two newly proposed methods are generally very consistent with each other, both correlate well with the critical stresses obtained from the FSM when pure buckling occurs. Moreover, a good correlation was found between the critical stresses obtained from the newly proposed methods and the existing methods, except for: (1) extremely short half-wavelengths, where the critical stresses obtained from the newly proposed methods were significantly lower than those obtained from the existing methods due to the involvement of the Poisson’s effect and shear deformation in the distortional and global buckling modes. (2) distortional buckling of the hollow flange channel section, where a noticeable divergence in results was observed due to the difference in the definition of the distortional mode adopted by different methods.

# Modal decomposition of post-buckling deformed shapes

## Introduction

In the numerical analysis of the post-buckling behaviour of CFS members, the FEM instead of the FSM is often the natural choice of analysis method for its capacity to handle the typical change of the longitudinal shape in the post-buckling stage, and its flexibility in dealing with general geometries and boundary conditions. However, as the FEM itself provides no means of modal identification, the modal decomposition can only be performed qualitatively rather than quantitatively using visual inspection of the deformed shape. Moreover, the modal participation factors are not constant but tend to vary in the post-buckling range. Therefore, in order to obtain a more scientific modal decomposition process and a more accurate evaluations of the modal participation factors in the post-buckling stage, an approach is needed to allow application of the modal decomposition methods to FE shell models.

Relevant research in this area has been done by Schafer [47]. In this method, the buckling modes are obtained from the modal decomposition method in the cFSM before being transformed into FE based modes. One of the main issue with this transformation is the unequal number of DOFs in the FE and FSM models (shown in Fig. 6.1). In FE models, each node is associated with six DOFs, namely the translations and rotations with respect to the three perpendicular axes. In FSM models on the other hand, as only cross-sectional discretisation is conducted, each node (or nodal line) is associated with four DOFs only, specifically the translations along three axes and the rotation about the longitudinal X -axis. The aim of the transformation is to determine the displacements in the FE modes using the information provided by the FSM modes. In Schafer’s method, the nodal displacements in the FE model are determined as the product of the displacement values in the FSM model and the values of the trigonometric longitudinal shape function at the global longitudinal coordinate of the node. In order to determine the displacements associated with the additional DOFs in the FE models, first the global displacements are transformed into local displacements, and then, for all the nodes on a given nodal line, the rotation about the local z -axis θz is neglected while the rotation about the local y -axis θy is determined as the slope of the longitudinal shape function, as shown in Fig. 6.2. Once the FE modes are obtained, the modal participations of these modes are determined by solving an optimisation problem where the error between a linear combination of pure modes and the real deformed shape is minimised. The advantage of this method is that orthogonality among buckling modes is not required. However, it has the disadvantage that the calculated modal participation factors are dependent on the selection of modes.

Fig. 6.1 DOFs involved in FS and FE models

## Transformation from FSM to FE modes

In this study, an alternative approach is proposed using the basic principle of the nodal force method that buckling modes can be generated by imposing precisely defined nodal forces on the FE model. The resulting displacements are determined by performing a linear elastic analysis. The nodal forces are determined in three steps.

Fig. 6.2 Calculation of local rotations

Fig. 6.3 Nodal line loads

### Determination of line loads in the FE model

First, using the NFM the nodal forces associated with the various buckling modes are determined. These are translated in the FE model into line loads imposed on the equivalent of the nodal lines, as shown in Fig. 6.3. It can be proved that if the trigonometric longitudinal shape functions in Eq.  are used, the longitudinal distribution of the loads obeys the same trigonometric functions, namely:

where , , ,  are the longitudinal amplitudes of the line loads qX,i, qY,i, qZ,i and mX,i to be imposed on the i th nodal line in the FE model. qX,i, qY,i, qZ,i and mX,i correspond to the global translations U, V, W and the rotation Θ. In addition, m is the number of half-waves within the member length L.

The proof of Eq.  is given as follows: In a plate with thickness t, the stress resultants (per unit width of plate) are given as follows:

where Nx, Ny and Nxy are the membrane forces, Qx and Qy are the out-of-plane shear forces, and Mx, My and Mxy are the bending moments and torque, respectively.

When generating buckling modes in the FE model, all the loads are imposed on the nodal lines while no surface loads are imposed on the strips. In a geometrically linear analysis, the equilibrium equations of the deformed strips are:

The membrane stress-strain relations take the form:

while the bending moments and torque are linked to the curvatures and twist as follows:

In the above equations, the flexural rigidity of the plate D = Et3/12(1-v2).

The strains can be expressed in terms of displacements using the relations:

along with curvatures and twist:

Substituting Eq.  into Eq.  and Eq., and then into Eq. , Eq.  and Eq.  we obtain:

where , , , , and  are functions representing the cross-sectional distributions of the membrane forces, moments and torque in the i th strip.

(a)

(b)

(c)

Fig. 6.4 Equilibrium at strip junction

At the i th nodal line, equilibrium is examined for an element of length dx, which consists of the nodal line itself and a small portion of the (i -1) th and i th strips with width dy. When dy is chosen to be infinitesimally small compared to dx, the forces and moments applied on the transverse edges of the element vanish. Therefore, the moments in the y -direction (Fig. 6.4 (a)) need to satisfy:

where Myi-1 and Myi are bending moments in the (i -1) th and i th strip respectively, and mX,i is an applied distributed moment, with all resultants considered at the i th nodal line. Since My obeys a sinusoidal longitudinal shape function according to Eq. , we obtain from Eq.  that:

Substituting the last two equations of Eq.  into the last equation of Eq.  yields:

For the same element, equilibrium of the in-plane and out-of-plane forces (Fig. 6.4 (b) and (c)) requires:

where Qyi-1  and Qyi are the out-of-plane shear forces, Nyi-1  and Nyi are the membrane normal forces, and Nxyi-1  and Nxyi are the membrane shear forces of the (i -1) th and i th strip respectively, with all resultants considered at the i th nodal line. αi-1 and αi are the inclination angles of the strips. Considering the longitudinal profiles of Qy, Ny, Nxy indicated by Eq.  and Eq. , it is seen from Eq.  that:

Thus Eq.  has been proved.

### Determination of the nodal loads in the FE model

Second, the equivalent nodal forces and moments of the line loads established in the previous step are determined in the FE model.

Fig. 6.5 Transformation from line loads to nodal forces

For each element, this is done by integrating the product of the distribution load and the shape functions over the element length, as shown in Fig. 6.5. Considering an element with one longitudinal edge coinciding with the i th nodal line, we define first a local coordinate

where x1 and x2 are the global X -coordinates of node 1 and node 2. Substituting Eq.  into Eq. , considering that the shape functions have the form:

the equivalent nodal forces associated with the warping displacement are determined as:

and the equivalent nodal forces and moments associated with the other displacements are determined as:

### Generating FE modes

Third, once the nodal forces are determined, they are applied to the FE model in ABAQUS [1], where the boundary conditions are defined as presented in Fig. 6.6. In the model, edges 1 and 3 are constrained regarding translations in the Y - and Z -directions and rotation about the X -axis. Meanwhile, node 2 is constrained regarding translation along the X -axis. Since the loads applied to the column are in equilibrium, it is sufficient to constrain only one node to avoid overall longitudinal translation of the column.

Fig. 6.6 Boundary conditions applied to the FE model

Fig. 6.7 Flow chart of the mode generation process

After applying the nodal forces to the FE model, a linear static analysis (Nlgeom off) is performed, resulting in the sought FE modal shapes.

The FE buckling modes are generated using a PYTHON script (see appendix A) run in ABAQUS, as shown in Fig. 6.7. First, according to the input variables defining the geometry of the structural member and the material properties, an FE model is established, from which the FE nodal coordinates are extracted. Second, according to the FE nodal coordinates obtained in the previous step the corresponding FSM model is created. The buckling modes of this FSM model are obtained using NFM along with the corresponding nodal forces. Third, the nodal forces associated with the FSM modes are transformed to the corresponding FE nodal forces and applied to the FE model. Fourth, a linear elastic analysis is performed, yielding the deformed shapes of the FE modes. It is noted that in the deformed shapes, due to the constraint placed at node 2, an overall translation along the X -axis is introduced compared to the FSM mode, where the end displacements are equal in magnitude. This is corrected by finding the average of the maximum and minimum values of the longitudinal displacements and deducting it from the deformed shape. An example of a FE generated distortional buckling mode is shown in Fig. 6.8. Finally, the post-processed nodal displacements associated with the FE modes, along with their nodal forces, are output to two .txt files.

Fig. 6.8 FE buckling mode

### Orthogonalisation and normalisation

The work done by the line loads associated with one buckling mode in the displacements associated with another buckling mode is expressed as:

where α is a factor subject to m and n, fipm is the nodal force value corresponding to the i th FSM mode with m half-waves in the longitudinal direction, and djpn is the displacement value corresponding to the j th FSM mode with n half-waves. The proved consistency between the longitudinal shape functions of the nodal forces and displacements is reflected in Eq. , where fpm (x) and fpn (x) are the longitudinal shape functions with m and n half-waves respectively, both associated with the p th DOF. nDOF is the total number of DOFs in the FSM model. It can be seen that the work vanishes when m ≠ n due to the orthogonality between the harmonics and when i ≠ j due to the orthogonality between FSM modes. Therefore, orthogonality of the modes with respect to the global stiffness matrix is in principle satisfied. However, since in the FE model the shape functions of the elements are linear (when using ABAQUS shell element S4R) rather than trigonometric as assumed in the proof, small but acceptable error in orthogonality is incurred.

The FE modes are normalised so that the strain energy resulting from the displacements is unity:

where fi and di are the nodal force and displacement vectors corresponding to the i th FE mode.

## Modal decomposition of post-buckling deformed shapes

### Determination of modal participation

The displacements obtained from an ABAQUS non-linear analysis are measured with respect to the initial position of the nodes, which deviates from the nominal position due to imperfection. Nevertheless, in order to know the contributions of modes to not only the deformed shape, but also the imperfection shape, they are summed up and contained in the vector dFEM representing the displacements of the nodes with respect to their nominal positions. This displacement can be decomposed to a linear combination of buckling modes:

where Hfull is the matrix containing in its columns the buckling modal shapes, equal in number to the DOFs of the FE model, and afull is the vector containing the amplification factors of the modes. However, this complete modal decomposition is impractical due to the typically very large number of FE buckling modes. Moreover, it is unnecessary since the amplification factors of a large number, if not most, of the modes are negligible. For most purposes, only a limited number of modes are relevant and need to be included in the study. Thus, Eq.  is converted to:

where the matrix H and the vector a contain selected buckling modal shapes and their corresponding amplification factors, dr is the vector containing the residual displacements and dapx is the combined modal shape.

Taking advantage of the orthogonality among buckling modes with respect to the global stiffness matrix in the FE model KFEM, the modal amplification factors are simply determined as follows:

where F is the matrix containing in its columns the FE nodal force vectors associated with the FE buckling modes as previously determined in section 6.2. In this method, the amplification factors are independent of the initial selection of modes. Once the amplification factors are determined, the participation factors can be calculated using the energy-based definition in Eq. :

where pi and ai are the participation factor and amplification factor of the i th mode.

### Boundary conditions

Until now only buckling modes associated with sinusoidal longitudinal shape functions are discussed, which is sufficient for the modal decomposition of the deformed shapes of FE models with simple-simple and fixed-fixed boundary conditions. However, it is important to be able to deal with general boundary conditions, including: (1) simple-simple, (2) fixed-fixed, (3) simple-fixed, (4) fixed-free, (5) simple-guided and (6) fixed-guided. With respect to local and distortional buckling, the boundary conditions associated with the FSM displacements and rotation are as shown in Table 6.1, where the tick marks and crosses mean ‘allowed’ and ‘prohibited’ respectively.

Table 6.1 Boundary conditions

|  | U | V | W | Θx | θy |
| --- | --- | --- | --- | --- | --- |
| simple | √ | × | × | × | √ |
| fixed | × | × | × | × | × |
| guided | × | √ | √ | √ | × |
| free | √ | √ | √ | √ | √ |

According to [48], in a version of the cFSM developed for generalised end boundary conditions, the following longitudinal shape functions can be used for the displacements V, W and the rotation Θ, while the derivative of those shape functions is used for the displacement U:

(1) simple-simple:

(2) fixed-fixed:

(3) simple-fixed:

(4) fixed-free:

(5) guided-simple:

(6) fixed-guided:

In cases (1) and (5) each shape function takes the form of a single trigonometric function. In case (3) the shape function is a summation of two sinusoidal functions. Therefore it is equivalent to case (1). In cases (2) and (6) the shape functions are products of two sinusoidal functions. With the help of the product-to-sum identities they are transformed to be equivalent to case (3), except that the components of the shape functions are cosinusoidal rather than sinusoidal. Case (4) is equivalent to case (5) apart from the constant component. It is thus concluded that the deformed shape of FE models subject to any set of boundary conditions can be decomposed solely into buckling modes associated with sinusoidal and cosinusoidal shape functions. The method in which modes associated with sinusoidal shape functions are generated can be slightly altered to generate modes associated with cosinusoidal shape functions. To achieve this it is first noted that the nodal forces in the FSM model remain unchanged and so do the longitudinal amplitudes of the line loads applied to the FE model. This is verified as follows: According to [48], the elements of the stiffness matrices K and G in the FSM are obtained through the following integrations:

where Ym and Yn are shape functions associated with m and n half-waves. It can be seen that if the shape functions Ym = sin (mπx/L) are replaced with Ym = cos (mπx/L), due to the properties of trigonometric functions, the values of the integrations are unchanged. Therefore, the nodal forces obtained from the nodal force method, which makes use of the matrices K and G are unchanged. However, when determining the nodal forces in the FE model the transformation equations in Eq.  and Eq.  need to be changed accordingly. In addition, the boundary conditions of the FE model shown in Fig. 6.6 are altered to those shown in Fig. 6.9. Edges 1 and 3 are constrained with respect to their warping displacements. Meanwhile, Node 2 is constrained with respect to the translations in the Y - and Z -directions. Node 4 is constrained with respect to the rotation about the X -axis. Since the loads applied to the column are in equilibrium, it is sufficient to constrain only two nodes to avoid overall translation and rotation of the column. The reason for separating the two constraints lies in the fact that if all DOFs of one node were constrained, the node would be omitted in the ABAQUS output file, thus causing mistakes when the output file is read by the PYTHON script. Finally the overall translations in the Y - and Z -directions, and the overall rotation about the X -axis in the deformed shapes are corrected, so that the end displacements are equal in magnitude, as in the FSM mode.

Fig. 6.9 Boundary conditions applied to the FE model

## Illustrative examples

In this section the method is illustrated with a number of examples. First, representing the simplest cases, FE models with simple-simple and fixed-fixed boundary conditions are studied using only modes associated with sinusoidal longitudinal shape functions. Thereafter, more complicated cases with general boundary conditions are studied by also using modes associated with cosinusoidal longitudinal shape functions.

Fig. 6.10 Single-cell box section

### Local buckling under simple-simple boundary condition

First, the method is used to study the post-buckling behaviour of a single-cell box (Fig. 6.10) column, with the following dimensions: web width bw = 100, flange width bf  = 100, web plate thickness tw = 2, flange plate thickness tf  = 1 and column length L = 213 (all in mm). The mechanical properties of the material are assumed to be: E = 210000 MPa, ν = 0.3. Both ends of the column are subject to the simple boundary conditions that the plates are free to rotate about the transverse edges, while any transverse, out-of-plane and non-uniform warping displacements are prohibited. The column is loaded by symmetrically applying end shortenings.

This study will focus on local buckling while the effects of other mode types are neglected. Since the cross-sectional deformations obtained from an ABAQUS geometrically non-linear analysis appear to be symmetric with respect to both principle axes of the cross-section, only modes exhibiting this symmetry are selected for the modal decomposition process. These are the first local mode (associated with the minimum critical stress) shown in Fig. 6.11 (a), and the fourth local mode (associated with the fourth smallest critical stress) shown in Fig. 6.11 (b), while the second and third local modes have antisymmetric modal shapes. Since the column length is chosen as three times the critical half-wavelength of the first local mode (associated with the minimum local buckling critical stress in the signature curve), three half-waves are contained in the deformed shape when buckling occurs. In addition, in the post-buckling stage the contributions of modes associated with other harmonics are small. Both of the modes selected in the modal decomposition process are associated with three half-waves and are notated as L1-3 and L4-3. It is noted that because the web is significantly thicker than the flange, only the flange buckles in mode L1-3. In addition to these buckling modes, a uniaxial compressive mode representing the pre-buckled deformed shape is also included, which is generated by simply applying a uniform compressive load to the column shown in Fig. 6.6. Since this uniaxial compressive mode has non-zero longitudinal nodal forces, it is identified as a shear mode according to the criteria of NFM, and is thus orthogonal to the local modes.

| (a) L1-3 | (b) L4-3 |
| --- | --- |

Fig. 6.11 Cross-sectional deformations of selected buckling modes

By performing a buckling analysis in ABAQUS the critical buckling modal shape associated with the lowest critical stress is obtained, in which the participation of mode L1-3 is 95%. The geometric imperfection is then defined as the critical buckling modal shape with a magnitude of 0.1tf (i.e. the maximum deviation from the nominal position is 0.1 mm), as suggested by Zeinoddini [111].

The modal participations are calculated for increasing load and the results are shown in Fig. 6.12. The horizontal axis represents end shortening strains ε (i.e. the end shortening on each side divided by half of the column length), and the vertical axis represents the participation of each mode. The vertical dashed line indicates the critical end shortening strain when buckling occurs. In light of the limited number of buckling modes considered, instead of absolute participations relative values are used calculated by dividing the participation of each mode by the total participation of both modes. It can be seen that the relative participation of mode L1-3, which nearly coincides with the imperfection shape and the buckling shape, is 100% in the pre-buckling stage, and is predominant in the initial post-buckling stage. However, as the load increases, mode L4-3 increasingly participates until finally a state is reached where the relative participations of both modes stabilises. The results are illustrated in Table 6.2, where the cross-sectional deformed shapes are presented along with the corresponding end shortening strains. The deformed shapes are extracted from the cross-section at 1/6 of the column height where the cross-sectional displacements reach maximum values. It can be seen that the deformed shape, initially coinciding with the modal shape of L1-3, evolves towards a combination of L1-3 and L4-3 where the flange and web buckle with equal amplitudes.

Fig. 6.12 Modal participations

Table 6.2 Cross-sectional deformed shapes with increasing load.

| ε (×10-3) | 0 | 0.3 | 0.6 | 0.9 |
| --- | --- | --- | --- | --- |
| Deformed shape |  |  |  |  |
| ε (×10-3) | 1.2 | 1.4 | 1.7 | 1.9 |
| Deformed shape |  |  |  |  |
| ε (×10-3) | 2.1 | 2.2 | 2.3 | 2.4 |
| Deformed shape |  |  |  |  |
| ε (×10-3) | 2.5 | 2.6 | 2.7 | 2.8 |
| Deformed shape |  |  |  |  |

### Local-distortional interactive buckling under fixed-fixed boundary condition

In a second example, the method is used to study the post-buckling behaviour of a lipped channel column (Fig. 6.13), with the following dimensions: web width bw = 119.55, flange width bf = 89.65, lip width d = 4.80, inner radius of the corner r = 1.30, thickness t = 1.085 and column length L = 400 (all in mm). The column is loaded by symmetrically applying end shortenings. In this example both ends are subject to fixed boundary conditions, where plate rotations about the transverse edges are prohibited. Other boundary conditions will be studied in the next section. With the purpose of avoiding the complexity incurred by rounded corners in the original section, the analysis is performed on the equivalent section with sharp corners instead.

(All dimensions are measured from the outer edge of the section)

Fig. 6.13 Lipped channel section

| (a) D | (b) L1 | (c) L3 |
| --- | --- | --- |

Fig. 6.14 Cross-sectional deformations of selected buckling modes

The study will focus on local-distortional interactive buckling while the effect of global buckling is neglected. Since the cross-sectional deformations obtained from an ABAQUS geometrically non-linear analysis appear to be symmetric, only modes exhibiting this symmetry are selected for the modal decomposition process. These are the first distortional mode (D) as shown in Fig. 6.14 (a), and the first and the third local mode (L1 and L3) as shown in Fig. 6.14 (b) and (c). The second local mode has an antisymmetric shape, and is therefore not considered. Since the deformed shape is also longitudinally symmetric, all the modes selected for the modal decomposition process are associated with an odd number of half-waves ranging from 1 to 7. Again, a uniaxial compressive mode is also included, which is orthogonal to the local and distortional modes.

The imperfection is again defined as the critical buckling modal shape (in this case distortional) with a magnitude of 0.1t.

The modal participations for increasing loads are presented in Fig. 6.15, where the horizontal axis represents end shortening strains. The modal participations of individual modes and mode types are presented in Fig. 6.15 (a) and (b) respectively, both divided by the total participation of all selected modes. The vertical dashed line indicates the critical end shortening strain when buckling occurs. It can be seen that the participation of the distortional modes is dominant in the pre-buckling stage and the initial post-buckling stage, which indicates the occurrence of distortional buckling as the critical buckling mode. However, as the load increases, local modes also become involved and local-distortional interactive buckling ensues.  Soon thereafter, the participation of the local modes surpasses that of the distortional modes and eventually the relative participation of both types of modes stabilises. The results are further illustrated in Table 6.3, where the deformed shapes of the column are presented along with the corresponding end shortening strains. It can be seen that, initially, for end shortening strains smaller than  the buckled shape appears to be purely distortional. When the strain is increased to , two buckles emerge in the web between the column ends and mid span, which indicates local buckling. As the load is further increased, the buckles are ‘stretched’ in the transverse direction, transforming their shape from round to elliptical. This is attributed to the involvement of a second local buckling mode such as mode L3.

Table 6.3 Change of displacement pattern with increasing load

| ε (×10-3) | 0 | 0.2 |
| --- | --- | --- |
| Deformed shape |  |  |
| ε (×10-3) | 0.4 | 0.6 |
| Deformed shape |  |  |
| ε (×10-3) | 0.8 | 1.3 |
| Deformed shape |  |  |
| ε (×10-3) | 1.5 | 2.0 |
| Deformed shape |  |  |

- Modal participations of individual modes

(b) Modal participations of mode types

Fig. 6.15 Modal participations (fixed-fixed)

### Illustrative examples with general boundary conditions

Two examples with fixed-fixed (case (2) according to the discussions in section 6.3.2) and fixed-free (case (4)) boundary conditions are discussed. First, for case (2) the column previously considered in section 6.4.2 is studied. And the following longitudinal shape functions was used:

It is noted that when m = 1, the first component of the longitudinal shape becomes a constant.

The modal participations are plotted for increasing loads in Fig. 6.16. When compared to Fig. 6.15, where the decomposition was made in terms of modes with pinned end conditions, it can be seen that the total participation of the distortional modes is consistent despite the use of different basis vectors in the distortional space.

Second, for case (4) the same column is studied. The longitudinal shape function consists of cosinusoidal terms and a constant term:

Case (4) is equivalent to case (5) apart from the constant component. After deducting this constant component, the longitudinal shape function consists of only cosinusoidal terms. Meanwhile, the deformed shape is transformed from Fig. 6.17 (a) to (b).  In order to generate the cosinusoidal modes associated with 0.5, 1.5, 2.5… half-waves, the boundary conditions in the FE models are altered to those shown in Fig. 6.18. Edge 1 is constrained with respect to the cross-sectional displacements while edge 2 is constrained with respect to the warping displacements.

(a) Modal participations of individual modes

(b) Modal participations of mode types

Fig. 6.16 Modal participations (fixed-fixed)

Regarding the warping displacement at the free end, two examples are studied with different boundary conditions. In the first example, the end is completely free and any warping displacement is allowed. On the other hand, in the second example a rigid end plate is added to the free end of the cantilever column so that any non-uniform warping displacements are prohibited. This additional constraint introduces more shear deformation in the deformed shape of distortional and global buckling, where the warping displacements are non-uniform. The shear deformation can be reflected in the increased modal participations of shear modes, if they are included in the set. However, according to the feature of our modal decomposition method, it will not affect the modal participations of other modes.

| (a) before | (b) after |
| --- | --- |

Fig. 6.17 Shifting of the deformed shape

Fig. 6.18 Boundary conditions applied to the FE model

The free end in the first example is enforced in ABAQUS by using a structural distributing coupling between the column end section (slave) and the reference point (master). A structural distributing coupling distinguishes itself from a kinematic coupling in that the relative movement between the master and slave nodes is not strictly prohibited. Instead a load is applied to the slave nodes to prevent their average movement relative to the master node.  As a result, the non-uniform warping displacements resulting from buckling are allowed at the free end, while the column can still be loaded by applying end shortenings.

(a) Modal participations of individual modes

(b) Modal participations of mode types

Fig. 6.19 Modal participations (fixed-free without end plate)

(a) Modal participations of individual modes

(b) Modal participations of mode types

Fig. 6.20 Modal participations (fixed-free with end plate)

The geometric imperfection is again defined as the critical buckling modal shape (which in this case is a coupled local and distortional mode) with a magnitude of 0.1t.

The modal participations are plotted for increasing load for both examples in Fig. 6.19 and Fig. 6.20 respectively. It can be seen that in Fig. 6.19 that the distortional component of the deformed shape is consistently dominant, while in Fig. 6.20 the local component has more contribution. This difference is confirmed by inspection of the final deformed shapes (ε = 4 × 10-3) shown in Fig. 6.21. In case (a) pure distortional buckling is observed while in case (b) local-distortional interactive buckling takes place.

| (a) without end plate | (b) with end plate |
| --- | --- |

Fig. 6.21 Final deformed shapes

## Evaluation of the error

If the deformed shape is approximated by a limited number of buckling modes, as in Eq. , an error exists, which can be evaluated either based on displacements, using Eq. , or based on energy, using Eq. .

In the above equations dr represents the residual displacements, as previously defined in Eq. . The energy-based error indicates the contribution of the strain energy resulting from dr in the total strain energy present in dFEM, which is the deformed shape to be decomposed.

First, the error is calculated, using both definitions, for the local buckling example shown in Fig. 6.22 (a). It is shown in Fig. 6.12 that in the post-buckling range the error is increasing steadily as loading proceeds, reaching a maximum of 19.3% based on displacements and 61.4% based on energy within the range of end shortening strains considered. The accuracy of the approximation can be evaluated visually by inspection of the deformed shapes shown in Fig. 6.23, among which the top one represents the combined modal shape dapx, the middle one represents the residual displacement dr and the bottom one represents the original deformed shape dFEM. The good agreement observed between dapx and dFEM indicates that the error tends to be overestimated if calculated based on energy. An in-depth comparison between the displacements of dapx and dFEM reveals that, while the differences regarding the cross-sectional displacements are negligible, a noticeable difference regarding the warping displacements is observed in Fig. 6.24. The difference at the ends is attributed to the discrepancy between the boundary conditions applied to the FE model when generating the uniaxial compressive mode representing the pre-buckled deformed shape, and when performing geometrically non-linear analysis. In the latter case, end plates are attached to the column prohibiting non-uniform end warping displacements in dFEM. On the other hand, the difference near midspan is due to the second-order geometric effect of the out-of-plane displacement on the warping displacement. In other words, out-of-plane bending of the plates causes an axial shortening of the plate. This effect is considered in dFEM , which is generated using a geometric non-linear analysis, but neglected in dapx as a linear analysis is performed when generating the buckling modes. These non-uniform warping displacements of dFEM inadvertently generate a significant proportion of the strain energy when this strain energy is calculated based on the elastic stiffness matrix KFEM. This yields an unduly higher strain energy associated with dFEM compared to the energy associated with dapx, despite the good correlation between the deformed shapes. Therefore, the error is overestimated.

The situation is similar for the distortional buckling example, where the displacement-based and energy-based errors reach a maximum of 13.0% and 90.7% respectively, within the range of end shortening strains considered in Fig. 6.15. Judging from the deformed shapes shown in Fig. 6.25, the overestimation of the error when calculated based on energy is even more. Again, the pronounced difference in the distributions of the warping displacement along the cross-section, witnessed in Fig. 6.26 for dapx and dFEM can be held responsible for this.

(a) Local buckling of box section

(b) Local-distortional interactive buckling of lipped channel section

Displacement-based error          Energy-based error

Fig. 6.22 Error when only conventional modes are involved

| dapx |  |  |
| --- | --- | --- |
| dr |  |  |
| dFEM |  |  |

Fig. 6.23 Comparison between the total displacement magnitudes

| dapx |  |  |
| --- | --- | --- |
| dFEM |  |  |

Fig. 6.24 Comparison between the warping displacements

| dapx |  |  |
| --- | --- | --- |
| dr |  |  |
| dFEM |  |  |

Fig. 6.25 Comparison between the total displacement magnitudes

| dapx |  |  |
| --- | --- | --- |
| dFEM |  |  |

Fig. 6.26 Comparison between the warping displacements

It is concluded from the two examples that the second-order membrane deformations need to be considered if the energy-based error is to be evaluated correctly. This can be done by introducing the transverse extension and shear modes into the modal decomposition process. It is noted that the uniaxial compressive mode representing the pre-buckled deformed shape overlaps the shear modes newly added to the set. Therefore, in order to regain orthogonality among all selected modes, the contributions of the shear modes need to be deducted from the uniaxial compressive mode:

where dUA is the original uniaxial compressive modal shape,  is the same modal shape after processing. Hs is the matrix containing in its columns the shear modal shapes. KFEM is the global stiffness matrix of the FE model.

After including all transverse extension and shear modes associated with a number of half-waves ranging from 1 to 7, the maximum energy-based error is reduced from 61.4% to 17.7% for the first example regarding local buckling, and from 90.7% to 31.3% for the second example regarding local-distortional interactive buckling. Meanwhile, the maximum displacement-based error decreases slightly from 19.3% to 18.3% for the first example and from 13.0% to 10.7% for the second example.

(a) Local buckling of box section

(b) Local-distortional interactive buckling of lipped channel section

Displacement-based error          Energy-based error

Fig. 6.27 Error when transverse extension and shear modes are included

After including the transverse extension and shear modes, the errors of the other three examples with general boundary conditions are as shown in Fig. 6.28. In the example with fixed-fixed boundary conditions the maximum displacement-based and energy-based errors are 12.1% and 30.4%, respectively. In the example with fixed-free boundary conditions without an end plate at the free end, the maximum displacement-based and energy-based errors are 20.8% and 24.0%, respectively. In the example with fixed-free boundary conditions with an end plate at the free end, the maximum displacement-based and energy-based errors are 20.7% and 46.4%, respectively.

(a) Fixed-fixed

(b) Fixed-free without end plate at the free end

(c) Fixed-free with end plate at the free end

Displacement-based error          Energy-based error

Fig. 6.28 Error of the examples with general boundary conditions

## Effect of imperfection

### Introduction

In this section the effect of imperfections on the participation factors in the post-buckling stage is studied for a column with fixed-fixed boundary conditions. The modal decomposition process is performed repeatedly for a variable geometric imperfection. The columns taken in section 6.4.1 and 6.4.2 as examples are thereby reconsidered. In the first example, only the two local modes L1 and L4, both associated with three half-waves, are considered in the modal decomposition. Combinations of these two modes also constitute the imperfection shapes. In order to generate the imperfection shapes of the FE model, the buckling modes are first normalised so that the maximum cross-sectional displacement is unity before being combined in various ratios using a method proposed by Dinis and Camotim [25]. Fig. 6.29 (a) provides a visualisation of the imperfection shapes: each point on the unit radius circle defined by the angle α measured counter-clockwise from the horizontal axis represents a particular imperfection shape generated by combining the two modes in a specific proportion. The points associated with α = 0° and α = 180°, for example, represent the imperfection shapes consisting exclusively of mode L1 with a positive and negative amplitude, respectively. Similarly the points associated with α = 90° and α = 270° represent the imperfection shapes corresponding to the pure mode L4. The points associated with angles in-between represent imperfection shapes with contributions from both modes.  In the second example, the FSM modes D1, L1and L3 are considered in the modal decomposition with different longitudinal harmonics. While the effect of mode L3 is negligible, the two modes D1 and L1 are used to generate the imperfection shapes. The longitudinal shapes of both modes are made correlating to the buckled shapes under the same fixed-fixed boundary condition, with high-frequency harmonics omitted. The resulting longitudinal shapes are plotted in Fig. 6.30.

(a)                                                                         (b)

Fig. 6.29 Imperfection shapes as combinations of buckling modes

Fig. 6.30 Longitudinal shapes of modes constituting imperfection shapes

### Local buckling

The results for local-local interaction of box-sections are shown in Fig. 6.31, where the vertical axis represents the participation of mode L1 relative to mode L4, while the horizontal axis represents the end shortening strain. Local buckling is symmetric, in the sense that the buckling behaviour is identical regardless of the direction in which out-of-plane displacements develop. Therefore, only imperfection shapes corresponding to angles ranging from 0° to 180° are studied, which amount to the same effect as angles ranging from 180° to 360°. The right and left halves of the graph show the results associated with imperfection shapes with angles ranging from 0° to 90° and from 90° to 180°, respectively. The curves associated with different angles are distinguished by the different colours introduced in Fig. 6.29. For example, the red curves in both halves represent the same angle of 90°. The green curve in the left half represents an angle of 180°, while the green curve in the right half represents an angle of 0°. In addition, the post-buckling deformed shapes at ε = 5.6 × 10-3 for various imperfections are tabulated in Table 6.4.

The column buckles initially in mode L1, while mode L4 becomes involved at a later stage and interactive buckling takes place. Therefore, buckling associated with mode L1 is referred to as primary buckling, while buckling associated with mode L4 is designated secondary buckling. It can be seen in Table 6.4 that the direction in which primary buckling occurs correlates with the sign of the imperfection component associated with mode L1. For angles ranging from 0° to 60°, the imperfection component is positive, and primary buckling occurs with the flanges bulging outwards. On the other hand, for angles ranging from 120° to 180° the imperfection component is negative and primary buckling occurs with the flanges bulging inwards. In addition, for an angle of 90° the imperfection component associated with mode L1 vanishes, and primary buckling occurs in the same direction as for angles ranging from 0° to 60°.  However, unlike in primary buckling, the direction in which secondary buckling occurs is independent of the sign of the imperfection component associated with mode L4, but is subject to the direction in which primary buckling occurs. For angles ranging from 0° to 90°, the primary buckling occurs in the positive direction, and secondary buckling also occurs in the positive direction, which means that the flanges bulge inwards. In this case, the direction of secondary buckling is consistent with the sign of the corresponding imperfection component L4, which is always positive within the range of angles studied. For angles ranging from 120° to 180°, primary buckling occurs in the negative direction, and secondary buckling consequently also occurs in the negative direction, meaning that the flanges bulge outwards. In this case, however, the direction of secondary buckling is opposite to the sign of the corresponding imperfection component L4.

Fig. 6.31 Modal participation factors subject to imperfection

It can be seen in the right half of Fig. 6.31 that, although the curves start at different initial values, indicating different participations of mode L1 in the imperfection, they eventually converge to approximately the same value of around 0.3. In addition, the curves are ‘laminated’ which means that they do not intersect. This reveals that, while the participation of mode L1 at buckling and in the initial post-buckling stage positively correlates with its participation in the initial imperfection, the participation far into the post-buckling stage is independent of the initial imperfection. Even if the imperfection shape consists of only one mode, the other mode will still participate as the column buckles. In the left half of Fig. 6.31 similar convergence of the curves is seen, although intersections do occur due to the fact that the direction of secondary buckling is opposite to the sign of the corresponding imperfection component. Therefore, the overall conclusion can be drawn that the participation of mode L1 in the advanced post-buckling stage is independent of the imperfection.

Table 6.4 Post-buckling deformed shapes subject to imperfection

| Angle | Deformed shape | Angle | Deformed shape | Angle | Deformed shape |
| --- | --- | --- | --- | --- | --- |
| 0° |  | 30° |  | 45° |  |
| 60° |  | 90° |  | 120° |  |
| 135° |  | 150° |  | 180° |  |

### Local-distortional interactive buckling

The results for local-distortional interaction of a lipped channel are shown in Fig. 6.32, where the vertical axis represents the participation of the distortional modes. Since distortional buckling is asymmetric when non-uniform warping displacements are prohibited [81], it is necessary to study the full set of imperfection shapes corresponding to angles ranging from 0° to 360°. The four quarters of Fig. 6.32, starting from the top-right and moving counter-clockwise represent imperfection combinations corresponding to angles α ranging from 0° to 90°, 90° to 180°, 180° to 270° and 270° to 360°. In addition, the post-buckling deformed shapes at  for various imperfections are tabulated in Table 6.5.

The column initially buckles in the distortional mode before local modes become involved and interactive buckling takes place. Therefore, primary buckling is distortional buckling, while secondary buckling is local buckling. It can be seen in Table 6.5 that the direction in which the primary buckling occurs correlates primarily to the sign of the corresponding distortional imperfection component, but is also affected by the sign of the local imperfection component. For angles ranging from 0° to 30°, and from 240° to 360° (0°), primary buckling occurs in the positive direction, meaning that the flanges of the section opens up. Contrarily, for angles ranging from 45° to 210° primary buckling occurs in the negative direction and the section closes up with the flanges moving inwards. In particular, for an angle α = 225° primary buckling occurs in the positive direction but the longitudinal shape is different from that in all other cases. Again the direction in which secondary buckling occurs is subject to the direction of primary buckling. Thus the deformed shape of the column without sign is roughly consistent for the whole range of angles except when α = 225°, which is veriefied by Table 6.5.

Fig. 6.32 Modal participation factors subject to imperfection

As previously addressed, this example differs from the last one mainly because distortional buckling is asymmetric. Therefore, the direction in which the out-of-plane displacement develops will affect the buckling behaviour, including the total participation of the distortional modes. When comparing Table 6.5 with Fig. 6.32, it can be seen that for positive distortional buckling the participation of the distortional modes will reach a final steady value of approximately 0.3, while for negative distortional buckling the final value is around 0.2. For an angle α = 225°, while the longitudinal shape is different from all the other cases, the final value is close to 0.35.

Table 6.5 Post-buckling deformed shapes subject to imperfection

| Angle | Deformed shape | Angle | Deformed shape |
| --- | --- | --- | --- |
| 0° |  | 30° |  |
| 45° |  | 60° |  |
| 90° |  | 120° |  |
| 135° |  | 150° |  |
| 180° |  | 210° |  |
| 225° |  | 240° |  |
| 270° |  | 300° |  |
| 315° |  | 330° |  |

Note: deformations are not illustrated at the same scale

### Conclusion

It is concluded that for local buckling, the initial imperfection does not affect the modal participations in the post-buckling stage while for distortional buckling it does. This is due to the fact that distortional buckling is asymmetric so that the buckling behaviour varies with the direction in which the out-of-plane displacement develops, which is dependent on the initial imperfection.

# Modal decomposition of geometric imperfections

## Introduction

This chapter provides a methodology for the characterisation of geometric imperfections, which provides a foundation for the computational modelling of CFS members. Rather than determining deterministic imperfections from simple empirical rules, a probabilistic concept is introduced leading to a more scientific characterisation of geometric imperfections. Spectrums of the imperfection magnitudes against the buckling mode half-wavelengths are established with the purpose of creating random imperfections for the computational modelling process. First, 3D imperfection shapes of CFS specimens are measured. Then, the spectrums of the imperfection magnitudes associated with various buckling modes, are obtained by performing a modal decomposition of the measured imperfection shape.

## Imperfection measurements

### Equipment

The geometric imperfections of 88 CFS specimens were measured at the University of Sheffield. The measuring facility was similar to the one used by Schafer [70]. The difference resided in the fact that the direct-current differential transformer (DCDT) was replaced by a laser transmitter/receiver, which continuously measured distance to the face of cold-formed steel specimens placed on a stable table. Readings were taken every 1 mm along the specimen length. The laser transmitter/receiver was driven by electric motors and could move in a horizontal plane in two perpendicular directions. The electric motors were controlled by a computer programme so that the laser transmitter/receiver could move according to a predefined pattern. The complete imperfection measurement set-up is shown in Fig. 7.1. In order for the imperfection measurements to be accurate and reliable, the frame itself has to be perfectly flat. In a first step, this was ensured roughly by calibrating the six adjustable supports of the frame using a dumpy level. Second, any residual deflection of the frame was detected by measuring a liquid level. Milk was used for this purpose. The imperfections of the frame itself were then deducted from the imperfection measurements. In order to estimate the measurement error, the imperfections along a longitudinal line on the specimen was measured. Then the specimen was flipped and the same line was measured again. By comparing the imperfection shapes obtained from the two measurements, it was found that the maximum difference was around 0.1 mm, which indicated a measurement error of around 0.05 mm.

Fig. 7.1 Imperfection measurement set-up

### Specimen dimensions

The specimens with five different cross-sections were measured, including four plain channel sections and one lipped channel section. The specimen geometries are tabulated in Table 7.1, with the notations defined in Fig. 7.2. For each geometry, several specimens were available.

Table 7.1 Nominal dimensions of specimens

| Section | Section type | h (mm) | b (mm) | c (mm) | t (mm) | L (mm) | Number of specimens |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T12012 | Plain Channel | 120 | 40 | - | 1.2 | 3000 | 26 |
| T13014 | Plain Channel | 130 | 36 | - | 1.4 | 3000 | 12 |
| T15414 | Plain Channel | 154 | 54 | - | 1.4 | 1800 | 4 |
|  |  |  |  |  |  | 2500 | 8 |
| T7914 | Plain Channel | 79 | 37 | - | 1.4 | 1800 | 4 |
|  |  |  |  |  |  | 2500 | 9 |
| S11012 | Lipped channel | 110 | 50 | 9.8 | 1.2 | 3000 | 25 |

(a) Lipped-channel section                   (b) Plain channel section

Fig. 7.2 Definition of notations

### Imperfection measurements

For each of the specimens, measurements were taken along evenly spaced lines on each of the plate segments, as shown in Fig. 7.3 (a) (b) and Fig. 7.4. The lines close to the corners were measured 5mm away from the corners in order to ensure that the measurements were taken on the flat plate. For the webs and flanges, four lines were measured. Using the imperfection values along these lines, a 2D imperfection profile was obtained for each cross-section along the length. Imperfection values in between the measured lines were interpolated using a third degree polynomial. On the other hand, since the lips were relatively too narrow, only two lines were measured, and linear interpolation was used in the transverse direction.

In order to assemble the 2D imperfect geometries of the web, flanges and lips into a 3D imperfect geometry of the entire specimen, the corners were also measured by tilting the specimen and moving the transmitter/receiver in the transverse direction along a single line close to the edge of the specimen, as shown in Fig. 7.3 (d). Due to the limited range of the laser when measuring distances, when measuring the web-flange corner, it was impossible to enclose all four nodes of the web and all four nodes of the flange in one measurement. Therefore, it was decided to enclose two nodes on the web close to the corner and all four nodes on the flange. An exception was made for sections T15414 and S11012. For these sections, only three nodes on the flange close to the corner could be enclosed due to the large flange width. For section T15414, the web was so wide that it was impossible to reach the second node on the web. Therefore two additional lines were measured on the web between lines 1 and 2, and 3 and 4. These two lines were used only for the purpose of assembling the imperfect geometry of the flange and the web, while the imperfect geometry of the web itself was still defined using only the data measured along the four original lines.

(a) Web                                            (b) Flange

(c) Lip                                              (d) Corner

Fig. 7.3 Imperfection measurements

(a) Lipped-channel section              (b) Plain channel section

Fig. 7.4 Imperfection measurement locations along cross-section

## Post-processing of the data

### Assembly of the imperfect geometry

In order to explain the assembly process of the imperfect geometries of the different plate segments, an example is given regarding the right half of a sample specimen as shown in Fig. 7.5. A plain channel is considered to facilitate the explanation, one end section of the specimen is marked ‘side 1’, while the other is marked ‘side 2’. The imperfect geometry consists of four components, including the imperfect geometry of the right half of the web and the right flange, and the imperfect geometry in the transverse direction of the corners on side 1 and side 2. Since the imperfect geometry of the corner includes two nodes on the web and all four nodes on the flange, there exist 12 pairs of matching nodes in total, among which four pairs connect the web and the corner profile, while eight pairs connect the corner profile and the flange. The total error of assembly is the root mean square of the distances between the matching nodes:

The aim is to find the optimal relative position of the four components of the imperfect geometry so that the total error is minimised.

Reference lines are needed to represent the positions of the imperfect geometry, as shown in Fig. 7.5 with dashed lines. For the imperfect geometry of the right half of the web, the reference line is drawn between the two nodes on lines 5 and 6 in Fig. 7.4 (b) at each specimen end. For the imperfect geometry of the right flange, the reference line is defined by connecting the two nodes on lines 1 and 4 in Fig. 7.4 (b) at each specimen end. In Fig. 7.5 (b), the inclination angles of the reference lines of the right half of the web with respect to the horizontal axis on side 1 and side 2 are θ1 and θ2 respectively. In Fig. 7.5 (c), the angle between the two flange reference lines on side 1 and side 2 is Δβ. In Fig. 7.5 (d), the corners are placed relative to the web so that the angles between their reference lines on side 1 and side 2 are η1 and η2 respectively. In Fig. 7.5 (e), the flange is placed relative to the corners so that the angles between their reference lines are η3 and η4, and the distances are d3 and d4 respectively. In Fig. 7.5 (f) and (g), the opening angles of the corner on side 1 and side 2 are α1 and α2 respectively. For the flange part of the corner on side 1, the deviations of the two intermediate nodes from the reference line are p1 and p2, and for the flange the deviations are p1′ and p2′. The corresponding deviations on side 2 are p3, p4, p3′ and p4′. We define Δpi = pi′ − pi (i = 1,2,3,4).

The total error consists of four components, namely the errors between the web and the corner profile on side 1 (E1) and on side 2 (E2), and the errors between the corner profile and the imperfect flange geometry on side 1 (E3) and on side 2 (E4). The squared errors can be expressed as:

And

where b1 and b2 are the spacing between two measurement lines on the web and the two lines on the flange respectively, as shown in Fig. 7.3 (a) (b). It is noted that, while η1, η2, η3, d3, d4 are independent variables, η4 is dependent and can be expressed as

The optimal values of the independent variables are determined by setting the derivatives of the total squared error E equal to zero:

Once the relative positions of the components of the imperfect geometry are determined, the real positions of the nodes are taken as the midpoint of two matching nodes. This procedure of assembling the imperfect geometry is repeated for the left flange and for the lips when dealing with lipped channel sections, so that the complete imperfect geometry of the specimen can be determined. Finally, in order to compare the imperfect geometry with the nominal (perfect) geometry, the overall position of the former has to be defined in 3D space with respect to the latter. This relative position is defined so that the imperfection values of the web at three of the four corners are equal to zero, as shown in Fig. 7.5 (a). It will be shown later that any effect on the imperfection shape caused by different relative positions can be eliminated during the post-processing of the data before modal decomposition is performed.

The average errors of the assembly of the imperfect geometry for various sections are listed in Table 7.2. Considering a measurement error of around 0.05 mm, the error of assembly is satisfactory.

Table 7.2 Errors of the assembly of the imperfect geometry

| Section | T12012 | T13014 | T15414 | T7914 | S11012 |
| --- | --- | --- | --- | --- | --- |
| Average error (mm) | 0.05 | 0.02 | 0.04 | 0.06 | 0.07 |

(a)

(b)                                                                         (c)

(d)                                                                         (e)

(f)                                                                         (g)

Fig. 7.5 Assembly of the imperfect geometries

### Calculation of imperfection magnitudes

Deviations of the nodes from their positions in the nominal geometry are calculated based on the imperfection values as shown in Fig. 7.4, and are determined in the global coordinate system by the translations U, V, W and the rotation Θ. Among the displacements, the warping displacements U are indeterminate. However, since the corresponding nodal forces vanish for local, distortional and global modes according to the assumptions of the nodal force method, they have no effect on the modal decomposition process (as will be explained later) and they can thus be set equal to zero. The translations and can be easily determined from the imperfection measurements by performing simple coordinate transformations. Meanwhile, the rotations Θ are calculated by assuming a shape function for the out-of-plane displacements of each plate along the cross-sectional mid-line, which is a third degree polynomial for the web and flanges, while being linear for the lips.

Fig. 7.6 Calculation of node rotations

For the lipped channel sections with 16 points of measurement, in Fig. 7.6 the local rotation angles θ1, θ21 ~ θ24, θ31 ~ θ34, θ41 ~ θ44 and  are calculated by finding the derivatives of the assumed shape functions. Thus, the global rotations of the nodes are calculated as:

The nodal deviations of all sampling points are calculated and contained in a matrix D = [ dij ], where the row label i indicates the cross-sectional DOF, while the column label j indicates the longitudinal sampling point. For example, the row corresponding to the i th DOF is:

The nodal deviations associated with one cross-sectional DOF are shown schematically in Fig. 7.7 (a) along the specimen length.

Since the longitudinal sampling interval of the imperfect geometry is constant, the modal decomposition of the imperfection shape is performed in a way similar to the discrete Fourier transform (DFT), which uses sine and cosine waves with even numbers of half-waves. However, practically the magnitude of the global modes associated with one single half-wave is important since this is the shape of the critical flexural mode. The contribution of this mode would not be obtained if even numbers of half-waves were used. Therefore, sine and cosine waves with odd numbers of half-waves are used instead, as shown in Fig. 7.7 (c) (d) (e) (f). The selected set of harmonics results in an approximated imperfection shape with equal and opposite imperfection values at both ends, which is usually not the case for the measured imperfection shape. This difference in imperfection values at the ends results in unfavourable sharp bends in the approximated imperfection shape, as shown in Fig. 7.8 (a). In order to solve this problem, the imperfection shape is pre-processed by deducting a longitudinally constant component so that the end values become equal and opposite, as shown from Fig. 7.7 (a) to (b). The effectiveness of the pre-processing process is verified in Fig. 7.8 (b), where the sharp bends in the approximated imperfection shape disappear. This pre-processing of the imperfection does not affect the load carrying capacity of the specimen, which is not sensitive to a longitudinally constant imperfection shape. Another benefit of the pre-processing process is that any effect caused by different relative positions of the imperfect geometry with respect to the nominal geometry is eliminated, which justifies the decision previously made to set the imperfection values of three nodes at the web corners equal to zero.

The normalised nodal force vector associated with a particular mode is then obtained from the FSM using the NFM modal decomposition method and then is expanded into a matrix F =[ fij ] by multiplying the nodal force values with longitudinal harmonics. Again, in F the row label i indicates the cross-sectional DOFs, while the column label j indicates the longitudinal sampling points. For example, the row corresponding to the i th DOF in F is:

The nodal forces associated with one cross-sectional DOF are shown schematically in Fig. 7.7 (c) - (f). Due to the orthogonality of the buckling modes with respect to the global stiffness matrix K, as explained in chapter 6, the modal participation of this particular mode can be calculated as:

Fig. 7.7 Sampling of imperfection and nodal forces

(a) Before pre-processing

(b) After pre-processing

Fig. 7.8 Effectiveness of the pre-processing process

To achieve modal decomposition of the imperfections in the channel under consideration, six FSM modes were considered, including the critical and second critical local modes L1 and L2, the critical and second critical distortional modes D1 and D2 (only for lipped channel sections), the flexural global modes G1 and G2 and the torsional mode G3, as shown in Fig. 7.9. Each FSM mode is associated with multiple harmonics.

The modal participation factors are then transformed to modal magnitudes. For the local and distortional modes the maximum cross-sectional displacement is divided by the plate thickness δ0/t, for the flexural global modes the maximum overall translation is divided by specimen length δ0/L, and for the torsional mode the maximum overall rotation is divided by specimen length θ0/L (measured in deg/m).

| L1 | L2 | D1 | D2 | G1 | G2 | G3 |
| --- | --- | --- | --- | --- | --- | --- |

Fig. 7.9 Modes involved in the modal decomposition of imperfection shape

### Results

The modal magnitudes of various modes are plotted against the number of half-waves for section T12012 in Fig. 7.12. The plots for sections T13014, T15415 (L=1800, L=2500), T7914 (L=1800, L=2500), and S11012 are included in appendix B.

The goal is to establish a database of imperfection magnitudes which can be used in the modelling of geometric imperfections of a specimen with general geometry. In light of this, it is important to make the imperfection data associated with different specimen lengths comparable to each other. Therefore, they are rearranged according to the ‘relative half-wavelength’, which is defined as the critical half-wavelength of the concerning buckling mode (e.g.  for the critical local mode) divided by the half-wavelength L of the harmonic. For the local and distortional modes, the critical half-wavelength corresponds to the minimum critical stress in the signature curve, as shown in Fig. 7.10. For the global modes, since the signature curve is monotonically decreasing, the critical half-wavelength equals the specimen length, and thus, the relative half-wavelength equals the number of half-waves. The calculated critical half-wavelengths of local and distortional modes of the five cross-sections under consideration are tabulated in Table 7.3. For local and distortional imperfections, the reason why the relative half-wavelength is used to present the data, rather than the absolute half-wavelength or the number of half-waves, can be explained by two examples. First, we consider two specimens with identical cross-sections, but one specimen is two times as long as the other one. In this case only imperfection data associated with the same half-wavelength are comparable, such as in Fig. 7.11 (a) and (b). Therefore, the half-wavelength is preferred over the number of half-waves. On the other hand, consider two other specimens with all the geometric dimensions of one specimen twice as large as those of the other one. In this case only imperfection data associated with the same number of half-waves are comparable, such as in Fig. 7.11 (a) and (c). In this case, the number of half-waves is preferred over the half-wavelength. In can be concluded that neither the number of half-waves nor the half-wavelength is universally preferred. However, this problem is resolved if the relative half-wavelength is used. In the first example, in Fig. 7.11 (a) and (b), since the cross-section is identical, the critical half-wavelengths are equal. Thus, comparable imperfection data associated with the same half-wavelength also have the same relative half-wavelength. In the second example, since the cross-sectional dimensions are doubled, the critical half-wavelengths are also doubled. Thus, comparable imperfection data associated with the same number of half-waves also have the same relative half-wavelength. In both cases the imperfection data associated with the same relative half-wavelength are comparable. The imperfection data points of various sections are combined and plotted in Fig. 7.13.

Fig. 7.10 Critical half-wavelengths determined from the signature curve

Table 7.3 Critical half-wavelength for local and distortional modes

| section | (mm) | (mm) | (mm) | (mm) |
| --- | --- | --- | --- | --- |
| T12012 | 124 | 85 | - | - |
| T13014 | 128 | 81 | - | - |
| T15414 | 164 | 115 | - | - |
| T7914 | 96 | 72 | - | - |
| S11012 | 83 | 49 | 336 | 268 |

Fig. 7.11 Comparable imperfection data

Fig. 7.12 Imperfection modal magnitudes of section T12012

Fig. 7.13 Imperfection modal magnitudes of various sections

Using these data points it is possible to obtain the mean and standard deviation of the imperfection magnitudes. For each buckling mode, this is achieved by dividing the data into 10 groups according to the x -axis value from the lowest to the highest (i.e. the relative half-wavelength for local and distortional buckling, or the number of half-waves for global buckling). Within each group, the mean and standard deviation of the data are obtained, as shown in Fig. 7.14, where the red solid dots represent the mean values of the imperfection magnitudes, while the red hollow dots represent the mean values plus/minus the standard deviations. Next interpolation can be performed between the points using the following formula:

This produces the red solid lines representing the mean values, and the red dashed lines representing the mean values plus/minus the standard deviations.

Fig. 7.14 Mean and variance of imperfection modal magnitudes

The 3D imperfection shape associated with the i th mode can be expressed as follows:

Fig. 7.15 Statistical distribution of the phase angle

Where A is the imperfection magnitude, which can be determined from Fig. 7.14 and is always positive,  is a function of the transverse local coordinate representing the cross-sectional modal shape of the i th mode, n is the number of half-waves, x is the longitudinal coordinate, L is the specimen length, and φ is the phase angle ranging from 0 to 2π. The statistical distribution of φ is shown in Fig. 7.15. Contrary to the uniform distribution suggested by Zeinoddini [111], it is clearly seen that the phase angle has significantly greater chance of taking a value close to 0, π or 2π, and slightly greater chance of taking a value close to π/2 or 3π/2. Since the probability distribution is symmetric with respect to π/2, π and 3π/2, it is converted to the distribution shown in Fig. 7.16 (a) with φ ranging from

(a)                                                                (b)

Fig. 7.16 ‘Folded’ Statistical distribution of the phase angle

0 to π/2, by folding over the probability mass corresponding to other values of φ. The probabilistic density function of φ is assumed to be polynomial, and is plotted in Fig. 7.16 (b). Therefore, φ in Eq.  is determined as follows:

where φr is a random value obeying the probabilistic density function in Fig. 7.16 (b), while r1 and r2 are two independent random values being either 0 or 1.

In summary, the imperfection of a specimen can be simulated statistically taking the following steps. First, it is decided which imperfection shapes associated with which FSM modes and harmonics are to be involved in the simulation. Second, the critical half-wavelengths of the local and distortional modes are calculated, along with the relative half-wavelengths for different numbers of half-waves. Third, for each mode a random number is generated for the imperfection magnitude A in Eq. , using the obtained mean and standard deviation values corresponding to either the relative half-wavelength for the local and distortional modes, or the number of half-waves for the global modes. Fourth, the phase angle is determined using Eq. , and the imperfection shape associated with each mode is determined from Eq. . The final imperfection shape of the specimen is the combination of the imperfection shapes associated with the various modes.

# Calculation of initial post-buckling stiffness

## Introduction

This and the following two chapters discuss the application of the modal decomposition method to the design of CFS columns undergoing coupled instabilities. The development of the design approach is based on the same observation exploited in the DSM, that, while in general the ultimate load can only be obtained computationally by performing time-consuming non-linear inelastic analysis, the critical load can be obtained through easy to perform linear elastic analysis. The DSM links the former to the latter via empirical formulas so that the ultimate load can be obtained via simple elastic analysis. In addition, the DSM design approach is advantageous over full computational analysis as it helps to better understand the nature of the buckling behaviour and thus provides a pathway to design.

Currently in the DSM the critical load is the only input parameter characterising the elastic buckling behavior, which contributes to the simplicity of the approach while on the other hand some notable deficiencies are incurred. First, the design approach is cumbersome to implement since different empirical formulas are proposed to give predictions of the ultimate load of members undergoing pure local, distortional and global buckling. An extra formula is proposed to deal with local-global interactive buckling, where the yield stress of the material in the formula for pure local buckling is replaced by the ultimate load associated with pure global buckling. Although research has been done on other types of interactive buckling, such as local-distortional interactive buckling [74] [80] [81], the proposed formulas have not been included in the design approach. In addition to true interactive buckling due to the closeness of critical stresses of different buckling modes, although it has been found that the ultimate load can also be eroded by interactive buckling due to secondary bifurcation [51], it is not considered in the design approach. Second, an overly conservative prediction of the ultimate load is obtained for sections where there is significant disparity between the slenderness values of different parts [71] [73]. This is attributed to the fact that the prediction of the ultimate load is based on the critical load. When one part of the section is extremely slender, it will dramatically lower the critical load of the cross-section to the critical load of this individual part, thus incorrectly lowering the predicted ultimate load ignoring that the rest of the section still has a large residual load carrying capacity.

In the proposed improvement of the DSM, the initial post-buckling stiffness is added as a parameter to the design formula in addition to the critical load. Despite the increased complexity, this will help to overcome the above deficiencies. With regard to the first deficiency, as different types of buckling are associated with different post-buckling stiffness, it is possible to unify the design formulas when the initial post-buckling stiffness is added to the design parameters. With regard to the second deficiency, when one part of the section buckles far prior to the other parts, this causes a relatively minor reduction of the stiffness, compared to the case where all parts of the section buckle simultaneously. Thus, the design formula will be able to give more accurate predictions of the load-carrying capacity. In addition, when interaction occurs between buckling modes, its negative effect on the ultimate load is reflected in a further reduction of the post-buckling stiffness compared to pure buckling. This reduction can be either a reduction to the stiffness in the initial post-buckling stage due to true interactive buckling, or in the far post-buckling stage due to secondary buckling. Using the initial post-buckling stiffness of both the primary and the secondary buckling mode, it is possible to establish an approximating model being able to cover both the two cases of reduction to the stiffness. This is achieved by figuring out the average post-buckling stiffness, which is an average of the post-buckling stiffness from the critical point to the ultimate point, before adding it to the design formula. In this way, when the initial post-buckling stiffness is added to the parameters, the same design formula used for pure buckling can also be applied to coupled instabilities. In conclusion, it is anticipated that the introduction of the initial post-buckling stiffness will lead to an improved design approach.

The implementation of the improved design approach will take three steps. First, the critical stresses and the initial post-buckling stiffness are calculated. These parameters characterising the elastic buckling behavior are then used in an approximation of the elastic post-buckling behaviour. Finally, the ultimate load is linked to the elastic post-buckling behaviour using empirical formulas.

The investigation of the proposed design approach will focus on pure local and distortional buckling, and local-distortional interactive buckling, although it is also applicable to pure global buckling. The proposed design approach is initially applied to the less complicated case of local-local interaction, which paves the way for its application to the more complicated local-distortional interaction. Due to the unique property of global buckling that no (or negligible) post-buckling residual capacity is present, interactions involving global buckling are not considered at the current stage.

This chapter deals with the calculation of the initial post-buckling stiffness of columns undergoing local and distortional buckling, which lays the foundation for the discussions in the following two chapters. The columns are perfect (i.e. without geometric imperfection), and are made of a purely elastic material. After assessing the balance between the complexity and accuracy of various methods described in the literature, the method proposed by Graves Smith and Sridharan [85] [86] [87] was selected for the purpose of this study. In addition, it is the only known method which is able to deal with distortional buckling.

(a)                                                                          (b)

Fig. 8.1 Coordinate systems and displacements

## Local buckling

In a first stage, the initial post-buckling stiffness is calculated when only local buckling is involved. This is the simplest case since only the out-of-plane displacements w appear in the first-order components of the deformed shape after buckling. For the convenience of the reader, the coordinate systems and displacements are shown again in Fig. 8.1.

### Governing equations

In a plate of thickness t, the stress resultants (per unit width of plate) are as follows:

where Nx, Ny and Nxy are the membrane forces; Qx and Qy are the shear forces; Mx, My and Mxy are the bending moments and twisting torque; and t is the plate thickness.

For an initially flat plate without lateral load, the equilibrium equations after deformation are:

The first two equations are satisfied if the membrane stresses obey Airy’s stress function defined as:

Eliminating Qx and Qy, the last three equations in Eq.  are condensed into one equation. After substituting Airy’s stress function into this equation, it becomes:

The stress-strain relations take the form:

while bending moments and twisting torque are linked to curvatures and twist:

In the above equations, the flexural rigidity of plate D = Et3/12(1-v2).

The strains can be expressed in terms of displacements using the relations:

along with curvatures and twist:

Eliminating displacements u and v in Eq.  while leaving w as the only independent displacement, the three equations are condensed into one equation.  After incorporating Eq.  and Eq.  into this equation, the compatibility equation is obtained:

Using Eq.  and Eq. , Eq.  is transformed into:

Eq.  and Eq.  are called von Kármán’s equations, where w and Φ are the unknowns. They constitute the governing equations for local buckling.

### Displacement functions

It is assumed that for local buckling, the following boundary conditions are present. At the junctions between the plate segments, the out-of-plane displacement w = 0, and the membrane transverse stress σy = 0 in both plate segments. At the column ends, the out-of-plane displacement w = 0, the out-of-plane curvature ∂2 w/∂ x2 = 0, and the warping displacement is constant along the edges: u = ± Lλ/2, where λ is a dimensionless end compression factor. The last condition is only valid for the end boundary condition further referred to as ‘uniform compression’, and is altered for other end boundary conditions. Also at the column ends, the membrane in-plane shear stress τxy = 0.

Since we are only interested in the initial post-buckling behaviour close to the critical point, the perturbation technique is applied to find approximate solutions to von Kármán’ s equations, where the out-of-plane displacements and Airy’s stress function are expressed in a formal power series in terms of a small parameter known as the perturbation parameter:

In the above, s is the perturbation parameter and the factors w(i) and Φ(i) are functions of y. The term Φ(0) = − Eλy2/2 represents the contribution of the unbuckled configuration to Φ. It is assumed that w(i)  is independent from Φ(i). Meanwhile, the functions w(i)  are orthogonal to each other, as are the functions Φ(i) to ensure the uniqueness of the expressions in Eq. .

Substituting Eq.  into Eq.  and Eq.  , the following equations result:

where

Since s is arbitrary, we obtain for the first-order factors:

and for the second-order factors:

With similar equations for the higher order terms of s are included in Eq. .

Eq.  expresses the eigenvalue problem associated with the buckling analysis while due to the independence of Φ(1) from w(1) Eq.  always gives the trivial solution

Due to the need to satisfying the boundary conditions specified at the beginning of section 8.2.2, the solution to Eq.  takes the form:

where n0 is the number of half-waves of the buckled shape. Substituting Eq.  into Eq.  noting that w(2) is orthogonal to w(1) and independent of Φ(2), results in:

Substituting Eq.  into Eq. , and taking the boundary conditions into account, the solution to  takes the form:

Continuing the calculation with the higher order terms leads to:

Substituting Eq. , Eq. , Eq.  and Eq.  into Eq.  yields:

Using the stress-strain relations in Eq.  and the strain-displacement relations in Eq. , along with the boundary conditions, u and v can be obtained in the form

The displacements u and v can be expressed in a formal power series similar to w:

where u(0)  and v(0) are displacement terms associated with the unbuckled configuration.

It results from Eq.  that

For our purpose of calculating the initial post-buckling stiffness, it is sufficient to consider only the first-order term w(1) for w, and up to the second-order terms u(2) and v(2) for u and v. Therefore the following displacement functions are produced:

where the terms associated with the unbuckled configuration are:

The first-order terms are:

The second-order terms are:

It is important to note that since the displacements u and v are associated with different longitudinal trigonometric functions, compatibility is not satisfied at the junctions where two plates meet each other at an angle. Thus it is neither necessary nor possible to establish a system of nodal global displacements. In light of this, an approximation is made that, while the out-of-plane displacements vanish at the corner, the transverse displacements can develop independently in each of the two plates, allowing membrane transverse stress to be released at the junction.

In addition, for each plate, the overall transverse translation is eliminated by setting v0(2) equal to zero at one of the two transverse edges of the plate:

where b is the plate width.

Table 8.1 Constraints on DOFs of a lipped channel section

| Section | Location | v0 | vm | w | θ |
| --- | --- | --- | --- | --- | --- |
|  | 1 | × | √ | √ | √ |
|  | 2 | √ | √ | × | √ |
|  | 3 | × | √ | × | √ |
|  | 4 | √ | √ | × | √ |
|  | 5 | × | √ | × | √ |
|  | 6 | √ | √ | × | √ |
|  | 7 | × | √ | × | √ |
|  | 8 | √ | √ | × | √ |
|  | 9 | × | √ | × | √ |
|  | 10 | √ | √ | √ | √ |

Table 8.2 Constraints on DOFs of an I-section

| Section | Location | v0 | vm | w | θ |
| --- | --- | --- | --- | --- | --- |
|  | 1 | √ | √ | √ | √ |
|  | 2 | × | √ | × | √ |
|  | 3 | √ | √ | √ | √ |
|  | 4 | √ | √ | × | √ |
|  | 5 | × | × | √ | × |

The constraints applied to the cross-sectional DOFs are illustrated by the following examples of a lipped channel section (Table 8.1) and an I-section (Table 8.2). In these tables v0 is the component of v which is longitudinally constant, vm is the component of v which varies longitudinally obeying trigonometric functions.

### Potential energy

The total potential energy (which is here equal to the internal strain energy) associated with each strip Ustrip is expressed as

where Astrip is the surface area of the strip. Substituting Eq. , Eq. , Eq.  and Eq.  into  yields the total potential energy of the strip expressed in terms of the local displacements:

where q and r stand for the local displacement vectors corresponding to first- and second-order displacements respectively. The i, j, k, l and n, p range over the total number of DOFs associated with first- and second-order displacements respectively.

On the basis of Eq.  it is possible to generate an expression of the total potential energy of the structural member in terms of the global displacements:

where Q and R stand for the first- and second-order global displacement vectors containing the local displacement values of all strips. It should be noted that the displacements in Q and R associated with different strips are independent from each other while no transformation is performed from local to global coordinate system due to the aforementioned reasons.

### Equilibrium equation

In a state of equilibrium, Eq.  is stationary with respect to Q. When only the lowest order terms are included in Eq. , this results in the usual eigenvalue problem associated with buckling analysis:

Once Q is obtained from Eq.  as the eigenvector, it is expressed in terms of the perturbation parameter:

Eq.  also has to be stationary with respect to R, which yields:

Substituting Eq.  into Eq.  results in the solution of R in terms of the perturbation parameter:

Substituting Eq.  and Eq.  into Eq.  yields the following expression of the total potential energy:

In order for expression  to be stationary with respect to s at equilibrium, we require:

Thus, the solution of λ can be found from  and takes the form:

where

### Compressive force

The compressive force associated with the column is expressed as

where A is the surface area of the column. The longitudinal stress σx is calculated using Eq.  and Eq. . In Eq.  only terms up to the second order in s are included: ∂u(0)/∂x, ∂v(0)/∂y, ∂u(2)/∂x, ∂v(2)/∂y, (∂w(1)/∂x)2/2 and (∂w(1)/y)2/2. The compressive force P can be written as:

where Pc is the critical compressive force when the column buckles.

While λ = λc + λ(2)s(2), the post-buckling stiffness Kp can be expressed in a form independent of the perturbation parameter s:

## Distortional buckling

Compared to local buckling, distortional buckling is more complicated, because, apart from the out-of-plane displacement w, the displacements u and v are also involved in the buckling behaviour. In light of this, the calculation method of the initial post-buckling stiffness developed for local buckling needs to be altered.

### Governing equations

First, the first two equilibrium equations in Eq.  are altered to take into account the second-order effects of the longitudinal stress resultant Nx in producing a deviation force in the y -direction:

Consequently, Airy’s stress function, as derived from the original two equations, is abandoned.

Second, an extra term (the underlined term in Eq. ) is added to the strain-displacement relations in Eq.  to take into account the non-linear effects of the transverse displacement v:

Consequently, Eq.  is no longer valid, and is replaced by Eq. , which now constitutes, along with Eq. , the governing equations for distortional buckling:

### Displacement functions

The boundary conditions used for local buckling are no longer valid for distortional buckling, and more approximations have to be made.

Expanding all the variables involved in Eq.  using the perturbation technique results in:

where Nx(0) = − Etλ is the contribution of the unbuckled configuration to the longitudinal stress resultant.

Substituting Eq.  into Eq. , Eq.  and Eq.  yields the following equations:

where the A, B, and C terms are derived from Eq. , Eq.  and Eq.  respectively. Since s is arbitrary, the first-order terms are obtained from:

with the second-order terms being determined by:

With a similar procedure for the higher order terms when they are include in Eq. .

Eq. , Eq.  and Eq.  constitute the eigenvalue problem for buckling analysis where w and v are uncoupled. The solution gives the first-order displacement terms:

Substituting Eq.  into Eq. , Eq.  and Eq.  yields the following solution of the second-order displacement terms:

It can be seen that if n0 is even, the second-order displacement terms are orthogonal to their corresponding first-order counterparts, which is necessary for the uniqueness of the displacement decompositions in Eq. . In the particular case where n0 = 2, the displacement functions shown in Eq.  and Eq.  satisfy the boundary conditions illustrated in Fig. 8.2, on the condition that the origin of the coordinate system is relocated to the mid-height of the column. Both ends of the column are assumed to be attached to a frictionless rigid end plate to ensure a uniform end shortening, while the cross-sectional displacements are free to develop.

Fig. 8.2 Boundary conditions for distortional buckling

In contrast to local buckling, for distortional buckling local displacements u, v, w and θ can be transformed to global ones U, V, W and Θ, since the longitudinal trigonometric functions attached to displacements v and w are consistent.

The overall rotation Θ0(2) and the translations V0(2) and W0(2) of a single node of the column should be set equal to zero to eliminate any overall translation and rotation.

### Potential energy

The expression of the total potential energy in Eq.  is altered to

### Equilibrium equation

The derivation of equilibrium equation follows the same procedure as for local buckling, except that Eq.  is altered to

### Compressive force

The calculation of the compressive force follows the same procedure as for local buckling. However, regarding the terms in Eq.  up to the second order in s, in addition to ∂u(0)/∂x, ∂v(0)/∂y, ∂u(2)/∂x, ∂v(2)/∂y, (∂w(1)/∂x)2/2 and (∂w(1)/y)2/2, new terms emerge, namely the second-order term (∂v(1)/∂x)2/2, and the first-order terms ∂u(1)/∂x and ∂v(1)/∂y. However, when n0 is even, the integrations of both first-order terms along the column length vanish, and the expression of the compressive force P in Eq. , where the first-order term in s is absent, remains valid. Therefore, an exact value of initial post-buckling stiffness can still be calculated.

## End boundary conditions

In the previous discussion, the ‘uniform compression’ boundary condition shown in Fig. 8.3(a) was assumed, where the warping displacements at the column ends are constant along the cross-section. Another option can be envisaged, here termed the ‘load through centroid’ option and shown in Fig. 8.3(b). In both cases, both ends of the column are connected to rigid-body end plates. While in the ‘uniform compression’ case the overall rotation of the end plate is constrained, it is allowed in the ‘load through centroid’ case. This difference in end boundary condition has repercussions when shift of effective centroid is considered. After the column buckles, the longitudinal stress is no longer uniformly distributed across the cross-section, but concentrates at the plate junctions. This re-distribution of the longitudinal stress introduces an eccentricity between the centre of the stress resultant and the centroid of the cross-section (Fig. 8.4 (a)). In the ‘uniform compression’ case this induces a bending moment on the end plate, while contrarily in the ‘load through centroid’ case this bending moment is released as the end plate is allowed to rotate. Thus, the ‘uniform compression’ case is valid for columns with fixed ends while the ‘load through centroid’ case is valid for columns with simply-supported ends.

(a)  Uniform compression              (b) Load through centroid

Fig. 8.3 Boundary conditions

The calculation of the initial post-buckling stiffness is slightly more complicated for the ‘load through centroid’ end boundary condition. It takes the following steps: First, the solutions of Q and R are found for the ‘uniform compression’ end boundary condition. Then the corresponding end moment is calculated. Second, the end plate is rotated about the minor axis by applying additional linear end warping displacements to the original shape, which causes a change in the total potential energy expression in Eq. :

where λ* is the parameter related to the modification of the end warping displacements. Since λ* is a function of Q and R, it can also be expanded as:

Thus, the solution of λ is found from Eq. , and takes the form:

where

In Eq.  which the first-order term is negligible [87], and thus a closed-form solution of the initial post-buckling stiffness Eq.  can still be found.

(a) Distribution of σx

(b) Distribution of u

Fig. 8.4 Shift of centroid

## Programme development

A programme was developed in MATLAB to calculate the initial post-buckling stiffness of columns using the previously described method. The programme has four variations, which deal, respectively, with local buckling of a simple open section, distortional buckling of a simple open section, local buckling of a box section, and local buckling of an I-section. The variation for local buckling of a simple open section is taken as an illustrative example.

As seen in the main flow chart presented in Fig. 8.5, inputs from the user include the nodal coordinates, the plate thickness, the mechanical properties of the elastic material, and the cross-sectional meshing size. The programme then performs a cross-sectional discretisation of the section, resulting in the coordinates of the sub nodes. Thereafter the programme performs one of the three functions according to user selection. The first function displays the discretised cross-section for the user to verify the inputs. The second function performs a buckling analysis of the section, and returns the minimum local buckling critical stress and the corresponding half-wavelength. The third function performs the second function first, and then carries out initial post-buckling analysis of the section, which returns the initial post-buckling stiffness.

The flow chart of the m-file ‘IniPost.m’, which performs the initial post-buckling analysis, is presented in Fig. 8.6. First, all input values are collected. Second, the multi-dimensional arrays Aij, Bij, Enp, Anij and Aijkl in Eq.  are established and initialised as zero. Third, a loop is performed to go through each of the strips constituting the discretised cross-section, calculating the local multi-dimensional arrays aij, bij, enp, anij and aijkl before adding them to the global multi-dimensional arrays as part of the assembly process. Once the global multi-dimensional arrays are obtained, the elements in these arrays associated with the constrained DOFs are eliminated using the boundary conditions. Finally, the first-order deformed shape Q, the critical load λc, the second-order displacements R and the second-order load coefficient λ(2) are calculated using Eq.  to Eq. . The results are displayed to the user.

Fig. 8.5 Main flow chart of the programme

Fig. 8.6 Flow chart of the m-file IniPost.m

# Elastic post-buckling behaviour

## Introduction

This chapter focuses on obtaining a multi-linear approximation of the load-end shortening curve of a perfect column made of an elastic material. It relies on the methodology developed in the previous chapter, where the initial post-buckling stiffness is calculated via the perturbation method. However, as the load increases further in the post-buckling stage, the stiffness is further reduced, either gradually or in some cases abruptly. This reduction of stiffness is attributed to the involvement of buckling modes other than the critical one in the deformed shape in the post-buckling stage.

## Local-local interactive buckling

Local buckling of box sections and I-sections is first studied, as relatively straightforward cases. The study focuses particularly on the cases where there is significant disparity between the slenderness of different parts constituting the cross-section. The slender plates with critical buckling loads lower than the critical buckling load of the cross-section as a whole are put in plate group 1 (pg1), while other plates with higher critical buckling loads are put in plate group 2 (pg2). In such cases, the DSM gives unduly conservative results because it links the ultimate load to the critical buckling load. The plates of pg1 with low critical buckling loads will dramatically lower the critical buckling load of the cross-section as a whole and consequently yield conservative predictions of the ultimate load, ignoring that the rest of the section still has a large residual load carrying capacity. In addition, in such cases there are two relevant local modes, being the primary and the secondary modes, where only the plates of pg1 and pg2 buckle, respectively. The column buckles initially in the primary mode when the plates of pg1 buckle. Then as the load increases and the plates of pg2 also buckle, the column buckles in an interactive mode between the primary and the secondary modes.

### Approximating model

The multi-linear model for local buckling is schematically shown in Fig. 9.1, where the compressive load on the vertical axis is transformed to a longitudinal stress (i.e. load P divided by cross-sectional area A), and the end shortening on the horizontal axis is transformed to a longitudinal strain (i.e. end shortening ΔL divided by column length L). In Fig. 9.1, ε1, σ1, ε2 and σ2 are the longitudinal strains and stresses when primary and secondary buckling occurs, respectively. When the longitudinal strain is lower than ε1, the column is unbuckled, and thus the stiffness K equals the Young’s modulus of the material E. Subsequently, when the longitudinal strain increases to a value between ε1 and ε2, the column is in a state of pure primary local buckling, and the stiffness is reduced to the initial post-buckling stiffness K1. Finally, when the longitudinal strain increases beyond ε2, the column undergoes interactive buckling with the stiffness further reduced to Kmix.

Fig. 9.1 Multi-linear model with involvement of two modes

Four signature parameters are essential in characterising the multi-linear model, namely ε1, ε2, K1 and Kmix. In the proposed model, these parameters are determined using the results of buckling and initial post-buckling analyses. First, they are determined under the idealised conditions that the column is simply supported at both ends while warping displacements are allowed, and that its length is an integer multiple of the half-wavelengths of both the primary and the secondary mode.

- Determination of ε1 and ε2

The primary buckling strain ε1 and stress σ1 equal the minimum critical strain εl1 associated with the primary local buckling mode L1 and its corresponding stress σl1 = Eεl1. The minimum critical strain εl1 = σ l1/E can be determined from the signature curve shown in Fig. 9.2 and corresponds to the critical half-wavelength Ll1. On the other hand, the determination of the secondary buckling strain ε2 and stress σ2 is a complicated problem of secondary bifurcation, which does not lend itself to an exact solution within a practical engineering design approach. In light of this, the problem is simplified by assuming that ε2 is equal to the critical strain εl2 = σl2/E associated with the secondary local buckling mode L2, which corresponds to the critical half-wavelength Ll2, shown in Fig. 9.2. This assumption is consistent with the one used in the Kutanova/Mennink model [42] [53] intended to predict the ultimate loads of local-local and local-distortional interactive buckling. The stress σ2 is determined from Fig. 9.1 taking into account the reduction of stiffness after primary buckling occurs:

Fig. 9.2 Critical half-wavelengths of local modes

- Determination of K1 and Kmix

K1 is the initial post-buckling stiffness of the column, and is determined by the approach described in chapter 8. Kmix is the interactive buckling stiffness after interaction occurs between the primary and the secondary buckling mode. In order to determine Kmix, the initial post-buckling stiffness associated with the secondary buckling mode K2 is first calculated. It is obvious that Kmix is dependent on the proportion between the modal participations of the two modes. However, an exact solution of Kmix is unfeasible within a design approach. Alternatively, an approximating formula is proposed to estimate Kmix:

where the relative stiffness kmix = Kmix/E, k1 = K1/E, and k2 = K2/E. This formula results in the following features of the interactive buckling stiffness: First, the relative interactive buckling stiffness ranges between 0 and 1. Second, the interactive buckling stiffness is lower than both of the pure buckling stiffness values. Third, when one of the pure buckling stiffness values is zero, the interactive buckling stiffness is also zero. The second feature accounts for the additional reduction in stiffness when the secondary buckling mode enters participation. The third feature accounts for the fact that when a global buckling mode with zero (or negligible) stiffness emerges, the overall column stiffness becomes zero.

In the above the signature parameters were determined under idealised conditions, while in practice these conditions are not always satisfied. For example, the column may have fixed ends, or its length may not be a multiple of the critical half-wavelengths. However, these changes in boundary conditions and column length are expected to have little effect on the signature parameters for local buckling, due to its short half-wavelength compared to the column length. Usually, a considerable number of half-waves can be accommodated within the column length, and thus the effects of the boundary conditions and a slightly modified half-wavelength, are minimised. Therefore, the previously determined signature parameters are still deemed to be applicable in the design approach.

### Verification against numerical results

- Box sections

The multi-linear model is first verified for a series of box sections, of which the corresponding cross-sectional dimensions along with the signature parameters, are tabulated in Table 9.1. The notations tw, bw, tf and bf are the thickness and the width of the webs and the flanges, respectively.

Table 9.1 Buckling parameters

| bw, bf, tf (mm) | tw (mm) | Ll1 (mm) | Ll2 (mm) | σl1 (MPa) | σl2 (MPa) | kl1 | kl2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| bw = 100, bf = 100, tf = 1 | 1 | 100 | 66 | 75.9 | 132.3 | 0.41 | 0.48 |
|  | 1.2 | 97 | 69 | 91.4 | 163.5 | 0.48 | 0.54 |
|  | 1.4 | 89 | 74 | 105.7 | 201.2 | 0.64 | 0.57 |
|  | 1.6 | 79 | 79 | 115.6 | 243.9 | 0.75 | 0.57 |
|  | 1.8 | 74 | 83 | 121.1 | 291.9 | 0.80 | 0.57 |
|  | 2 | 71 | 87 | 124.3 | 345.6 | 0.82 | 0.57 |
| bw, bf, tf (mm) | tw (mm) | Ll1 (mm) | Ll2 (mm) | σl1 (MPa) | σl2 (MPa) | kl1 | kl2 |
| bw = 100, bf = 110, tf = 1 | 1 | 105 | 69 | 68.0 | 122.1 | 0.45 | 0.54 |
|  | 1.2 | 100 | 73 | 79.5 | 155.3 | 0.57 | 0.61 |
|  | 1.4 | 92 | 78 | 89.3 | 193.6 | 0.68 | 0.61 |
|  | 1.6 | 85 | 82 | 96.0 | 236.5 | 0.75 | 0.61 |
|  | 1.8 | 80 | 86 | 100.1 | 284.8 | 0.79 | 0.60 |
|  | 2 | 78 | 89 | 102.7 | 338.9 | 0.81 | 0.59 |
| bw = 100, bf = 120, tf = 1 | 1 | 111 | 71 | 60.2 | 116.1 | 0.50 | 0.63 |
|  | 1.2 | 105 | 76 | 68.8 | 150.1 | 0.61 | 0.65 |
|  | 1.4 | 97 | 81 | 76.0 | 188.4 | 0.70 | 0.64 |
|  | 1.6 | 91 | 85 | 80.9 | 231.3 | 0.75 | 0.63 |
|  | 1.8 | 87 | 88 | 84.2 | 279.7 | 0.78 | 0.62 |
|  | 2 | 85 | 90 | 86.3 | 333.8 | 0.80 | 0.61 |
| bw = 100, bf = 130, tf = 1 | 1 | 117 | 73 | 53.1 | 112.3 | 0.55 | 0.68 |
|  | 1.2 | 110 | 78 | 59.8 | 146.4 | 0.63 | 0.68 |
|  | 1.4 | 103 | 82 | 65.2 | 184.5 | 0.70 | 0.66 |
|  | 1.6 | 98 | 86 | 69.2 | 227.3 | 0.74 | 0.64 |
|  | 1.8 | 94 | 89 | 71.8 | 275.4 | 0.77 | 0.63 |
|  | 2 | 92 | 91 | 73.5 | 329.4 | 0.79 | 0.62 |
| bw = 100, bf = 140, tf = 1 | 1 | 123 | 75 | 46.9 | 109.7 | 0.57 | 0.71 |
|  | 1.2 | 116 | 80 | 52.2 | 143.6 | 0.64 | 0.70 |
|  | 1.4 | 110 | 84 | 56.6 | 181.3 | 0.70 | 0.67 |
|  | 1.6 | 104 | 87 | 59.8 | 223.7 | 0.73 | 0.65 |
|  | 1.8 | 101 | 90 | 61.9 | 271.5 | 0.76 | 0.63 |
|  | 2 | 99 | 92 | 63.4 | 324.9 | 0.78 | 0.62 |
| bw = 100, bf = 150, tf = 1 | 1 | 130 | 76 | 41.5 | 76.2 | 0.58 | 0.73 |
|  | 1.2 | 123 | 81 | 45.9 | 141.2 | 0.65 | 0.71 |
|  | 1.4 | 116 | 85 | 49.5 | 178.4 | 0.69 | 0.68 |
|  | 1.6 | 111 | 88 | 52.2 | 220.3 | 0.73 | 0.66 |
|  | 1.8 | 108 | 91 | 54.0 | 267.4 | 0.75 | 0.63 |
|  | 2 | 105 | 92 | 55.2 | 319.9 | 0.77 | 0.61 |

The modal shapes of the primary and secondary local modes (L1 and L2) are presented in Table 9.2 for sections with bw = 100, bf  = 100, tf  = 1 and with tw varying from 1 to 2 (all in mm), while the modal shapes of the remaining sections are tabulated in Table 1 in appendix D. It is seen that when the flange and the web have similar slenderness, they tend to bend with similar amplitude in the primary local buckling mode. Thus, the end rotation of one plate is little restrained by the adjacent plates, which leads to near independent buckling of each plate under simple-simple boundary condition along the longitudinal edges. Meanwhile, the secondary local buckling mode is characterised by independent buckling of each plate under boundary conditions which can be considered nearly fixed along the longitudinal edges. On the other hand, when the webs become increasingly stocky compared to the flanges, the end rotations of the flanges are restrained by the web. In the corresponding secondary local mode, the webs develop the largest out-of-plane displacements.

Table 9.2 Buckling modal shapes

| bw, bf, tf (mm) | tw (mm) | L1 | L2 |
| --- | --- | --- | --- |
| bw = 100, bf = 100, tf = 1 | 1 |  |  |
|  | 1.2 |  |  |
|  | 1.4 |  |  |
|  | 1.6 |  |  |
|  | 1.8 |  |  |
|  | 2 |  |  |

Table 9.3 Verification of the multi-linear model

| bw = 100 mm, bf = 100 mm, tf = 1 mm |  |
| --- | --- |
| tw = 1 mm, L = 300 mm | tw = 1.2 mm, L = 292 mm |
| tw = 1.4 mm, L = 267 mm | tw = 1.6 mm, L = 237 mm |
| tw = 1.8 mm, L = 221 mm | tw = 2 mm, L = 213 mm |
| Multi-linear model ABAQUS analysis |  |

In Table 9.3, the multi-linear model (plotted in red) is verified against the results from an elastic geometrically non-linear ABAQUS analysis (plotted in blue) for sections with bw = 100 mm, bf = 100 mm, tf  = 1 mm and with tw varying from 1 to 2 mm. The length of the column L was taken as three times the half-wavelength of the primary local buckling mode. The horizontal and vertical axes represent the longitudinal strain and stress (in MPa), respectively. The results of the remaining sections are tabulated in Table 1 in appendix E. Since a post-buckling analysis of a geometrically perfect column in ABAQUS typically does not yield satisfactory results, with the analysis continuing on the (unstable) fundamental equilibrium path above the buckling load, a small imperfection was applied with the magnitude being a coefficient of the flange thickness, i.e. 0.1tf as suggested by Zeinoddini [111]. In a few cases a slightly larger imperfection had to be applied to avoid this difficulty.

Table 9.4 Buckling parameters

| bw, bf, tw (mm) | tf (mm) | Ll1 (mm) | Ll2 (mm) | σl1 (MPa) | σl2 (MPa) | kl1 | kl2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| bw = 150, bf = 100, tw = 1 | 1 | 174 | 95 | 36.3 | 74.6 | 0.51 | 0.55 |
|  | 1.2 | 165 | 106 | 44.1 | 92.2 | 0.59 | 0.58 |
|  | 1.4 | 127 | 117 | 50.9 | 112.2 | 0.75 | 0.59 |
|  | 1.6 | 112 | 129 | 54.1 | 134.4 | 0.82 | 0.60 |
|  | 1.8 | 106 | 141 | 55.7 | 158.8 | 0.84 | 0.60 |
|  | 2 | 104 | 153 | 56.6 | 185.6 | 0.85 | 0.60 |
|  | 2.2 | 102 | 164 | 57.2 | 214.9 | 0.87 | 0.60 |
|  | 2.4 | 102 | 176 | 57.6 | 246.6 | 0.88 | 0.61 |

- I-sections

Second, the multi-linear model is verified for I-sections, of which the cross-sectional dimensions, along with the corresponding signature parameters, are tabulated in Table 9.4.

The modal shapes of the primary and secondary local modes (L1 and L2) are presented in Table 9.5. Similarly to the box sections, when the flange and web are approximately equally slender, the rotations of the plates at the junctions are not mutually restrained, which leads to independent plate buckling of the web and the flange under the simple-simple and simple-free boundary conditions, respectively. The secondary local buckling mode is in this case characterised by independent plate buckling of the web and the flange under nearly fixed-fixed and fixed-free boundary conditions respectively. On the other hand, when the flange becomes increasingly stocky compared to the web, the rotations of the junctions are increasingly restrained, which leads to buckling of the web and the flange under boundary conditions resembling fixed-fixed and fixed-free conditions, respectively. The out-of-plane displacement at the mid span of the web is then significantly larger than that at the free end of the flange. The opposite occurs in the secondary local mode, where the flange buckles under near simple-free boundary conditions, developing significantly greater out-of-plane displacements at the free end than that at the mid span of the web.

In Table 9.6, the multi-linear model is verified against the results from an elastic geometrically non-linear ABAQUS analysis.

Fig. 9.3 Verification of the multi-linear model

It can be seen for both box-sections and I-sections that in general a good agreement is observed between the multi-linear model and ABAQUS analysis, except in some cases where different parts of section are approximately equally slender. In these cases, the contribution of the secondary mode in the interactive buckling is overestimated by the multi-linear model, thus yielding an underestimation of the post-buckling stiffness. For example, regarding the box section with bw = 100, bf  = 100, tw = 1, tf  = 1 (all in mm), the longitudinal stress-strain curve of elastic buckling is plotted in Fig. 9.3. The blue curve represents the results from an elastic geometrically non-linear ABAQUS analysis, while the solid red curve represents the multi-linear model with an initial post-buckling stiffness of k1, which is reduced to kmix = k1k2 after secondary bifurcation. It can be seen that the post-buckling stiffness is underestimated. On the other hand, the dashed red curve represents a different multi-linear model with a constant post-buckling stiffness of k1, which correlates better with the results of ABAQUS analysis. Therefore, it is decided that in these cases the second model is used, and the column is considered to buckle solely in the primary mode. It is also noted that for such sections, the initial post-buckling stiffness associated with the primary mode is lower than that associated with the secondary mode: k1 < k2, which serves as the criterion to judge whether the second model should be used instead of the original model.

Table 9.5 Buckling modal shapes

| bw, bf, tw (mm) | tf (mm) | L1 | L2 |
| --- | --- | --- | --- |
| bw = 150, bf = 100, tw = 1 | 1.0 |  |  |
|  | 1.2 |  |  |
|  | 1.4 |  |  |
|  | 1.6 |  |  |
|  | 1.8 |  |  |
|  | 2.0 |  |  |
|  | 2.2 |  |  |
|  | 2.4 |  |  |

Table 9.6 Verification of the multi-linear model

| tf = 1 mm, L = 521 mm | tf = 1.2 mm, L = 494 mm |
| --- | --- |
| tf = 1.4 mm, L = 382 mm | tf = 1.6 mm, L = 335 mm |
| tf = 1.8 mm, L = 319 mm | tf = 2 mm, L = 311 mm |
| tf = 2.2 mm, L = 307 mm | tf = 2.4 mm, L = 305 mm (imp = tw) |
| Multi-linear model ABAQUS analysis |  |

## Local-distortional interactive buckling

### Introduction

Among the coupled instabilities which occur in CFS columns, considerable research work has been devoted to local-distortional interactive buckling, particularly in the last decades. Particularly promising outcomes of the work are the complements to the DSM design approach to account for local-distortional interaction. Various methodologies have been proposed, including the nominal strength against local-distortional (NLD) and distortional-local (NDL) interactions approach, and the modified NDL (MNDL) approach (see section 2.12). Nevertheless, all of these approaches have limitations. First, since design formulas for interactive buckling are different from the ones for pure buckling, it is difficult to make an objective judgement in a particular case whether the interaction should be considered and the corresponding formulas are used. Second, they work only for ‘true interaction’, stemming from a comparable magnitude between the local and distortional critical stresses. However, even if these critical stresses are not close, interaction can still happen in the post-buckling stage due to secondary bifurcation, which consequently significantly erodes the ultimate load [52], but is not currently considered by the DSM design approaches.

### Approximating model

Fig. 9.4 Multi-linear model with involvement of three modes

Fig. 9.5 Critical half-wavelengths of local and distortional modes

In this section, the multi-linear model proposed in the previous section is further developed to apply to local-distortional interactive buckling, and a new way to determine the four signature parameters ε1, ε2, K1 and Kmix is proposed. Under the same idealised conditions as assumed for local buckling, the strains ε1 and ε2 are the minimum critical strains of the primary and secondary modes, respectively, being either the primary local mode L1 or the distortional mode D, which correspond to the critical half-wavelengths Ll1 and Ld, as shown in Fig. 9.5. K1 is the initial post-buckling stiffness associated with the critical mode. Kmix is the interactive buckling stiffness when interaction occurs between the local and the distortional mode. The relative interactive buckling stiffness kmix is again calculated as:

where k1 and k2 are the relative stiffness values associated with the primary and the secondary mode, respectively. In some cases where the slenderness of the column  is high, in addition to the primary local mode and the distortional mode, the secondary local mode L2 also needs to be considered. Thus the more complicated multi-linear model shown in Fig. 9.4 is applied, where the secondary local mode becomes involved when the longitudinal strain exceeds ε3. The strain ε3 is determined as the critical strain associated with the secondary local bucking mode εl2, and the corresponding stress σ3 is calculated as:

After participation of the secondary local mode, the relative stiffness of the column is further reduced to:

where k3 = kl2 is the relative stiffness associated with the secondary local mode.

In the above the signature parameters are determined under the idealised conditions. However, in reality these conditions are not always satisfied. For example, in practical experiments the ends of the column are usually attached to a rigid end plate so that any non-uniform distribution of the warping displacement is prevented. While these changes in boundary conditions and column length have little effect on the signature parameters for local buckling, they do make a difference when the distortional buckling mode is involved due to its relatively longer half-wavelength.

In order to study the significance of the changes in the assumed idealised conditions, a lipped channel section is studied with the cross-sectional geometry defined in Table 9.7. The definitions of the notations are shown in Fig. 9.6, where r is the inner radius. The signature parameters are tabulated in Table 9.8, and the modal shapes are shown in Fig. 9.7.

(All dimensions are measured from the outer edge of the section)

Fig. 9.6 Simple lipped channel (CH1)

The verification of the multi-linear model is carried out in two steps. First, the model is verified under the idealised conditions. Second, it is verified with fixed-ended boundary conditions (consistent with most of the experiments when distortional buckling is studied) and with varying column length, for the purpose of investigating the effect on the signature parameters when the idealised conditions are violated.

Table 9.7 Section geometry

| section | t (mm) | bw (mm) | bf (mm) | d (mm) | r (mm) |
| --- | --- | --- | --- | --- | --- |
| CH1-5-800 | 1.085 | 119.55 | 89.65 | 4.80 | 1.30 |

Table 9.8 Buckling parameters (independent of column length)

| section | Ll1 (mm) | Ll2 (mm) | Ld (mm) | σl1 (MPa) | σl2 (MPa) | σd (MPa) | kl1 | kl2 | kd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH1-5-800 | 101 | 72 | 332 | 81.7 | 149.7 | 37.6 | 0.70 | 0.60 | 0.67 |

| section | D | L1 | L2 |
| --- | --- | --- | --- |
| CH1-5 |  |  |  |

Fig. 9.7 Cross-sectional modal shapes of buckling modes selected

In the first step, the column length takes the value of  in the ABAQUS model. Edge 3 of the column in Fig. 9.9 is in frictionless contact with a rigid end plate, so that cross-sectional displacements in the plane of the cross-section are free to develop while only uniform warping displacements are allowed. Edge 1 is constrained with respect to its warping displacement. Both edge 1 and edge 3 are constrained regarding rotations about the Y - and Z -axis. Node 2 at midspan is constrained regarding Y - and Z -axis translation and rotation about the X -axis to avoid rigid-body movement of the column. The loading is displacement-controlled through imposing an X -axis translation of the end plate. This model is advantageous over the fixed-ended or simply supported model in reflecting the nature of distortional buckling because of the unconstrained development of the warping displacements. This brings two benefits: First, in the fixed-ended or simply supported model, in order to minimise the effects of constrained end warping displacements, it is required to extend the column length to multiple times the half-wavelength. In contrast, in the proposed model the column is much shorter with a length equal to the half-wavelength, thus increasing the efficiency of the numerical calculations. Second, in the fixed-ended or simply supported model the distortional buckling load-end shortening curve is asymmetric, which means the curve is different whether the flanges of the deformed cross-

Fig. 9.9 ABAQUS model of distortional buckling

section move inwards or outwards. This adds to the complexity of the analysis. In contrast, in the proposed model the load-end shortening curve is symmetric. The result is shown in Fig. 9.10, where the red curve represents the multi-linear model based on the parameters tabulated in Table 9.8, while the blue curve represents the result from an elastic geometrically non-linear ABAQUS analysis using the aforementioned model. It is seen that the agreement between the two is satisfactory.

In the second step, where the idealised conditions are not satisfied, the signature parameters are modified. Taking into account the significant computational effort involved in calculating the stiffness under non-idealised conditions, the simplifying assumption is made that only the critical stresses are altered while the stiffness remains unchanged. The validity of this assumption is verified using the above example. The critical stresses of fixed-ended columns with various lengths L are calculated using the CUFSM programme and are tabulated in Table 9.9, which lists σl1 and σl2 determined when only local buckling is allowed, σd when only distortional buckling is allowed and σ when both local and distortional buckling are allowed. It can be seen that, while σl1 and σl2 do not change significantly with L, σd is quite dependent on L.

Fig. 9.10 Verification of the multi-linear model

The multi-linear approximations, using the above data, are tabulated in Table 9.10, where L is the length of the column. It can be seen that although the same stiffness parameters are used in the multi-linear model, as obtained under the idealised conditions, satisfactory agreement is achieved between the multi-linear model and the results from an ABAQUS analysis. Thus, the stiffness values in Table 9.8 are still applicable.

Table 9.9 Critical stresses subject to column length (fixed ends)

| L (mm) | 400 | 600 | 800 | 1000 |
| --- | --- | --- | --- | --- |
| σl1 (MPa) | 89.4 | 86.4 | 85.4 | 84.9 |
| σl2 (MPa) | 159.1 | 156.3 | 155.3 | 155.1 |
| σd (MPa) | 60.4 | 52.3 | 44.9 | 43.1 |
| σ (MPa) | 56.1 | 49.0 | 43.2 | 41.5 |

Table 9.10 Verification of the multi-linear model

| L = 400 mm | L = 600 mm |
| --- | --- |
| L = 800 mm | L = 1000 mm (imp = 0.5t) |
| Multi-linear model ABAQUS analysis |  |

### Verification against numerical results

The multi-linear model approximating the load-end shortening curve of a perfect column made of an elastic material is verified against an elastic geometrically non-linear ABAQUS analysis for various sections, obtained from [43] and [25].

In [43], lipped channel sections with and without web stiffeners are studied experimentally. Their cross-sectional geometry is listed in Table 9.11 and Table 9.12. The notations needed to define section CH1 were previously shown in Fig. 9.6, while those of section CH2 are shown in Fig. 9.11. In [25], a range of sections are studied numerically including hat sections, Z-sections and rack sections. Their cross-sectional geometry is shown in Table 9.13 and Table 9.14, along with the corresponding signature parameters.

It is important to note that only part of the data included in [43] and [25] were selected for this study. One of the criteria of selection was that the slenderness of the column should be high so that it has a considerable residual capacity after buckling. Another criterion was that the global critical stress should be significantly higher than the local and distortional critical stresses as well as the yield stress of the material, so that global buckling and elastic-plastic interactive buckling are absent.

The signature parameters are tabulated in Table 9.16. For the stiffened lipped channel sections CH2, the secondary local modes are not considered due to their very high critical stresses. The modal shapes of the modes involved are presented in Table 9.17 for a limited number of the sections, while the results of the remaining sections are presented in Table 2 in appendix D.

(All dimensions are measured from the outer edge of the section)

Fig. 9.11 Stiffened lipped channel (CH2)

The results for the selected sections are presented in Table 9.18, while the results of the remaining sections are shown in Table 2 of appendix E. It can be seen that a good agreement is observed between the multi-linear model and ABAQUS analysis.

Table 9.11 CH1 section geometry

| section | L (mm) | t (mm) | bw (mm) | bf (mm) | d (mm) | r (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| CH1-5-800 | 800 | 1.085 | 119.55 | 89.65 | 4.80 | 1.30 |
| CH1-6-800 | 800 | 1.085 | 119.80 | 89.75 | 6.00 | 1.30 |
| CH1-7-400 | 400 | 1.095 | 120.80 | 89.70 | 7.00 | 1.30 |
| CH1-7-600 | 600 | 1.095 | 120.40 | 89.55 | 7.00 | 1.30 |
| CH1-7-800 | 800 | 1.100 | 120.50 | 89.50 | 7.00 | 1.30 |

Table 9.12 CH2 section geometry

| section | L (mm) | t (mm) | bw (mm) | bf (mm) | d (mm) | r (mm) | s1 (mm) | s2 (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH2-7-800 | 800 | 1.100 | 120.50 | 90.15 | 7.00 | 1.30 | 10.00 | 20.00 |
| CH2-7-1000 | 1000 | 1.100 | 120.50 | 89.80 | 7.20 | 1.30 | 10.30 | 19.50 |
| CH2-8-1000 | 1000 | 1.095 | 119.95 | 89.95 | 8.00 | 1.30 | 10.15 | 20.00 |
| CH2-10-1000 | 1000 | 1.105 | 116.00 | 89.95 | 10.00 | 1.30 | 10.30 | 20.00 |
| CH2-12-1000 | 1000 | 1.100 | 120.50 | 89.95 | 12.00 | 1.30 | 10.00 | 19.50 |
| CH2-14-1000 | 1000 | 1.105 | 116.60 | 89.85 | 14.00 | 1.30 | 10.30 | 20.00 |

(All dimensions are measured from the mid-line of the section)

Fig. 9.11 Section geometry notations

Table 9.13 Hat section geometry

| section | L (mm) | t (mm) | bw (mm) | bf (mm) | bs (mm) |
| --- | --- | --- | --- | --- | --- |
| H5-1 | 1030 | 1.9 | 180 | 110 | 10 |
| H5-2 | 1030 | 1.9 | 180 | 130 | 10 |
| H5-3 | 1030 | 1.9 | 180 | 90 | 10 |
| H12-1 | 1850 | 1.5 | 200 | 190 | 12.5 |
| H12-2 | 1850 | 1.5 | 200 | 190 | 14 |
| H12-3 | 1850 | 1.5 | 200 | 190 | 11 |
| H11-1 | 1430 | 1.2 | 150 | 140 | 10 |
| H11-2 | 1430 | 1.2 | 150 | 130 | 10 |
| H11-3 | 1430 | 1.2 | 150 | 150 | 10 |
| H13-1 | 1950 | 1.9 | 235 | 160 | 12.5 |
| H13-2 | 1950 | 1.9 | 235 | 160 | 14 |
| H13-3 | 1950 | 1.9 | 235 | 160 | 11 |

Table 9.14 Z-section geometry

| section | L (mm) | t (mm) | bw (mm) | bf (mm) | bs (mm) |
| --- | --- | --- | --- | --- | --- |
| Z2-1 | 1030 | 1.9 | 180 | 110 | 10 |
| Z2-2 | 1030 | 1.9 | 180 | 130 | 10 |
| Z2-3 | 1030 | 1.9 | 180 | 90 | 10 |
| Z8-1 | 1400 | 1.0 | 100 | 50 | 5 |
| Z8-2 | 1400 | 1.0 | 100 | 45 | 5 |
| Z8-3 | 1400 | 1.0 | 100 | 55 | 5 |
| Z12-1 | 1850 | 1.5 | 200 | 190 | 12.5 |
| Z12-2 | 1850 | 1.5 | 200 | 190 | 14 |
| Z12-3 | 1850 | 1.5 | 200 | 190 | 11 |
| Z5-1 | 1150 | 1.5 | 114 | 80 | 10 |
| Z5-2 | 1150 | 1.5 | 120 | 80 | 10 |
| Z5-3 | 1150 | 1.5 | 108 | 80 | 10 |
| Z11-1 | 1430 | 1.2 | 150 | 140 | 10 |
| Z11-2 | 1430 | 1.2 | 150 | 130 | 10 |
| Z11-3 | 1430 | 1.2 | 150 | 150 | 10 |
| Z13-1 | 1950 | 1.9 | 235 | 160 | 12.5 |
| Z13-2 | 1950 | 1.9 | 235 | 160 | 14 |
| Z13-3 | 1950 | 1.9 | 235 | 160 | 11 |

Table 9.15 Rack section geometry

| section | L (mm) | t (mm) | bw (mm) | bf (mm) | bs (mm) | bl (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| R11-1 | 1850 | 2.0 | 175 | 120 | 10 | 10 |
| R11-2 | 1850 | 2.0 | 160 | 120 | 10 | 10 |
| R11-3 | 1850 | 2.0 | 190 | 120 | 10 | 10 |
| R13-1 | 1300 | 2.0 | 200 | 175 | 10 | 10 |
| R13-2 | 1300 | 2.0 | 175 | 175 | 10 | 10 |
| R13-3 | 1300 | 2.0 | 225 | 175 | 10 | 10 |

Table 9.16 Buckling parameters

| section | Ll1 (mm) | Ll2 (mm) | Ld (mm) | σ (mm) | σl1 (MPa) | σl2 (MPa) | σd (MPa) | kl1 | kl2 | kd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH1-5-800 | 101 | 72 | 332 | 43.2 | 85.4 | 155.3 | 44.9 | 0.70 | 0.60 | 0.67 |
| CH1-6-800 | 101 | 72 | 396 | 57.3 | 85.1 | 155.9 | 61.0 | 0.70 | 0.60 | 0.66 |
| section | Ll1 (mm) | Ll2 (mm) | Ld (mm) | σ (mm) | σl1 (MPa) | σl2 (MPa) | σd (MPa) | kl1 | kl2 | kd |
| CH1-7-400 | 101 | 72 | 445 | 85.1 | 88.1 | 160.3 | 130.8 | 0.71 | 0.61 | 0.66 |
| CH1-7-600 | 101 | 72 | 445 | 78.4 | 85.2 | 157.6 | 87.1 | 0.71 | 0.61 | 0.66 |
| CH1-7-800 | 101 | 72 | 445 | 72.9 | 84.1 | 156.5 | 80.4 | 0.71 | 0.61 | 0.66 |
| CH2-7-800 | 76 | — | 464 | 84.8 | 147.8 | — | 89.0 | 0.67 | — | 0.67 |
| CH2-7-1000 | 76 | — | 464 | 73.2 | 147.4 | — | 75.7 | 0.67 | — | 0.67 |
| CH2-8-1000 | 76 | — | 512 | 88.6 | 148.2 | — | 92.0 | 0.67 | — | 0.66 |
| CH2-10-1000 | 76 | — | 596 | 123.4 | 149.9 | — | 128.1 | 0.68 | — | 0.65 |
| CH2-12-1000 | 76 | — | 683 | 148.0 | 150.2 | — | 153.6 | 0.68 | — | 0.64 |
| CH2-14-1000 | 76 | — | 757 | 150.5 | 151.6 | — | 191.2 | 0.68 | — | 0.64 |
| H5-1 | 147 | 94 | 605 | 113.2 | 114.2 | 288.0 | 147.5 | 0.73 | 0.67 | 0.66 |
| H5-2 | 152 | 108 | 670 | 104.2 | 110.4 | 212.0 | 113.3 | 0.71 | 0.62 | 0.66 |
| H5-3 | 143 | 79 | 539 | 116.4 | 117.2 | 420.6 | 181.1 | 0.72 | 0.70 | 0.65 |
| H11-1 | 143 | 105 | 848 | 53.7 | 54.4 | 81.7 | 63.0 | 0.50 | 0.50 | 0.65 |
| H11-2 | 137 | 101 | 809 | 57.3 | 57.8 | 89.9 | 71.9 | 0.60 | 0.50 | 0.66 |
| H11-3 | 150 | 108 | 885 | 49.4 | 50.4 | 76.4 | 55.7 | 0.44 | 0.61 | 0.65 |
| H12-1 | 193 | 142 | 1126 | 46.2 | 47.0 | 70.4 | 53.0 | 0.48 | 0.52 | 0.65 |
| H12-2 | 193 | 142 | 1220 | 46.5 | 47.1 | 70.5 | 60.7 | 0.48 | 0.52 | 0.65 |
| H12-3 | 193 | 142 | 1032 | 40.6 | 46.9 | 70.3 | 45.2 | 0.47 | 0.52 | 0.65 |
| H13-1 | 196 | 135 | 930 | 64.5 | 64.9 | 137.4 | 74.2 | 0.72 | 0.64 | 0.66 |
| H13-2 | 196 | 135 | 1005 | 64.6 | 64.9 | 137.9 | 88.5 | 0.72 | 0.65 | 0.66 |
| H13-3 | 196 | 135 | 853 | 58.8 | 64.9 | 136.9 | 62.4 | 0.72 | 0.64 | 0.66 |
| Z2-1 | 147 | 94 | 607 | 112.9 | 114.2 | 288.0 | 145.2 | 0.73 | 0.67 | 0.65 |
| Z2-2 | 152 | 108 | 671 | 103.1 | 110.3 | 212.0 | 112.5 | 0.71 | 0.62 | 0.66 |
| Z2-3 | 143 | 79 | 541 | 116.0 | 117.2 | 420.6 | 174.5 | 0.72 | 0.70 | 0.63 |
| Z5-1 | 96 | 67 | 501 | 169.2 | 170.5 | 348.6 | 190.7 | 0.73 | 0.65 | 0.65 |
| Z5-2 | 100 | 67 | 505 | 154.5 | 155.8 | 345.3 | 186.1 | 0.74 | 0.66 | 0.65 |
| Z5-3 | 91 | 66 | 496 | 181.4 | 193.3 | 354.5 | 197.0 | 0.72 | 0.62 | 0.66 |
| Z8-1 | 79 | 44 | 271 | 102.0 | 103.4 | 438.1 | 116.6 | 0.72 | 0.71 | 0.60 |
| Z8-2 | 78 | 40 | 249 | 104.6 | 104.6 | 572.6 | 130.7 | 0.71 | 0.71 | 0.56 |
| Z8-3 | 80 | 48 | 291 | 95.1 | 102.2 | 345.9 | 102.6 | 0.73 | 0.70 | 0.62 |
| Z11-1 | 143 | 105 | 833 | 53.4 | 54.4 | 81.8 | 63.0 | 0.50 | 0.51 | 0.65 |
| Z11-2 | 137 | 102 | 795 | 57.1 | 57.8 | 90.3 | 71.4 | 0.61 | 0.50 | 0.65 |
| Z11-3 | 150 | 108 | 870 | 49.2 | 50.4 | 76.4 | 55.8 | 0.44 | 0.61 | 0.64 |
| Z12-1 | 193 | 142 | 1102 | 46.0 | 47.0 | 70.7 | 53.0 | 0.48 | 0.53 | 0.64 |
| section | Ll1 (mm) | Ll2 (mm) | Ld (mm) | σ (mm) | σl1 (MPa) | σl2 (MPa) | σd (MPa) | kl1 | kl2 | kd |
| Z12-2 | 193 | 142 | 1194 | 46.3 | 47.1 | 70.9 | 60.7 | 0.48 | 0.53 | 0.64 |
| Z12-3 | 193 | 142 | 1007 | 38.2 | 46.9 | 70.6 | 45.0 | 0.47 | 0.53 | 0.64 |
| Z13-1 | 196 | 135 | 904 | 64.2 | 64.9 | 138.4 | 73.3 | 0.72 | 0.65 | 0.63 |
| Z13-2 | 196 | 135 | 978 | 64.4 | 64.9 | 138.9 | 87.2 | 0.72 | 0.65 | 0.63 |
| Z13-3 | 196 | 135 | 827 | 55.7 | 64.9 | 137.9 | 61.8 | 0.72 | 0.64 | 0.63 |
| R11-1 | 141 | 87 | 797 | 130.0 | 132.6 | 346.7 | 143.6 | 0.76 | 0.69 | 0.63 |
| R11-2 | 131 | 86 | 785 | 139.7 | 155.9 | 353.9 | 148.9 | 0.76 | 0.67 | 0.63 |
| R11-3 | 151 | 88 | 807 | 111.9 | 114.0 | 341.4 | 138.3 | 0.75 | 0.70 | 0.62 |
| R13-1 | 169 | 122 | 1047 | 91.7 | 96.9 | 160.4 | 104.4 | 0.73 | 0.59 | 0.63 |
| R13-2 | 155 | 117 | 1025 | 97.2 | 116.9 | 172.3 | 108.3 | 0.61 | 0.52 | 0.63 |
| R13-3 | 185 | 124 | 1067 | 76.7 | 79.6 | 155.8 | 100.5 | 0.75 | 0.65 | 0.63 |

Table 9.17 Buckling modal shapes

| section | D | L1 | L2 |
| --- | --- | --- | --- |
| CH1-6 |  |  |  |
| CH2-7 |  |  | None |
| H5-1 |  |  |  |
| Z2-1 |  |  |  |

Table 9.18 Verification of the multi-linear model

| CH1-5-800 | CH1-6-800 |
| --- | --- |
| CH2-7-800 | CH2-7-1000 |
| H5-1 | Z2-1 |
| Multi-linear model ABAQUS analysis |  |

### Verification against experimental results

Young [107] conducted experimental research of local-distortional interactive buckling of lipped channel sections, where one example of the load-end shortening curves is obtained. Thus the multi-linear model approximating the load-end shortening curve of a perfect column made of an elastic material is verified against this example. The geometry of the lipped channel section is tabulated in Table 9.19. The definitions of the the notations were shown in Fig. 9.6.

Table 9.19 Section geometry

| section | L (mm) | t (mm) | bw (mm) | bf (mm) | d (mm) | r (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| T1.5-HSS-3 | 1852 | 1.478 | 203.7 | 192.7 | 19.0 | 2.42 |

The result of the comparison is shown in Fig. 9.12, where the red curve represents the multi-linear model based on the parameters tabulated in Table 9.20, while the blue curve represents the experimental result. It is seen that the agreement between the two is satisfactory in a range of longitudinal stress from the critical stress of 46 MPa to around 80 MPa, while beyond 80 MPa the blue curve deviates from the red curve due to the yielding of the material.

Table 9.20 Buckling parameters

| section | Ll1 (mm) | Ll2 (mm) | Ld (mm) | σl1 (MPa) | σl2 (MPa) | σd (MPa) | kl1 | kl2 | kd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1.5-HSS-3 | 192 | 141 | 1411 | 45.9 | 68.8 | 81.1 | 0.5 | 0.52 | 0.69 |

Fig. 9.12 Verification of the multi-linear model

# DSM equations for CFS column

## Introduction

This chapter focuses on improving the current version of the DSM design equations, while making use of the multi-linear model proposed in the previous chapter.

## Design approach

### Design formula

In addition to the slenderness parameters, which are currently already incorporated in the DSM, a new parameter is introduced to the design formula, namely the average post-buckling stiffness Kp = Δσ/Δε. As shown in Fig. 10.1, the solid line represents the longitudinal stress-strain curve of elastic buckling of a geometrically perfect column, while the dashed line represents the stress-strain curve of elastic-plastic buckling of the same column but with geometric imperfections. Δσ and Δε are the increments of stress and strain on the elastic buckling curve between the critical stress σcr and the ultimate stress σult. Correspondingly, the relative value of Kp is defined as kp = Kp/E.

Fig. 10.1 Average post-buckling stiffness

In general, the design formula should satisfy the following requirements in two extreme cases:

- When kp = 0, which means the column has no residual capacity after buckling, such as in the case of global buckling, the ultimate load should equal the critical load: 
Pu = Pcr or Pu = (Pcr/Py) Py = λ-2 Py ≤ Py, where λ is the slenderness.

- When kp ≈ 1, which means that the influence of a minor part of the section buckling is so small that the column as a whole can be considered unbuckled, the ultimate load should equal the yield load: Pu = Py.

Therefore, the following formula is proposed:

where g is a function of kp, and reflects the effect of kp on the ultimate load. The function g should satisfy:  g(1) = 0 and g(0) = 1.

Practically, a reduction should be applied to the ultimate load to take into account the effect of imperfections. The proposed approach is inspired by the EC3 design formula for global buckling of hot-rolled and welded members, where the ultimate load Pu = λ-2 Py ≤ Py is reduced using the Ayrton-Perry formula:

where α is the imperfection factor. A similar formula is adapted in our design approach. Since in Eq.  the power of λ is not necessarily equal to 2, but changes depending on the value of kp, the formula is revised accordingly:

where it is ensured that

Meanwhile, the imperfection factor α, which is constant in Eq. , needs to be inversely proportional to kp in Eq.  in order to reflect the reduced imperfection sensitivity when kp is high. Since kp is also inversely proportional to g, the imperfection factor α can be made directly proportional to g, and is assumed to take the form:

Fig. 10.2 Illustration of the design formula

where α0 is a constant factor indicating the magnitude of imperfection. It is found that when 
α0 = 0.3, a best agreement is observed between the proposed and current design formulas, as illustrated in Fig. 10.2, where the horizontal axis represents the slenderness , with σy and σcr being the yield stress of the material and the critical stress of the column, respectively. The vertical axis represents the ratio of the ultimate load to the yield load. The dark blue curves represent (from the bottom to the top) the current DSM design formulas for global, distortional and local buckling respectively. The dashed lines in varying colours represent the proposed design formulas for the perfect column using Eq., while the solid lines in the same colours represent the design formulas for the imperfect column using Eq.. It can be seen that the reduction of the ultimate load due to imperfections is maximal when the slenderness λ = 1, which reflects the fact that the imperfection sensitivity of the column is high when the yield stress of the material coincides with the critical stress of the column. This has been confirmed by Becque in his previous studies [13]. In addition, it is also noted that for a slenderness λ = 1, the reduction of the ultimate load is directly proportional to g, as previously addressed.

### Determination of the function g

Since the factor g in the design formula in Eq.  is a function of the average post-buckling stiffness kp, the exact relation can be found by following two steps. First, by substituting the slenderness and the ultimate load, known from either experimental study or numerical modelling, into the design formula, a solution for g can be found. Second, when the ultimate load is known, kp can be determined from the longitudinal stress-train curves obtained from an elastic geometrically nonlinear ABAQUS analysis and used in the previous chapter to verify the multi-linear model. The calculated values of kp and g for pure local buckling are tabulated in Table 10.1, while those of distortional and local-distortional interactive buckling are tabulated in Table 10.2. The ultimate loads of local buckling are obtained from numerical studies, while those of local-distortional interactive buckling are obtained from experimental [25] and numerical studies [43], marked by ‘*’ and ‘Δ’ respectively.

Table 10.1 kp and g for local buckling of box-section

| bw, bf, tf, fy (mm, MPa) | tw (mm) | Pu (MPa) | kp | g | bw, bf, tf, fy (mm, MPa) | tw (mm) | Pu (MPa) | kp | g |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bw = 100, bf = 100, tf = 1, fy = 350 | 1 | 138.6 | 0.35 | 0.53 | bw = 100, bf = 110, tf = 1, fy = 350 | 1 | 134.3 | 0.35 | 0.52 |
|  | 1.2 | 149.4 | 0.36 | 0.54 |  | 1.2 | 144.3 | 0.37 | 0.52 |
|  | 1.4 | 159.8 | 0.40 | 0.54 |  | 1.4 | 155.3 | 0.43 | 0.50 |
|  | 1.6 | 173.7 | 0.56 | 0.50 |  | 1.6 | 168.6 | 0.55 | 0.47 |
|  | 1.8 | 191.6 | 0.71 | 0.44 |  | 1.8 | 186.1 | 0.71 | 0.41 |
|  | 2 | 213.8 | 0.78 | 0.36 |  | 2 | 205.5 | 0.76 | 0.35 |
|  | 2.2 | 234.1 | 0.82 | 0.30 |  | 2.2 | 224.6 | 0.82 | 0.29 |
| bw, bf, tf, fy (mm, MPa) | tw (mm) | Pu (MPa) | kp | g | bw, bf, tf, fy (mm, MPa) | tw (mm) | Pu (MPa) | kp | g |
|  | 2.4 | 253.1 | 0.84 | 0.24 |  | 2.4 | 243.2 | 0.83 | 0.24 |
|  | 2.6 | 268.8 | 0.85 | 0.19 |  | 2.6 | 259.3 | 0.84 | 0.19 |
|  | 2.8 | 279.8 | 0.86 | 0.16 |  | 2.8 | 271.2 | 0.85 | 0.17 |
|  | 3 | 288.4 | 0.87 | 0.14 |  | 3 | 279.4 | 0.86 | 0.15 |
|  | 4 | 309.5 | 0.90 | 0.09 |  | 4 | 303.4 | 0.89 | 0.09 |
|  | 5 | 316.2 | 0.91 | 0.07 |  | 5 | 311.3 | 0.91 | 0.08 |
| bw = 100, bf = 120, tf = 1, fy = 350 | 1 | 129.8 | 0.35 | 0.50 | bw = 100, bf = 130, tf = 1, fy = 350 | 1 | 125.6 | 0.35 | 0.49 |
|  | 1.2 | 140.0 | 0.38 | 0.50 |  | 1.2 | 136.3 | 0.41 | 0.48 |
|  | 1.4 | 151.1 | 0.44 | 0.48 |  | 1.4 | 147.2 | 0.46 | 0.45 |
|  | 1.6 | 165.0 | 0.54 | 0.44 |  | 1.6 | 159.9 | 0.56 | 0.42 |
|  | 1.8 | 179.6 | 0.67 | 0.40 |  | 1.8 | 176.2 | 0.65 | 0.37 |
|  | 2 | 197.7 | 0.75 | 0.34 |  | 2 | 192.3 | 0.72 | 0.33 |
|  | 2.2 | 250.1 | 0.81 | 0.20 |  | 2.2 | 211.2 | 0.80 | 0.28 |
|  | 2.4 | 235.7 | 0.82 | 0.24 |  | 2.4 | 257.2 | 0.81 | 0.17 |
|  | 2.6 | 252.0 | 0.83 | 0.19 |  | 2.6 | 245.5 | 0.82 | 0.19 |
|  | 2.8 | 264.4 | 0.84 | 0.17 |  | 2.8 | 258.0 | 0.83 | 0.17 |
|  | 3 | 272.7 | 0.85 | 0.15 |  | 3 | 266.3 | 0.84 | 0.15 |
|  | 4 | 297.8 | 0.88 | 0.09 |  | 4 | 292.2 | 0.87 | 0.10 |
|  | 5 | 306.7 | 0.90 | 0.08 |  | 5 | 302.2 | 0.89 | 0.08 |
| bw = 100, bf = 140, tf = 1, fy = 350 | 1 | 121.5 | 0.36 | 0.48 | bw = 100, bf = 150, tf = 1, fy = 350 | 1 | 117.7 | 0.35 | 0.47 |
|  | 1.2 | 132.1 | 0.39 | 0.46 |  | 1.2 | 128.4 | 0.40 | 0.45 |
|  | 1.4 | 143.5 | 0.45 | 0.44 |  | 1.4 | 140.0 | 0.47 | 0.42 |
|  | 1.6 | 157.3 | 0.55 | 0.40 |  | 1.6 | 154.0 | 0.53 | 0.39 |
|  | 1.8 | 170.8 | 0.65 | 0.36 |  | 1.8 | 167.2 | 0.63 | 0.35 |
|  | 2 | 187.8 | 0.72 | 0.32 |  | 2 | 184.0 | 0.67 | 0.31 |
|  | 2.2 | 206.2 | 0.79 | 0.27 |  | 2.2 | 201.9 | 0.78 | 0.26 |
|  | 2.4 | 223.9 | 0.81 | 0.23 |  | 2.4 | 219.3 | 0.80 | 0.22 |
|  | 2.6 | 239.8 | 0.82 | 0.19 |  | 2.6 | 234.6 | 0.81 | 0.19 |
|  | 2.8 | 252.2 | 0.83 | 0.17 |  | 2.8 | 246.7 | 0.82 | 0.17 |
|  | 3 | 260.4 | 0.83 | 0.15 |  | 3 | 254.9 | 0.83 | 0.15 |
|  | 4 | 286.7 | 0.87 | 0.10 |  | 4 | 281.3 | 0.86 | 0.10 |
|  | 5 | 297.5 | 0.89 | 0.08 |  | 5 | 293.0 | 0.88 | 0.08 |

Table 10.2 kp and g for distortional buckling

| section | fy (MPa) | Pu (MPa) | kp | g | section | fy (MPa) | Pu (MPa) | kp | g |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH1-5-800* | 590 | 147.8 | 0.32 | 0.50 | CH2-7-1000* | 590 | 193.2 | 0.53 | 0.49 |
| CH1-6-800* | 590 | 147.3 | 0.31 | 0.56 | CH2-8-1000* | 590 | 192.1 | 0.55 | 0.54 |
| CH1-7-400* | 590 | 160.0 | 0.26 | 0.62 | CH2-10-1000* | 590 | 202.2 | 0.40 | 0.60 |
| CH1-7-600* | 590 | 155.7 | 0.31 | 0.61 | CH2-12-1000* | 590 | 206.2 | 0.47 | 0.66 |
| CH1-7-800* | 590 | 149.5 | 0.29 | 0.61 | CH2-14-1000* | 590 | 214.4 | 0.42 | 0.64 |
| CH2-7-800* | 590 | 198.5 | 0.55 | 0.51 |  |  |  |  |  |
| H5-1Δ | 250 | 133.0 | 0.45 | 0.57 | H5-2 Δ | 250 | 120.0 | 0.44 | 0.63 |
|  | 350 | 158.0 | 0.48 | 0.57 |  | 350 | 143.0 | 0.39 | 0.62 |
|  | 550 | 198.0 | 0.42 | 0.57 |  | 550 | 181.0 | 0.33 | 0.60 |
|  | 750 | 233.0 | 0.32 | 0.56 |  | 750 | 213.0 | 0.28 | 0.59 |
| H5-3 Δ | 250 | 147.0 | 0.70 | 0.48 | H11-1 Δ | 250 | 82.0 | 0.27 | 0.64 |
|  | 350 | 160.0 | 0.70 | 0.57 |  | 350 | 99.0 | 0.24 | 0.62 |
|  | 550 | 219.0 | 0.53 | 0.52 |  | 550 | 125.0 | 0.21 | 0.60 |
|  | 750 | 254.0 | 0.44 | 0.53 |  | 750 | 148.0 | 0.19 | 0.58 |
| H11-2 Δ | 250 | 86.0 | 0.28 | 0.64 | H11-3 Δ | 250 | 77.0 | 0.26 | 0.65 |
|  | 350 | 104.0 | 0.25 | 0.61 |  | 350 | 92.0 | 0.23 | 0.63 |
|  | 550 | 132.0 | 0.22 | 0.59 |  | 550 | 118.0 | 0.20 | 0.60 |
|  | 750 | 155.0 | 0.20 | 0.58 |  | 750 | 139.0 | 0.18 | 0.59 |
| H12-1 Δ | 150 | 58.0 | 0.29 | 0.67 | H12-2 Δ | 150 | 58.0 | 0.35 | 0.67 |
|  | 250 | 76.0 | 0.26 | 0.63 |  | 250 | 76.0 | 0.28 | 0.64 |
|  | 350 | 92.0 | 0.23 | 0.61 |  | 350 | 91.0 | 0.25 | 0.62 |
|  | 550 | 117.0 | 0.20 | 0.59 |  | 550 | 112.0 | 0.21 | 0.61 |
| H12-3 Δ | 150 | 59.0 | 0.35 | 0.61 | H13-1 Δ | 150 | 76.0 | 0.53 | 0.59 |
|  | 250 | 76.0 | 0.29 | 0.59 |  | 250 | 99.0 | 0.35 | 0.58 |
|  | 350 | 90.0 | 0.26 | 0.59 |  | 350 | 117.0 | 0.30 | 0.58 |
|  | 550 | 116.0 | 0.22 | 0.57 |  | 550 | 144.0 | 0.25 | 0.58 |
| H13-2 Δ | 150 | 0.64 | 0.64 | 0.63 | H13-3 Δ | 150 | 78.0 | 0.44 | 0.53 |
|  | 250 | 0.66 | 0.66 | 0.73 |  | 250 | 100.0 | 0.36 | 0.55 |
|  | 350 | 0.35 | 0.35 | 0.61 |  | 350 | 119.0 | 0.32 | 0.54 |
|  | 550 | 0.27 | 0.27 | 0.59 |  | 550 | 150.0 | 0.27 | 0.54 |
|  |  |  |  |  |  | 750 | 175 | 0.25 | 0.54 |
| Z2-1 Δ | 250 | 132.0 | 0.65 | 0.58 | Z2-2 Δ | 250 | 120.0 | 0.37 | 0.63 |
|  | 350 | 157.0 | 0.41 | 0.58 |  | 350 | 143.0 | 0.33 | 0.61 |
|  | 550 | 197.0 | 0.34 | 0.57 |  | 550 | 182.0 | 0.29 | 0.59 |
| section | fy (MPa) | Pu (MPa) | kp | g | section | fy (MPa) | Pu (MPa) | kp | g |
| Z2-1 Δ | 750 | 231.0 | 0.30 | 0.57 | Z2-2 Δ | 750 | 215.0 | 0.26 | 0.58 |
| Z2-3 Δ | 250 | 145.0 | 0.70 | 0.49 | Z5-1 Δ | 350 | 178.0 | 0.37 | 0.65 |
|  | 350 | 178.0 | 0.61 | 0.49 |  | 550 | 229.0 | 0.37 | 0.62 |
|  | 550 | 225.0 | 0.46 | 0.50 |  | 750 | 261.0 | 0.32 | 0.62 |
|  | 750 | 264.0 | 0.37 | 0.50 |  |  |  |  |  |
| Z5-2 Δ | 350 | 184.0 | 0.63 | 0.57 | Z5-3 Δ | 350 | 196.0 | 0.62 | 0.58 |
|  | 550 | 222.0 | 0.39 | 0.61 |  | 550 | 233.0 | 0.40 | 0.63 |
|  | 750 | 260.0 | 0.33 | 0.60 |  | 750 | 269.0 | 0.33 | 0.63 |
| Z8-1 Δ | 250 | 126.0 | 0.47 | 0.58 | Z8-2 Δ | 250 | 129.0 | 0.49 | 0.56 |
|  | 350 | 151.0 | 0.41 | 0.57 |  | 350 | 151.0 | 0.50 | 0.58 |
|  | 550 | 185.0 | 0.36 | 0.58 |  | 550 | 186.0 | 0.44 | 0.58 |
|  | 750 | 207.0 | 0.34 | 0.60 |  | 750 | 206.0 | 0.41 | 0.60 |
| Z8-3 Δ | 250 | 123.0 | 0.44 | 0.57 | Z11-1 Δ | 150 | 67.0 | 0.26 | 0.63 |
|  | 350 | 145.0 | 0.42 | 0.57 |  | 250 | 77.0 | 0.25 | 0.68 |
|  | 550 | 186.0 | 0.37 | 0.56 |  | 350 | 105.0 | 0.22 | 0.59 |
|  | 750 | 208.0 | 0.35 | 0.57 |  | 550 | 126.0 | 0.20 | 0.60 |
|  |  |  |  |  |  | 750 | 148.0 | 0.18 | 0.59 |
| Z11-2 Δ | 150 | 62.0 | 0.48 | 0.73 | Z11-3 Δ | 150 | 58.0 | 0.33 | 0.71 |
|  | 250 | 86.0 | 0.32 | 0.64 |  | 250 | 81.0 | 0.27 | 0.63 |
|  | 350 | 102.0 | 0.28 | 0.62 |  | 350 | 97.0 | 0.24 | 0.61 |
|  | 550 | 113.0 | 0.27 | 0.66 |  | 550 | 120.0 | 0.21 | 0.60 |
|  | 750 | 131.0 | 0.24 | 0.65 |  | 750 | 145.0 | 0.19 | 0.58 |
| Z12-1 Δ | 150 | 58.0 | 0.28 | 0.68 | Z12-2 Δ | 150 | 58.0 | 0.36 | 0.68 |
|  | 250 | 72.0 | 0.26 | 0.67 |  | 250 | 76.0 | 0.28 | 0.64 |
|  | 350 | 84.0 | 0.24 | 0.66 |  | 350 | 91.0 | 0.25 | 0.62 |
|  | 550 | 117.0 | 0.20 | 0.59 |  | 550 | 112.0 | 0.21 | 0.61 |
|  | 750 | 137.0 | 0.18 | 0.59 |  | 750 | 133.0 | 0.19 | 0.60 |
| Z12-3 Δ | 150 | 59.0 | 0.34 | 0.58 | Z13-1 Δ | 150 | 73.0 | 0.51 | 0.64 |
|  | 250 | 76.0 | 0.28 | 0.58 |  | 250 | 99.0 | 0.35 | 0.59 |
|  | 350 | 91.0 | 0.25 | 0.57 |  | 350 | 117.0 | 0.30 | 0.58 |
|  | 550 | 116.0 | 0.21 | 0.55 |  | 550 | 144.0 | 0.26 | 0.58 |
|  | 750 | 135.0 | 0.18 | 0.55 |  | 750 | 169.0 | 0.23 | 0.58 |
| Z13-2 Δ | 150 | 72.0 | 0.62 | 0.65 | Z13-3 Δ | 150 | 84.0 | 0.43 | 0.45 |
|  | 250 | 82.0 | 0.61 | 0.72 |  | 250 | 100.0 | 0.36 | 0.53 |
|  | 350 | 111.0 | 0.36 | 0.61 |  | 350 | 115.0 | 0.33 | 0.55 |
| section | fy (MPa) | Pu (MPa) | kp | g | section | fy (MPa) | Pu (MPa) | kp | g |
| Z13-2 Δ | 550 | 145.0 | 0.28 | 0.58 | Z13-3 Δ | 550 | 155.0 | 0.27 | 0.51 |
|  | 750 | 170.0 | 0.24 | 0.57 |  | 750 | 185.0 | 0.24 | 0.51 |
| R11-1 Δ | 250 | 148.0 | 0.47 | 0.53 | R11-2 Δ | 250 | 155.0 | 0.55 | 0.50 |
|  | 350 | 170.0 | 0.36 | 0.58 |  | 350 | 162.0 | 0.53 | 0.64 |
|  | 550 | 193.0 | 0.31 | 0.64 |  | 550 | 200.0 | 0.37 | 0.64 |
|  | 750 | 219.0 | 0.27 | 0.64 |  | 750 | 229.0 | 0.30 | 0.64 |
|  | 1000 | 255.0 | 0.22 | 0.63 |  | 1000 | 262.0 | 0.26 | 0.63 |
|  | 1200 | 282.0 | 0.20 | 0.61 |  | 1200 | 288.0 | 0.23 | 0.62 |
| R11-3 Δ | 250 | 143.0 | 0.65 | 0.50 | R13-1 Δ | 250 | 109.0 | 0.32 | 0.66 |
|  | 350 | 166.0 | 0.42 | 0.54 |  | 350 | 115.0 | 0.31 | 0.72 |
|  | 550 | 192.0 | 0.34 | 0.59 |  | 550 | 155.0 | 0.25 | 0.64 |
|  | 750 | 221.0 | 0.29 | 0.59 |  | 750 | 182.0 | 0.22 | 0.63 |
|  | 1000 | 254.0 | 0.25 | 0.59 |  | 1000 | 209.0 | 0.20 | 0.62 |
|  | 1200 | 277.0 | 0.22 | 0.58 |  | 1200 | 227.0 | 0.19 | 0.62 |
| R13-2 Δ | 250 | 109.0 | 0.29 | 0.68 | R13-3 Δ | 250 | 99.3 | 0.46 | 0.67 |
|  | 350 | 126.0 | 0.34 | 0.68 |  | 350 | 118.0 | 0.36 | 0.65 |
|  | 550 | 160.0 | 0.28 | 0.65 |  | 550 | 149.0 | 0.28 | 0.62 |
|  | 750 | 188.0 | 0.25 | 0.63 |  | 750 | 175.0 | 0.25 | 0.61 |
|  | 1000 | 217.0 | 0.22 | 0.62 |  | 1000 | 201.0 | 0.22 | 0.60 |
|  | 1200 | 236.0 | 0.21 | 0.61 |  | 1200 | 219.0 | 0.20 | 0.60 |

Note: Sections with the ultimate load obtained from experimental and numerical studies are marked by ‘*’ and ‘Δ’ respectively.

Once both g and kp are known, the former is plotted against the latter in Fig. 10.3, where the light blue dots represent the data points associated with distortional buckling and local-distortional interactive buckling, while the red dots represent the data points associated with pure local buckling. The dark blue line represents the proposed design formula to predict g when kp is known, which reads:

Fig. 10.3 Relation between kp and g

### Predicting ultimate loads

After calibrating the proposed model in the previous section, a further verification is carried out, whereby the relative average post-buckling stiffness kp is determined using the multi-linear model proposed in the previous chapter. We define the ultimate stress σu = Pu/Ag, where Ag is the gross area of the cross-section. It can be seen that kp is a function of the ultimate stress σu: kp = f1 (σu). For example, for the multi-linear model shown in Fig. 9.1, the function f1 reads:

The function f1 is monotonically decreasing, reaching its minimum value kpmin when the ultimate load equals σy, i.e. f1(σy) = kpmin. On the other hand, when σ1 ≤ σu ≤ σ2, the function f1 reaches its maximum value k1. Therefore the relative average post-buckling stiffness .

Two considerations are important when applying Eq. . First, it is proved in the previous chapter that when the initial post-buckling stiffness associated with the primary mode is lower than that associated with the secondary mode: k1 < k2, the column should be considered to buckle solely in the primary mode so that kp = k1. Second, while the multi-linear model simulates the elastic post-buckling behaviour of a geometrically perfect column, in practice, due to imperfections, the post-buckling stiffness is not reduced abruptly as simulated at bifurcation points, but gradually. This results in an overestimation of kp. Therefore, a reduction factor β = 0.55 is applied to reduce the stresses of the secondary and tertiary bifurcation points as follows:

As previously pointed out, kp is a function of the ultimate load σu. Conversely, according to Eq., σu is also a function of kp: σu = f2 (kp). The function f2 is monotonically increasing, with .

We now define the monotonically increasing function

Then the goal of the design process is to find the solution of the equation

Since F(kpmin) < 0 and F(k1) > 0, a unique solution of kp can be found.

Finally, the ultimate load predictions made by the improved DSM are verified against ultimate loads obtained from experiments and numerical modelling. The results for pure local buckling of box-sections are tabulated in Table 10.3, while the results for distortional and local-distortional interactive buckling of various open sections are tabulated in Table 10.4. In both tables, σu′ and σu″ are the ultimate loads predicted by the proposed design approach and the DSM, respectively, σu is the ultimate load obtained from either numerical or experimental study.

Table 10.3 Predictions of the ultimate load of box-sections in local buckling

| bw, bf, tf, fy (mm, MPa) | tw (mm) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| --- | --- | --- | --- | --- | --- | --- |
| bw = 100, bf = 100, tf = 1, fy = 350 | 1 | 130.0 | 174.5 | 138.6 | 0.94 | 1.26 |
|  | 1.2 | 148.5 | 186.6 | 149.4 | 0.99 | 1.25 |
|  | 1.4 | 160.4 | 196.7 | 159.8 | 1.00 | 1.23 |
|  | 1.6 | 189.0 | 203.1 | 173.7 | 1.09 | 1.17 |
|  | 1.8 | 212.6 | 206.5 | 191.6 | 1.11 | 1.08 |
|  | 2 | 233.9 | 208.4 | 213.8 | 1.09 | 0.97 |
|  | 2.2 | 258.3 | 209.6 | 234.1 | 1.10 | 0.90 |
|  | 2.4 | 289.0 | 210.5 | 253.1 | 1.14 | 0.83 |
|  | 2.6 | 301.4 | 211.0 | 268.8 | 1.12 | 0.79 |
|  | 2.8 | 304.0 | 211.5 | 279.8 | 1.09 | 0.76 |
|  | 3 | 306.3 | 211.8 | 288.4 | 1.06 | 0.73 |
|  | 4 | 314.8 | 212.5 | 309.5 | 1.02 | 0.69 |
|  | 5 | 320.5 | 212.8 | 316.2 | 1.01 | 0.67 |
| bw = 100, bf = 110, tf = 1, fy = 350 | 1 | 125.8 | 167.6 | 134.3 | 0.94 | 1.25 |
|  | 1.2 | 148.1 | 177.4 | 144.3 | 1.03 | 1.23 |
|  | 1.4 | 152.6 | 185.1 | 155.3 | 0.98 | 1.19 |
|  | 1.6 | 178.1 | 190.0 | 168.6 | 1.06 | 1.13 |
|  | 1.8 | 199.3 | 192.9 | 186.1 | 1.07 | 1.04 |
|  | 2 | 219.2 | 194.6 | 205.5 | 1.07 | 0.95 |
|  | 2.2 | 241.7 | 195.8 | 224.6 | 1.08 | 0.87 |
|  | 2.4 | 271.1 | 196.6 | 243.2 | 1.11 | 0.81 |
|  | 2.6 | 286.2 | 197.1 | 259.3 | 1.10 | 0.76 |
|  | 2.8 | 295.8 | 197.5 | 271.2 | 1.09 | 0.73 |
|  | 3 | 298.4 | 197.8 | 279.4 | 1.07 | 0.71 |
|  | 4 | 308.1 | 198.5 | 303.4 | 1.02 | 0.65 |
|  | 5 | 314.7 | 198.8 | 311.3 | 1.01 | 0.64 |
| bw = 100, bf = 120, tf = 1, fy = 350 | 1 | 122.8 | 160.3 | 129.8 | 0.95 | 1.23 |
|  | 1.2 | 150.5 | 168.3 | 140.0 | 1.07 | 1.20 |
|  | 1.4 | 146.7 | 174.5 | 151.1 | 0.97 | 1.15 |
|  | 1.6 | 168.3 | 178.6 | 165.0 | 1.02 | 1.08 |
|  | 1.8 | 187.8 | 181.1 | 179.6 | 1.05 | 1.01 |
|  | 2 | 207.0 | 182.8 | 197.7 | 1.05 | 0.92 |
|  | 2.2 | 227.5 | 183.8 | 250.1 | 0.91 | 0.74 |
|  | 2.4 | 253.9 | 184.6 | 235.7 | 1.08 | 0.78 |
| bw, bf, tf, fy (mm, MPa) | tw (mm) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| bw = 100, bf = 120, tf = 1, fy = 350 | 2.6 | 267.4 | 185.1 | 252.0 | 1.06 | 0.73 |
|  | 2.8 | 279.7 | 185.5 | 264.4 | 1.06 | 0.70 |
|  | 3 | 290.5 | 185.8 | 272.7 | 1.07 | 0.68 |
|  | 4 | 301.4 | 186.5 | 297.8 | 1.01 | 0.63 |
|  | 5 | 308.8 | 186.7 | 306.7 | 1.01 | 0.61 |
| bw = 100, bf = 130, tf = 1, fy = 350 | 1 | 119.3 | 153.0 | 125.6 | 0.95 | 1.22 |
|  | 1.2 | 147.8 | 159.8 | 136.3 | 1.08 | 1.17 |
|  | 1.4 | 139.8 | 165.1 | 147.2 | 0.95 | 1.12 |
|  | 1.6 | 159.2 | 168.6 | 159.9 | 1.00 | 1.05 |
|  | 1.8 | 177.1 | 170.9 | 176.2 | 1.01 | 0.97 |
|  | 2 | 196.0 | 172.4 | 192.3 | 1.02 | 0.90 |
|  | 2.2 | 215.9 | 173.4 | 211.2 | 1.02 | 0.82 |
|  | 2.4 | 235.0 | 174.1 | 257.2 | 0.91 | 0.68 |
|  | 2.6 | 248.8 | 174.6 | 245.5 | 1.01 | 0.71 |
|  | 2.8 | 261.6 | 175.0 | 258.0 | 1.01 | 0.68 |
|  | 3 | 272.5 | 175.3 | 266.3 | 1.02 | 0.66 |
|  | 4 | 291.8 | 175.9 | 292.2 | 1.00 | 0.60 |
|  | 5 | 300.0 | 176.2 | 302.2 | 0.99 | 0.58 |
| bw = 100, bf = 140, tf = 1, fy = 350 | 1 | 117.2 | 146.1 | 121.5 | 0.96 | 1.20 |
|  | 1.2 | 142.9 | 152.1 | 132.1 | 1.08 | 1.15 |
|  | 1.4 | 132.9 | 156.6 | 143.5 | 0.93 | 1.09 |
|  | 1.6 | 150.5 | 159.8 | 157.3 | 0.96 | 1.02 |
|  | 1.8 | 168.0 | 161.9 | 170.8 | 0.98 | 0.95 |
|  | 2 | 185.9 | 163.3 | 187.8 | 0.99 | 0.87 |
|  | 2.2 | 205.2 | 164.3 | 206.2 | 1.00 | 0.80 |
|  | 2.4 | 216.9 | 164.9 | 223.9 | 0.97 | 0.74 |
|  | 2.6 | 228.6 | 165.4 | 239.8 | 0.95 | 0.69 |
|  | 2.8 | 235.6 | 165.7 | 252.2 | 0.93 | 0.66 |
|  | 3 | 242.2 | 166.0 | 260.4 | 0.93 | 0.64 |
|  | 4 | 273.2 | 166.7 | 286.7 | 0.95 | 0.58 |
|  | 5 | 288.1 | 166.9 | 297.5 | 0.97 | 0.56 |
| bw = 100, bf = 150, tf = 1, fy = 350 | 1 | 113.8 | 139.7 | 117.7 | 0.97 | 1.19 |
|  | 1.2 | 137.3 | 145.0 | 128.4 | 1.07 | 1.13 |
|  | 1.4 | 126.3 | 149.1 | 140.0 | 0.90 | 1.07 |
|  | 1.6 | 142.8 | 152.0 | 154.0 | 0.93 | 0.99 |
| bw, bf, tf, fy (mm, MPa) | tw (mm) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| bw = 100, bf = 150, tf = 1, fy = 350 | 1.8 | 159.3 | 153.9 | 167.2 | 0.95 | 0.92 |
|  | 2 | 176.5 | 155.2 | 184.0 | 0.96 | 0.84 |
|  | 2.2 | 194.4 | 156.1 | 201.9 | 0.96 | 0.77 |
|  | 2.4 | 199.4 | 156.8 | 219.3 | 0.91 | 0.71 |
|  | 2.6 | 204.0 | 157.2 | 234.6 | 0.87 | 0.67 |
|  | 2.8 | 208.9 | 157.5 | 246.7 | 0.85 | 0.64 |
|  | 3 | 215.7 | 157.8 | 254.9 | 0.85 | 0.62 |
|  | 4 | 245.6 | 158.4 | 281.3 | 0.87 | 0.56 |
|  | 5 | 272.7 | 158.6 | 293.0 | 0.93 | 0.54 |

Table 10.4 Predictions of the ultimate load in distortional buckling

| Section | fy (MPa) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| --- | --- | --- | --- | --- | --- | --- |
| CH1-5-800 | 590 | 117.4 | 119.1 | 147.8 | 0.79 | 0.81 |
| CH1-6-800 | 590 | 134.7 | 141.5 | 147.3 | 0.91 | 0.96 |
| CH1-7-400 | 590 | 169.0 | 214.8 | 160.0 | 1.06 | 1.34 |
| CH1-7-600 | 590 | 158.5 | 172.4 | 155.7 | 1.02 | 1.11 |
| CH1-7-800 | 590 | 152.5 | 164.9 | 149.5 | 1.02 | 1.10 |
| CH2-7-800 | 590 | 187.3 | 174.4 | 198.5 | 0.94 | 0.88 |
| CH2-7-1000 | 590 | 175.0 | 159.6 | 193.2 | 0.91 | 0.83 |
| CH2-8-1000 | 590 | 189.7 | 177.6 | 192.1 | 0.99 | 0.92 |
| CH2-10-1000 | 590 | 221.1 | 212.4 | 202.2 | 1.09 | 1.05 |
| CH2-12-1000 | 590 | 240.1 | 233.8 | 206.2 | 1.16 | 1.13 |
| CH2-14-1000 | 590 | 245.5 | 261.9 | 214.4 | 1.15 | 1.22 |
| H5-1 | 250 | 142.8 | 149.0 | 133.0 | 1.07 | 1.12 |
|  | 350 | 166.7 | 177.4 | 158.0 | 1.06 | 1.12 |
|  | 550 | 199.7 | 221.4 | 198.0 | 1.01 | 1.12 |
|  | 750 | 227.2 | 256.0 | 233.0 | 0.98 | 1.10 |
| H5-2 | 250 | 131.1 | 131.3 | 120.0 | 1.09 | 1.09 |
|  | 350 | 150.1 | 155.3 | 143.0 | 1.05 | 1.09 |
|  | 550 | 181.7 | 192.5 | 181.0 | 1.00 | 1.06 |
|  | 750 | 207.0 | 221.9 | 213.0 | 0.97 | 1.04 |
| H5-3 | 250 | 152.5 | 163.6 | 147.0 | 1.04 | 1.11 |
|  | 350 | 173.0 | 196.0 | 160.0 | 1.08 | 1.23 |
| Section | fy (MPa) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| H5-3 | 550 | 212.7 | 246.2 | 219.0 | 0.97 | 1.12 |
|  | 750 | 240.4 | 285.6 | 254.0 | 0.95 | 1.12 |
| H11-1 | 250 | 80.6 | 97.4 | 82.0 | 0.98 | 1.19 |
|  | 350 | 91.9 | 113.9 | 99.0 | 0.93 | 1.15 |
|  | 550 | 108.9 | 139.7 | 125.0 | 0.87 | 1.12 |
|  | 750 | 122.1 | 160.1 | 148.0 | 0.83 | 1.08 |
| H11-2 | 250 | 86.6 | 104.4 | 86.0 | 1.01 | 1.21 |
|  | 350 | 99.0 | 122.3 | 104.0 | 0.95 | 1.18 |
|  | 550 | 118.1 | 150.3 | 132.0 | 0.90 | 1.14 |
|  | 750 | 132.9 | 172.4 | 155.0 | 0.86 | 1.11 |
| H11-3 | 250 | 76.8 | 91.2 | 77.0 | 1.00 | 1.18 |
|  | 350 | 87.6 | 106.5 | 92.0 | 0.95 | 1.16 |
|  | 550 | 104.0 | 130.4 | 118.0 | 0.88 | 1.11 |
|  | 750 | 116.9 | 149.3 | 139.0 | 0.84 | 1.07 |
| H12-1 | 150 | 60.0 | 69.6 | 58.0 | 1.03 | 1.20 |
|  | 250 | 73.4 | 88.9 | 76.0 | 0.97 | 1.17 |
|  | 350 | 83.5 | 103.7 | 92.0 | 0.91 | 1.13 |
|  | 550 | 99.0 | 126.8 | 117.0 | 0.85 | 1.08 |
| H12-2 | 150 | 60.6 | 74.5 | 58.0 | 1.04 | 1.28 |
|  | 250 | 74.0 | 95.5 | 76.0 | 0.97 | 1.26 |
|  | 350 | 84.1 | 111.6 | 91.0 | 0.92 | 1.23 |
|  | 550 | 99.6 | 136.8 | 112.0 | 0.89 | 1.22 |
| H12-3 | 150 | 55.6 | 64.1 | 59.0 | 0.94 | 1.09 |
|  | 250 | 67.9 | 81.6 | 76.0 | 0.89 | 1.07 |
|  | 350 | 77.1 | 95.0 | 90.0 | 0.86 | 1.06 |
|  | 550 | 91.2 | 116.0 | 116.0 | 0.79 | 1.00 |
| H13-1 | 150 | 81.3 | 82.2 | 76.0 | 1.07 | 1.08 |
|  | 250 | 100.5 | 106.1 | 99.0 | 1.01 | 1.07 |
|  | 350 | 115.7 | 124.4 | 117.0 | 0.99 | 1.06 |
|  | 550 | 139.7 | 152.9 | 144.0 | 0.97 | 1.06 |
| H13-2 | 150 | 84.2 | 89.4 | 73.0 | 1.15 | 1.22 |
|  | 250 | 102.2 | 116.1 | 80.0 | 1.28 | 1.45 |
|  | 350 | 117.4 | 136.6 | 111.0 | 1.06 | 1.23 |
|  | 550 | 141.9 | 168.4 | 142.0 | 1.00 | 1.19 |
| H13-3 | 150 | 76.9 | 75.5 | 78.0 | 0.99 | 0.97 |
| Section | fy (MPa) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| H13-3 | 250 | 95.2 | 96.9 | 100.0 | 0.95 | 0.97 |
|  | 350 | 109.6 | 113.3 | 119.0 | 0.92 | 0.95 |
|  | 550 | 132.3 | 138.9 | 150.0 | 0.88 | 0.93 |
|  | 750 | 150.6 | 159.2 | 175.0 | 0.86 | 0.91 |
| Z2-1 | 250 | 141.5 | 147.9 | 132.0 | 1.07 | 1.12 |
|  | 350 | 165.7 | 176.0 | 157.0 | 1.06 | 1.12 |
|  | 550 | 198.8 | 219.5 | 197.0 | 1.01 | 1.11 |
|  | 750 | 226.4 | 253.9 | 231.0 | 0.98 | 1.10 |
| Z2-2 | 250 | 130.5 | 130.9 | 120.0 | 1.09 | 1.09 |
|  | 350 | 149.3 | 154.7 | 143.0 | 1.04 | 1.08 |
|  | 550 | 180.6 | 191.8 | 182.0 | 0.99 | 1.05 |
|  | 750 | 205.8 | 221.0 | 215.0 | 0.96 | 1.03 |
| Z2-3 | 250 | 149.6 | 160.9 | 145.0 | 1.03 | 1.11 |
|  | 350 | 170.5 | 192.6 | 178.0 | 0.96 | 1.08 |
|  | 550 | 209.6 | 241.5 | 225.0 | 0.93 | 1.07 |
|  | 750 | 236.8 | 280.1 | 264.0 | 0.90 | 1.06 |
| Z5-1 | 350 | 200.1 | 200.9 | 178.0 | 1.12 | 1.13 |
|  | 550 | 244.9 | 252.7 | 229.0 | 1.07 | 1.10 |
|  | 750 | 279.5 | 293.5 | 261.0 | 1.07 | 1.12 |
| Z5-2 | 350 | 194.2 | 198.6 | 184.0 | 1.06 | 1.08 |
|  | 550 | 235.9 | 249.6 | 222.0 | 1.06 | 1.12 |
|  | 750 | 268.4 | 289.8 | 260.0 | 1.03 | 1.11 |
| Z5-3 | 350 | 202.6 | 204.0 | 196.0 | 1.03 | 1.04 |
|  | 550 | 251.3 | 256.9 | 233.0 | 1.08 | 1.10 |
|  | 750 | 287.4 | 298.6 | 269.0 | 1.07 | 1.11 |
| Z8-1 | 250 | 130.5 | 133.2 | 126.0 | 1.04 | 1.06 |
|  | 350 | 154.9 | 157.6 | 151.0 | 1.03 | 1.04 |
|  | 550 | 191.9 | 195.5 | 185.0 | 1.04 | 1.06 |
|  | 750 | 217.5 | 225.4 | 207.0 | 1.05 | 1.09 |
| Z8-2 | 250 | 131.8 | 140.7 | 129.0 | 1.02 | 1.09 |
|  | 350 | 155.0 | 167.0 | 151.0 | 1.03 | 1.11 |
|  | 550 | 192.4 | 207.7 | 186.0 | 1.03 | 1.12 |
|  | 750 | 220.3 | 239.9 | 206.0 | 1.07 | 1.16 |
| Z8-3 | 250 | 124.8 | 125.0 | 123.0 | 1.01 | 1.02 |
|  | 350 | 148.9 | 147.5 | 145.0 | 1.03 | 1.02 |
| Section | fy (MPa) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| Z8-3 | 550 | 181.1 | 182.5 | 186.0 | 0.97 | 0.98 |
|  | 750 | 206.1 | 210.1 | 208.0 | 0.99 | 1.01 |
| Z11-1 | 150 | 66.6 | 75.9 | 67.0 | 0.99 | 1.13 |
|  | 250 | 81.7 | 97.4 | 77.0 | 1.06 | 1.26 |
|  | 350 | 93.2 | 113.9 | 105.0 | 0.89 | 1.08 |
|  | 550 | 110.7 | 139.7 | 126.0 | 0.88 | 1.11 |
|  | 750 | 124.3 | 160.1 | 148.0 | 0.84 | 1.08 |
| Z11-2 | 150 | 71.4 | 80.7 | 62.0 | 1.15 | 1.30 |
|  | 250 | 87.1 | 104.0 | 86.0 | 1.01 | 1.21 |
|  | 350 | 99.5 | 121.9 | 102.0 | 0.98 | 1.19 |
|  | 550 | 118.7 | 149.7 | 113.0 | 1.05 | 1.32 |
|  | 750 | 133.6 | 171.8 | 131.0 | 1.02 | 1.31 |
| Z11-3 | 150 | 63.1 | 71.4 | 58.0 | 1.09 | 1.23 |
|  | 250 | 77.7 | 91.3 | 81.0 | 0.96 | 1.13 |
|  | 350 | 88.6 | 106.6 | 97.0 | 0.91 | 1.10 |
|  | 550 | 105.3 | 130.5 | 120.0 | 0.88 | 1.09 |
|  | 750 | 118.2 | 149.5 | 145.0 | 0.81 | 1.03 |
| Z12-1 | 150 | 60.5 | 69.6 | 58.0 | 1.04 | 1.20 |
|  | 250 | 74.2 | 88.9 | 72.0 | 1.03 | 1.23 |
|  | 350 | 84.4 | 103.7 | 84.0 | 1.00 | 1.23 |
|  | 550 | 100.0 | 126.8 | 117.0 | 0.85 | 1.08 |
|  | 750 | 112.2 | 145.2 | 137.0 | 0.82 | 1.06 |
| Z12-2 | 150 | 60.9 | 74.5 | 58.0 | 1.05 | 1.28 |
|  | 250 | 74.5 | 95.5 | 76.0 | 0.98 | 1.26 |
|  | 350 | 84.7 | 111.6 | 91.0 | 0.93 | 1.23 |
|  | 550 | 100.4 | 136.8 | 112.0 | 0.90 | 1.22 |
|  | 750 | 112.4 | 156.8 | 133.0 | 0.85 | 1.18 |
| Z12-3 | 150 | 53.6 | 64.0 | 59.0 | 0.91 | 1.08 |
|  | 250 | 65.5 | 81.4 | 76.0 | 0.86 | 1.07 |
|  | 350 | 74.3 | 94.8 | 91.0 | 0.82 | 1.04 |
|  | 550 | 87.9 | 115.7 | 116.0 | 0.76 | 1.00 |
|  | 750 | 98.4 | 132.3 | 135.0 | 0.73 | 0.98 |
| Z13-1 | 150 | 80.6 | 81.7 | 73.0 | 1.10 | 1.12 |
|  | 250 | 99.7 | 105.4 | 99.0 | 1.01 | 1.06 |
|  | 350 | 114.8 | 123.6 | 117.0 | 0.98 | 1.06 |
| Section | fy (MPa) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| Z13-1 | 550 | 138.5 | 151.9 | 144.0 | 0.96 | 1.05 |
|  | 750 | 157.6 | 174.3 | 169.0 | 0.93 | 1.03 |
| Z13-2 | 150 | 82.6 | 88.8 | 72.0 | 1.15 | 1.23 |
|  | 250 | 101.1 | 115.2 | 82.0 | 1.23 | 1.41 |
|  | 350 | 116.0 | 135.5 | 111.0 | 1.04 | 1.22 |
|  | 550 | 139.6 | 167.1 | 145.0 | 0.96 | 1.15 |
|  | 750 | 158.7 | 192.0 | 170.0 | 0.93 | 1.13 |
| Z13-3 | 150 | 74.0 | 75.2 | 84.0 | 0.88 | 0.89 |
|  | 250 | 91.6 | 96.4 | 100.0 | 0.92 | 0.96 |
|  | 350 | 105.3 | 112.7 | 115.0 | 0.92 | 0.98 |
|  | 550 | 127.0 | 138.2 | 155.0 | 0.82 | 0.89 |
|  | 750 | 144.3 | 158.4 | 185.0 | 0.78 | 0.86 |
| R11-1 | 250 | 148.4 | 147.1 | 148.0 | 1.00 | 0.99 |
|  | 350 | 177.3 | 175.0 | 170.0 | 1.04 | 1.03 |
|  | 550 | 216.9 | 218.3 | 193.0 | 1.12 | 1.13 |
|  | 750 | 247.7 | 252.4 | 219.0 | 1.13 | 1.15 |
|  | 1000 | 280.2 | 287.7 | 255.0 | 1.10 | 1.13 |
|  | 1200 | 302.7 | 312.2 | 282.0 | 1.07 | 1.11 |
| R11-2 | 250 | 149.3 | 149.6 | 155.0 | 0.96 | 0.97 |
|  | 350 | 180.8 | 178.2 | 162.0 | 1.12 | 1.10 |
|  | 550 | 222.1 | 222.5 | 200.0 | 1.11 | 1.11 |
|  | 750 | 253.1 | 257.3 | 229.0 | 1.11 | 1.12 |
|  | 1000 | 286.2 | 293.5 | 262.0 | 1.09 | 1.12 |
|  | 1200 | 309.0 | 318.6 | 288.0 | 1.07 | 1.11 |
| R11-3 | 250 | 140.4 | 144.5 | 143.0 | 0.98 | 1.01 |
|  | 350 | 166.8 | 171.8 | 166.0 | 1.00 | 1.03 |
|  | 550 | 202.7 | 214.0 | 192.0 | 1.06 | 1.11 |
|  | 750 | 230.5 | 247.3 | 221.0 | 1.04 | 1.12 |
|  | 1000 | 260.1 | 281.9 | 254.0 | 1.02 | 1.11 |
|  | 1200 | 280.7 | 305.8 | 277.0 | 1.01 | 1.10 |
| R13-1 | 250 | 118.5 | 126.1 | 109.0 | 1.09 | 1.16 |
|  | 350 | 136.5 | 148.9 | 115.0 | 1.19 | 1.29 |
|  | 550 | 165.2 | 184.2 | 155.0 | 1.07 | 1.19 |
|  | 750 | 187.8 | 212.2 | 182.0 | 1.03 | 1.17 |
|  | 1000 | 211.5 | 241.2 | 209.0 | 1.01 | 1.15 |
| Section | fy (MPa) | σu′ (MPa) | σu″ (MPa) | σu (MPa) | σu′/σu | σu″/σu |
| R13-1 | 1200 | 227.7 | 261.2 | 227.0 | 1.00 | 1.15 |
| R13-2 | 250 | 118.8 | 128.4 | 109.0 | 1.09 | 1.18 |
|  | 350 | 135.5 | 151.7 | 126.0 | 1.08 | 1.20 |
|  | 550 | 162.4 | 187.9 | 160.0 | 1.02 | 1.17 |
|  | 750 | 183.6 | 216.5 | 188.0 | 0.98 | 1.15 |
|  | 1000 | 205.1 | 246.1 | 217.0 | 0.95 | 1.13 |
|  | 1200 | 220.2 | 266.7 | 236.0 | 0.93 | 1.13 |
| R13-3 | 250 | 114.0 | 123.8 | 99.3 | 1.15 | 1.25 |
|  | 350 | 131.1 | 146.0 | 118.0 | 1.11 | 1.24 |
|  | 550 | 159.0 | 180.5 | 149.0 | 1.07 | 1.21 |
|  | 750 | 181.2 | 207.7 | 175.0 | 1.04 | 1.19 |
|  | 1000 | 204.1 | 236.1 | 201.0 | 1.02 | 1.17 |
|  | 1200 | 220.3 | 255.7 | 219.0 | 1.01 | 1.17 |

The mean and standard deviations of the ratio between the predicted and the real ultimate loads are tabulated in Table 10.5. It can be seen that when compared to the DSM, the proposed approach gives significantly better predictions of the ultimate load for local buckling of box sections, since it overcomes the deficiency of the DSM that the ultimate load is underestimated when one part of the section is extremely slender. Meanwhile, it gives similar predictions for local-distortional buckling of open sections. The ratio is also plotted against the slenderness λ in Fig. 10.4, offering a more vivid insight into the performance of the design approaches.

Table 10.5 Comparion between the design approaches

|  | Proposed approach |  | DSM |  |
| --- | --- | --- | --- | --- |
|  | Mean | St. Dev. | Mean | St. Dev. |
| Local buckling of box sections | 1.00 | 0.07 | 0.88 | 0.22 |
| Distortional buckling of open sections | 0.99 | 0.09 | 1.11 | 0.10 |

(a) Local buckling

(b) Distortional buckling

Fig. 10.4 Performance of the design approaches

# Summary and recommendations for future work

## Summary

This thesis presents a comprehensive study on modal decomposition and its applications in the analysis and design of cold-formed steel structures. The research consists of three parts, namely (1) the development of two new modal decomposition methods in the context of the FSM, (2) the application of these methods to the modal decomposition of post-buckling deformed shapes and the statistical analysis of geometric imperfections, and (3) the development of a design approach based on the direct strength method.

Two new modal decomposition methods, namely the polarisation method (PM) and the nodal force method (NFM), were derived, implemented in software, and validated against a variety of sections including both simple open sections and complex cross-sections with closed parts. In both methods, buckling modes are categorised into five types including local, distortional, global, transverse extension and shear. In the PM, buckling modes are identified according to the proportion of the plate bending strain energy to the total strain energy in the buckled shape. The buckling modes are sorted in an order from the highest proportion to the lowest. The different types of modes can then be identified based on the number of modes associated with each type, which is calculated prior to the modal decomposition process. The local modes are associated with the highest proportions of plate bending energy, while the transverse extension and shear modes are associated with the lowest. Since the transverse extension and shear modes have similar proportions of plate bending energy, the transverse extension modes are first filtered out using the criterion of null membrane transverse stress. This is done in advance, before the sorting process of the buckling modes. On the other hand, in the NFM, the buckling modes are generated by applying precisely defined nodal forces, obeying a definitive set of mechanical criteria for each of the buckling types. The criteria applied for modal identification include the criterion of non-zero longitudinal force to separate the shear modes from the other modes, the criterion of non-zero membrane transverse stress to identify the transverse extension modes, and the equilibrium of the cross-sectional forces and moments to distinguish the distortional modes from the global modes. In addition, orthogonality with respect to the global stiffness matrix K is consistently maintained while establishing the buckling modes.

A comparison was made between the newly proposed methods and the existing methods, namely the constrained finite strip method (cFSM) and generalised beam theory (GBT). The two new modal decomposition methods are distinct from the existing methods because of a number of features. First, they are able to deal with complex cross-sections without any increase in computational effort. Second, the Poisson’s effect and the shear effect are accounted for in the distortional and global modes, thus allowing the associated membrane transverse strain, the transverse curvature, the membrane shear strain and the non-linear warping displacements within each flat plate segment. Consequently, the problem of generating overly rigid distortional and global modal shapes, which incurs an up to 10% overestimation of the critical stress and is typical for the existing methods, is resolved. Third, all modes are consistently orthogonal to each other with respect to K. Finally, as a result of the second feature, the cross-sectional deformations of the modal shapes vary slightly with the half-wavelength due to the increased importance of the Poisson’s effect and the shear effect at short half-wavelengths. The critical stresses of various modes of various sections obtained from the different methods were compared. It was found that in general the results of different methods correlate well with each other, with exceptions at very short half-wavelengths, where the critical stresses of the newly proposed methods are significantly lower than those of the existing methods due to the Poisson’s effect and the shear effect.

In chapter 6 the NFM was applied to the analysis the post-buckling deformed shapes of cold-formed steel columns with various cross-sections. A limited number of orthogonal buckling modes were selected for the modal decomposition process. Box-sections undergoing local-local interactive buckling were initially studied, followed by lipped channel sections undergoing local-distortional interactive buckling. A methodology was developed to accommodate general end boundary conditions, including simple-simple, fixed-fixed and fixed-free conditions. Finally, the effect of geometric imperfections on the modal participations in the post-buckling stage was studied. It was found that for the box-sections, the column buckles initially in the primary local mode. Then as the load increases, the secondary local mode is also involved and a local-local interactive buckling occurs. The proportion between the modal participations of the two local modes in the interactive buckling is independent of the geometric imperfection. On the other hand, for the lipped channel sections, the column buckles initially in the local or distortional mode. Then as the load increases, the distortional or local mode is also involved and a local-distortional interactive buckling occurs. However, the proportion between the modal participations in the interactive buckling is dependent on the geometric imperfection.

In chapter 7 the NFM was applied to the statistical analysis of geometric imperfections, which would lead to a more scientific charactorisation of geometric imperfections. The imperfections of 88 specimens with plain and lipped channel sections were measured. First, the imperfect geometries of the plate and corner zones constituting the cross-section were measured independently using a frame with a laser transmitter/receiver which continuously sampled the distance to the faces of the plates. The obtained imperfect geometries were then assembled to form the 3D imperfect geometry of the specimen as a whole. This geometry was then decomposed into modes generated using the NFM, namely the first and second critical local and distortional modes, the flexural global modes and the torsional global mode. The imperfection magnitudes associated with these modes were plotted against the relative half-wavelength for all geometries. The mean and standard deviation values of the imperfection magnitudes as a function of the relative half-wavelength were obtained, which is useful in generating random imperfection magnitudes.

Finally, a design approach was developed based on the concept of the direct strength method (DSM). In the proposed design approach, the initial post-buckling stiffness is considered as an input parameter in addition to the critical load, which improves the current DSM in three aspects. First, since different types of buckling are associated with different post-buckling stiffness, it is possible to unify the design formulas. Second, in those cross-sections where one part of the section buckles far prior to the other parts, a relatively minor reduction of the stiffness is incurred, compared to the case where all parts of the section buckle simultaneously. Thus, the unduly conservative prediction of the ultimate load given by the current DSM in those cases can be avoided. Third, when interaction occurs between bucking modes (this being either a ‘true interaction’ due to the closeness of the critical stresses or interaction due to secondary bifurcation), the erosion of the ultimate load is reflected in a further reduction of the stiffness. Therefore, it is possible to cover both pure buckling and both types of interactive buckling using a unified design formula. The investigation of the design approach focused on pure local and distortional buckling, and local-distortional interactive buckling, although it is also applicable to pure global buckling. The development of the design approach followed three steps. First, the initial post-buckling stiffness of columns was calculated using the perturbation method, in which the columns are perfect and are made of a purely elastic material. Second, the initial post-buckling stiffness, along with the critical stresses, were used to establish a multi-linear model approximating the elastic post-buckling behaviour. Finally, the ultimate load was linked to the elastic post-buckling behaviour using empirical formulas.

## Recommendations for future work

(a)                                (b)                                (c)

Fig. 11.1 Out-of-plane displacements of plates in contact

One of the merits of the newly proposed modal decomposition methods over the existing methods is that it can deal with complicated sections without any increase in computational effort. In order to take full advantage of this, in addition to the sections already discussed in this thesis, the method could also be applied to built-up sections, which are becoming increasingly popular in the construction industry. In these sections, the components are assembled using bolted connections so that some plates are in contact with each other. An additional constraint can be added to the contacting plates with respect to the out-of-plane displacements, so that plates can either deform in the same direction or separate, as shown in Fig. 11.1 (a) and (b), but cannot violate contact as shown in (c). The implementation of this additional constraint in the FSM and further in the PM and NFM needs to be studied. After extending the application of the modal decomposition methods to built-up sections, research could proceed on analysing the post-buckling deformed shapes and geometric imperfections of such sections using the modal decomposition methods. It would also be interesting to compare the imperfection magnitudes before and after the assembly of the section.

As for the design approach, more work could be done aimed at improving the method and extending its application. First, currently the buckling modes are considered as either involved or absent, while their exact modal participations are not taken into account. If these modal participations were added to the input parameters, the stiffness of interactive buckling could be determined more accurately. Second, the methodology currently lacks a design approach for the cases where the cold-formed structural member buckles initially in a local or distortional mode, but fails in a local-global or distortional-global interactive mode due to secondary bifurcation. In a more complicated case, the member may fail in local-distortional-global interactive buckling. Due to the property of global buckling that it has no post-buckling residual load-carrying capacity, the structural member will fail almost instantly after global buckling emerges. Therefore, the proposed design approach for local and distortional buckling no longer works. Further work in this direction is required before a design approach covering all types of pure and interactive buckling can be proposed. Third, the effect of geometric imperfections on the ultimate load needs to be further studied, especially with respect to the sensitivity of the ultimate load to imperfections. It is already known that when elastic-plastic buckling occurs, the sensitivity is high, which suggests that the sensitivity may depend on the slenderness. In addition, it has been found that the sensitivity of the critical load to imperfections is inversely proportional to the initial post-buckling stiffness. For example, the critical load of local buckling, which has a positive initial post-buckling stiffness, is not quite dependent on imperfections. In contrast, the critical load of cylindrical shell buckling, which has a negative initial post-buckling stiffness, is very sensitive to imperfections. Whether this conclusion can be generally extended remains to be unveiled. Fourth, the application of the design approach could be extended to beams.

# REFERENCES

- ABAQUS. Hibbitt, Karlsson & Sorensen, Inc, Pawtucket, USA; 2007.

- Ádány S, Schafer BW. A full modal decomposition of thin-walled, single branched open cross-section members via the constrained finite strip method. Journal of Construction Steel Research. 2008; 64(1): 12-29.

- Ádány S, Schafer BW. Buckling mode decomposition of single-branched open cross-section members via finite strip method: Derivation. Thin-Walled Structures. 2006; 44(5): 563-584.

- Ádány S, Schafer BW. Buckling mode decomposition of single-branched open cross-section members via finite strip method: Application and examples. Thin-Walled Structures. 2006; 44(5): 585-600.

- Ádány S, Schafer BW. Generalized constrained finite strip method for thin-walled members with arbitrary cross-section: Primary modes. Thin-Walled Structures. 2014; 84: 150-169.

- Ádány S, Schafer BW. Generalized constrained finite strip method for thin-walled members with arbitrary cross-section: Secondary modes, orthogonality, examples. Thin-Walled Structures. 2014; 84: 123-133.

- Ádány S, Silvestre N, Schafer BW, Camotim D. GBT and cFSM: two modal approaches to the buckling analysis of unbranched thin-walled members. Advanced steel construction. 2009; 5(2): 195-223.

- Ádány S. Buckling mode classification of members with open thin-walled cross-sections by using the Finite Strip Method, Research report, Johns Hopkins University; 2004.

- Ádány S. Decomposition of in-plane shear in thin-walled members. Thin-Walled Structures. 2013; 73: 27-38.

- Adetoro SA. An experimental investigation of the behaviour of thin-walled lipped channel compression members, Cranfield Institute of Technology, College of Aeronautics, CMCIT Thesis, 1988.

- Bebiano R, Camotim D, and Goncalves R. GBTUL 2.0 – A New/Improved Version of the GBT Based Code for Buckling Analysis of Cold-Formed Steel Members. In: Proceedings 22nd International Specialty Conference on Cold-Formed Steel Design and Construction (Eds. LaBoube and Yu); Nov 2014; St Louis, Missouri, USA: 1-19.

- Becque J. A new approach to modal decomposition of buckled shapes. Structures. 2015; 4: 2-12.

- Becque J. Local-overall interaction buckling of inelastic columns: A numerical study of the inelastic Van der Neut column. Thin-Walled Structures. 2014; 81: 101-107.

- Becque J. The Interaction of Local and Overall Buckling of Cold-Formed Stainless Steel Columns [PhD thesis]. Sydney, Australia: The University of Sydney - School of Civil Engineering; 2008.

- Benito R, Sridharan S. Interactive buckling analysis with finite strips. International Journal for Numerical Methods in Engineering. 1985; 21: 145-161.

- Bernard ES. Flexural Behavior of Cold-Formed Profiled Steel Decking [PhD thesis]. Sydney, Australia: University of Sydney; 1993.

- Bleich F. Buckling Strength of Metal Structures. New York: McGraw-Hill; 1952.

- Bryan GH. On the Stability of Plane Plate under Thrust in its Own Plane with Applications on the Buckling of the Sides of a Ship. Proceedings of the London Mathematical Society. 1891; 22(1): 54.

- Chai H Yoo, Sung Lee. Stability of Structures: Principles and Applications. Oxford: Butterworth-Heinemann; 2011.

- Charnvarnichborikarn P. Distortional buckling of cold-formed steel Z-sections [PhD thesis]. Manitoba, Canada: University of Manitoba - Department of Civil Engineering; 1992.

- Cheung YK. Finite strip method in structural analysis. Oxford: Pergamon; 1976.

- Dat DT, Pekoz TP. The Strength of Cold-Formed Steel Columns. Department of Structural Engineering Report, Cornell University, No. 80-4, Ithaca, New York; 1980.

- Davies JM, Leach P. First-Order Generalised Beam Theory. Journal of Constructional Steel Research. 1994; 31: 187-220.

- Desmond TP. The Behavior and Strength of Thin-Walled Compression Elements with Longitudinal Stiffeners [PhD thesis]. Ithaca, New York: Cornell University; 1977.

- Dinis PB, Camotim D. Cold-formed steel column undergoing local-distortional coupling: Behaviour and direct strength prediction against interactive failure. Computers and Structures. 2015; 147: 181-208.

- Dinis PB, Young B, Camotim D. Local–distortional interaction in cold-formed steel rack-section columns. Thin-Walled Structures. 2014; 81: 185–194.

- Dowson RG, Walker AC. Post buckling of geometrically imperfect plates. Journal of Structural engineering. 1972; 98(1): 75-94.

- Föppl A. Vorlesungen über technische Mechanik. Leipzig: B.G. Teubner; 1907.

- Gonçalves R, Camotim D. GBT Local and global analysis of aluminium and stainless steel columns. Computers and Structures. 2004; 82: 1473-1484.

- Gonçalves R, Camotim D. Thin-walled member plastic bifurcation analysis using generalized beam theory. Advances in Engineering Software. 2007; 38(8-9): 637-646.

- Gonçalves R, Ritto Corrêa M, Camotim D. A new approach to the calculation of cross-section deformation modes in the framework of Generalized Beam Theory. Computational Mechanics. 2010; 46(5): 759-781.

- Goodier JN. The Buckling of Compressed Bars by Torsion and Flexure.  Cornell Univ. Eng. Expt. Sta. Bull. No. 27 (Sec. 6.0); 1941.

- Graves Smith TR, Sridharan S. A finite strip method for the post-locally-buckled analysis of plate structures. International Journal of mechanical sciences. 1978; 20: 833-842.

- Graves Smith TR. The ultimate strength of locally buckled columns of arbitrary length. Thin-walled Structures. London: Crosby Lockwood; 1969. 35.

- Hancock GJ, Kwon YB, Bernard ES. Strength design curves for thin-walled sections undergoing distortional buckling. Journal of Constructional Steel Research. 1994; 31(2-3): 169-186.

- Hancock GJ. Non-linear analysis of thin sections in compression. Journal of Structural Division. 1981; 107 (3): 455-71.

- Hancock GJ. The influence of sectional stability in engineering design of thin-walled members. In: proceedings of the third Australasian Congress on Applied Mechanics; 2002 February 20-22; Sydney, Australia.

- Hunt GW, Simões da Silva L, Manzocchi GME. Interactive Buckling in Sandwich Structures. In: Proceedings of the Royal Society of London. Series A, Mathematical and Physical Sciences; 1988 May 9; 417: 155-177.

- Ingvarsson L. Cold-Forming Residual Stresses and Box Columns Built Up By Two Cold-Formed Channel Sections Welded Together. Bulletin of The Department of Building Statics and Structural Engineering, 121, The Royal Institute of Technology, Stockholm, Sweden; 1977.

- Kappus R. Drillknicken zentrisch gedrückter Stäbe mit offenem Profile im elastischen Bereich. Luftfahrt-Forschung 1937; 14(9): 444-457. English translation, Twisting Failure of Centrally Loaded Open-Sections Columns in the Elastic Range, NACA Tech. Mem. 851; 1938.

- Koiter WT. Over de stabiliteit van hetelastisch evenwicht (in Dutch), Phi-) thesis. HJ Paris, Amsterdam; 1945. English translation as NASA TT F- 10, 833 (1967) and AFFDL Report TR 70-25; 1970.

- Kutanova N. Cross-sectional instability of aluminium extrusions with complex cross-sectional shapes [PhD thesis]. Eindhoven University of Technology; 2009.

- Kwon YB, Hancock GJ. Strength tests of cold-formed channel sections undergoing local and distortional buckling. Journal of Structural Engineering. 1992; 118(7):1786-1803.

- Kwon YB, Kim BS, Hancock GJ. Compression tests of high strength cold-formed steel channels with buckling interaction. Journal of Constructional Steel Research. 2009; 65: 278-289.

- Lau SCW, Hancock GJ. Distortional Buckling Formulas for Channel Columns. Journal of Structural Engineering. 1987; 113(5): 1063 – 1078.

- Lau SCW, Sammy CW, Hancock GJ. Distortional buckling tests of cold-formed channel sections. In: Proceedings of the Ninth International Specialty Conference on Cold-Formed Steel Structures; 1988 November 8-9; St Louis, Missouri, USA.

- Li Z, Ádány S, Schafer BW. Modal identification for shell finite element models of thin-walled members in non-linear collapse analysis. Thin-walled structures. 2013; 67: 15-24.

- Li Z, Schafer BW. Buckling analysis of cold-formed steel members with general boundary conditions using CUFSM: conventional and constrained finite strip methods. In: Proceedings of the Twentieth International Specialty Conference on Cold-Formed Steel Structures; 2010 November 3-4; St Louis, Missouri, USA.

- Loughlan J, Yidris N, Jones K. The failure of thin-walled lipped channel compression members due to coupled local-distortional interaction and material yielding. Thin-Walled Structures. 2012; 61: 14–21.

- Loughlan J. Mode interaction in lipped channel columns under concentric or eccentric loading [PhD thesis]. Glasgow, Scotland: University of Strathclyde; 1979.

- Martins AD, Camotim D, Dinis PB, Young B. Local-distortional interaction in cold-formed steel columns: mechanics, testing, numerical simulation and design. Structures. 2015: 38-57.

- Martins AD, Dinis PB, Camotim D, Providência P. On the relevance of local-distortional interaction effects in the behaviour and design of cold-formed steel columns. Computers and Structures. 2015; 160: 57-89.

- Mennink J. Cross-sectional stability of aluminium extrusion [PhD thesis]. Eindhoven University of Technology; 2002.

- Mulligan GP, Peköz T. Locally buckled thin-walled columns. Journal of Structural Engineering. 1984; 110(11): 2635-2654.

- Mulligan GP. The Influence of Local Buckling on the Structural Behavior of Singly Symmetric Cold-Formed Steel Columns [PhD thesis]. Ithaca, New York: Cornell University; 1983.

- Nakamura T, Uetani K. The secondary buckling and post-secondary-buckling behaviours of rectangular plates. International Journal of Mechanical Sciences. 1979; 21: 265-286.

- Nedelcu M. GBT-based buckling mode decomposition from finite element analysis of

- Ovesy HR, Loughlan J, Assaee H. The compressive post-local bucking behaviour of thin plates using a semi-energy finite strip approach. Thin-walled structures. 2004; 42: 449-474.

- Peterman KD, Schafer BW. Sheathing Braced Design of Wall Studs. Final report to the American Iron and Steel Institute, Washington DC; 2013 Feb.

- Pignataro M, Luongo A, Rizzi N. On the effect of the local overall interaction on the postbuckling of uniformly compressed channels. Thin-walled Structures. 1985; 3: 293-321.

- Rasmussen KJR. Bifurcation of locally buckled members. Thin-Walled Structures. 1997; 28(2): 117-154.

- Rhodes J, Harvey JM. Interaction behaviour of plain channel columns under concentric or eccentric loading. In: ECCS. Proceedings of the 2nd International Colloquium on the Stability of Steel Structures; 1977; Liège.

- Rhodes J, Harvey JM. Plain channel section struts in compression and bending beyond the local buckling load. International Journal of Mechanical Sciences. 1976; 8: 511-519.

- Rhodes J, Harvey JM. Plates in uniaxial compression with various support conditions at the unloaded boundaries. International Journal of Mechanical Sciences. 1971; 13: 787-802.

- Rhodes J, Loughlan J. Simple design analysis of lipped channel columns. In: Proceedings of the Fifth International Specialty Conference on Cold-Formed Steel Structures; 1980 November 18-19; St. Louis, Missouri, USA.

- Saint-Venant M. Discussion en Théorie de l’Elasticité des Corps Solides, by Clebsch 1883; 704.

- Santos ES, Batista EM, Camotim D. Experimental investigation concerning lipped channel columns undergoing local-distortional-global mode interaction. Thin-Walled Structures. 2012; 54: 19-34.

- Schafer BW, Ádány S. Understanding and classifying local, distortional and global buckling in open thin-walled members. In: Proceedings of Annual Conference of Structural Stability Research Council; 2005; Montreal, Canada.

- Schafer BW, Hancock G. A history of distortional buckling of cold-formed steel columns. Available from: http://www.ce.jhu.edu/bschafer/dist_columns/appendix-a.pdf [Accessed 16th Nov 2016]

- Schafer BW, Peköz T. Computational modelling of cold-formed steel: characterizing geometric imperfections and residual stresses. Journal of Constructional Steel Research. 1998; 47: 193-210.

- Schafer BW, Peköz T. Direct strength prediction of cold-formed steel members using numerical elastic buckling solutions. In: Proceedings of the fourteenth international specialty conference on cold-formed steel structures; 1998; University of Missouri-Rolla.

- Schafer BW. Distortional Buckling of Cold-Formed Steel Columns. Final Report to the American Iron and Steel Institute, Washington DC; 2000.

- Schafer BW. Review: the direct strength method of cold-formed steel member design. Journal of constructional steel research. 2008; 64: 766-778.

- Schafer BW. Local, distortional and Euler buckling in thin-walled columns. Journal of Structural Engineering. 2002; 128: 289-99.

- Schardt R. Generalised Beam Theory−an adequate method for coupled stability problems. Thin-Walled Structures. 1994; 19(2-4): 161-180.

- Schardt R. Verallgemeinierte Technische Biegetheorie (generalised beam theory). Berlin: Springer; 1989.

- Sean LK, Rhodes J. Simplified buckling analysis of plate with compound edge stiffners. Journal of Engeering Mechanics. 1993; 119(1): 19-38.

- Shanley F. Inelastic column theory. Journal of Aeronautical Sciences. 1957; 14(5): 261-268.

- Shifferaw Y, Viera Jr, LCM, Schafer BW. Compression testing of coldformed steel columns with different sheathing configurations. In: Proceedings of Annual Stability Conference, Structural Stability Research Council; 2010: 593-612.

- Silverstre N, Camotim D, Dinis PB. Post-buckling behaviour and direct strength design of lipped channel columns experiencing local/distortional interaction. Journal of Constructional Steel Research. 2012; 73: 12-30.

- Silverstre N, Camotim D. Local-Plate and Distortional Postbuckling Behavior of Cold-Formed Steel Lipped Channel Columns with Intermediate Stiffeners. Journal of Structural Engineering. 2006; 132(4): 529.

- Silvestre N, Camotim D, Dinis PB. Post-buckling behaviour and direct strength design of lipped channel column experiencing local/distortional interaction. Journal of Constructional Steel Research. 2012; 73: 12-30.

- Silvestre N. Generalized beam theory to analyse the buckling behaviour of circular cylindrical shells and tubes. Thin-Walled Structures. 2007; 45(2): 185-198.

- Simão P, Simões da Silva L. Post-buckling bifurcation analysis of thin-walled prismatic members in the context of the generalised beam theory [PhD thesis]. Coimbra, Portugal:  University of Coimbra - Department of Civil Engineering; 2007.

- Sridharan S, Graves-smith TR. Postbuckling analysis with finite strips. Journal of the Engineering Mechanics Division ASCE. 1981; 107: 869-888.

- Sridharan S. A finite strip analysis of locally buckled plate structures subject to nonuniform compression. Engineering structures. 1982; 4: 249-255.

- Sridharan S. A semi-analytical method for the post-local-torsional buckling analysis of prismatic plate structures. International Journal for numerical methods in engineering. 1982; 18: 1685-1697.

- Supple WJ. Changes of wave-form of plated in the post-buckling range. International Journal of Solids Structures. 1970; 6: 1243-1258.

- Thomasson PO. Thin-walled C-shaped panels in axial compression. Stockholm, Sweden: Swedish Council for Building Research; 1978.

- Timoshenko SP, Gere JM. Theory of elastic stability. New York: McGraw-Hill; 1961.

- Timoshenko SP. Einige Stabilitätsprobleme der Elastizitätstheorie. Zeitschrift für Mathematik und Physik. 1910; 58: 337.

- Uemura M, O-Il Byon. Secondary buckling of a flat plate under uniaxial compression part 1: theoretical analysis of simply supported flat plate. International Journal of Non-linear Mechanics. 1977; 12: 355-370.

- Uemura M, O-Il Byon. Secondary buckling of a flat plate under uniaxial compression part 2: analysis of clamped plate by F.E.M. and comparison with experiments. International Journal of Non-linear Mechanics. 1978; 13: 1-14.

- Van der Neut A. The Interaction of Local Buckling and Column Failure of Thin-walled Compression Members. In: Hetenyi M, Vincenti WG, editors. Proceedings of the Twelfth International Congress of Applied Mechanics; 1968 Aug 26-31; Stanford, CA, USA. Berlin: Springer-Verlag, New York: Heidelberg; 1969: 389-399.

- Von Kármán T. Festigkeitsprobleme im Maschinenbau.  Encyk. der Math. Wiss. 1910; 4(4): 349.

- Wagner H, Pretschner W. Torsion and buckling of open sections. NACA Tech. Mem. 784; 1936.

- Wagner H. Verdrehung und Knickung von offenen Profilen. 25th Anniversary Publication, Technische Hochschule, Danzig, Translated in NACA Tech. Mem. 807; 1936.

- Wai-Fah Chen, Toshio Atsuta. Theory of Beam-Columns, Volume 2: Space Behavior and Design. New York: McGraw-Hill; 1977.

- Walker AC. The Post-Buckling Behaviour of Simply Supported Square Plates. Aeronautical Quarterly. 1969; 20: 203-222.

- Winter G. Commentary on the 1968 Edition of the Specification for the Design of Cold-Formed Steel Structural Members. New York: American Iron and Steel Institute; 1970.

- Winter G. Performance of thin steel compression flanges. Preliminary publication, presented at the 3rd Congress of The International Association for Bridge and Structural Engineering. Liege; 1948.

- Yamaki N. Post-Buckling Behaviour of Rectangular Plates with Small Initial Curvature Loaded in Edge Compression. Journal of Applied Mechanics. 1959; 26: 407-414.

- Yang D, Hancock GJ. Compression tests of high strength steel channel columns with interaction between local and distortional buckling. Journal of Structural Engineering. 2004; 130(12): 1954-1963.

- Ye J, Hajirasouliha I, Becque J, Pilakoutas K. Experimental investigation of local-flexural interactive buckling of CFS channel columns. In: Proceedings of the 7th International Specialty Conference on Coupled Instabilities in Metal Structures; 2016 November 7-8; Baltimore, Maryland, USA.

- Young B, Rasmussen KJR. Bifurcation of singly symmetric columns. Thin-Walled Structures. 1997; 28(2): 155-177.

- Young B, Rasmussen KJR. Tests of cold-formed channel columns. In: Proceedings of the Fourteenth International Specialty Conference on Cold-Formed Steel Structures; 1998 October 15-16; St. Louis, Missouri, USA.

- Young B, Silvestre N, Camotim D. Cold-formed steel lipped channel columns influenced by local-distortional interaction: strength and DSM design. Journal of Structural Engineering. 2013; 139(6): 1059-1074.

- Young B, Yan J. Design of cold-formed steel channel columns with complex edge stiffeners by direct strength method. Journal of Structural Engineering. 2004; 130(11): 1756-1763.

- Young B. The behavior and Design of cold-formed channel columns [PhD thesis]. Sydney, Australia: University of Sydney; 1997.

- Zeinoddini V, Schafer BW. Simulation of geometric imperfections in cold-formed steel members using spectral representation approach. Thin-Walled Structures. 2012: 60: 105-117.

- Zeinoddini V. Geometric Imperfections in Cold-Formed Steel Members [PhD thesis]. Baltimore, Maryland: John Hopkins University; 2011.

# APPENDIX A. Python Script generating FEM buckling modes

from numpy import *

from abaqus import *

from abaqusConstants import *

import sketch

import part

import regionToolset

################### Input #####################

web=117.39

flange=87.49

lip=3.72

t=1.085

# full length

L_t = 400

# mesh size

mesh_size=10

#

Eel=210000

v=0.3

# number of half-waves

num_hw=2

# mode select

Type='G'

mode_sel=1

################# Trans Input #################

web=float(web)

flange=float(flange)

lip=float(lip)

t=float(t)

L_t=float(L_t)

Eel=float(Eel)

v=float(v)

L=L_t/num_hw

################# Functions ###################

## find null space

def nullspace(A, atol=1e-13, rtol=0):

A = atleast_2d(A)

u, s, vh = linalg.svd(A)

tol = max(atol, rtol * s[0])

nnz = (s >= tol).sum()

ns = vh[nnz:].conj().T

return ns

## create model in abaqus part 1

def  Model_1():

global Coor,myModel,new_name

# Create the model

new_name='mode '+Type+str(mode_sel)+'_'+str(num_hw)

mdb.models.changeKey(fromName='Model-1', toName=new_name)

myModel=mdb.models[new_name]

# Create the part

mySketch=myModel.ConstrainedSketch(name='Column Profile', sheetSize=100)

mySketch.Line(point1=(-web/2+lip,0), point2=(-web/2,0))

mySketch.Line(point1=(-web/2,0), point2=(-web/2,flange))

mySketch.Line(point1=(-web/2,flange), point2=(web/2,flange))

mySketch.Line(point1=(web/2,flange), point2=(web/2,0))

mySketch.Line(point1=(web/2,0), point2=(web/2-lip,0))

myPart = myModel.Part(name='Column', dimensionality=THREE_D, type=DEFORMABLE_BODY)

myPart.BaseShellExtrude(sketch=mySketch, depth=L_t)

# Material

material=myModel.Material(name='steel')

material.Elastic(table=((Eel,v), ))

# Section

# create

myModel.HomogeneousShellSection(name='section', material='steel', thickness=t)

# assignment

pickregion = regionToolset.Region(faces=myPart.faces)

myPart.SectionAssignment(region=pickregion, sectionName='section')

# Assembly

import assembly

myAssembly=myModel.rootAssembly

myInstance=myAssembly.Instance(name='Column Instance',part=myPart,dependent=ON)

# Step

myModel.StaticStep(name='Step-1', previous='Initial', nlgeom=OFF)

# Boundary conditions

myPart.Set(edges=myPart.edges.findAt(((-web/2+lip/2,0,L_t),), ((-web/2,flange/2,L_t),),((-web/4,flange,L_t),),((web/4,flange,L_t),),((web/2,flange/2,L_t),),((web/2-lip/2,0,L_t),)),name='end')

region1=myInstance.sets['end']

myModel.DisplacementBC(name='end',

createStepName='Initial', region=region1, u1=SET, u2=SET, u3=UNSET,

ur1=UNSET, ur2=UNSET, ur3=SET, amplitude=UNSET, fixed=OFF,

distributionType=UNIFORM, fieldName='', localCsys=None)

myPart.Set(edges=myPart.edges.findAt(((-web/2+lip/2,0,0),), ((-web/2,flange/2,0),),((-web/4,flange,0),),((web/4,flange,0),),((web/2,flange/2,0),),((web/2-lip/2,0,0),)),name='start')

region2=myInstance.sets['start']

myModel.DisplacementBC(name='start',

createStepName='Initial', region=region2, u1=SET, u2=SET, u3=UNSET,

ur1=UNSET, ur2=UNSET, ur3=SET, amplitude=UNSET, fixed=OFF,

distributionType=UNIFORM, fieldName='', localCsys=None)

myPart.Set(vertices=myPart.vertices.findAt(((web/2-lip,0,0),)),name='node')

region3=myInstance.sets['node']

myModel.DisplacementBC(name='node',

createStepName='Initial', region=region3, u1=UNSET, u2=UNSET, u3=SET,

ur1=UNSET, ur2=UNSET, ur3=UNSET, amplitude=UNSET, fixed=OFF,

distributionType=UNIFORM, fieldName='', localCsys=None)

# Mesh

import mesh

elemType1 = mesh.ElemType(elemCode=S4R, elemLibrary=STANDARD)

myFaces= myPart.faces

pickregion = regionToolset.Region(faces=myFaces)

myPart.setElementType(regions=pickregion, elemTypes=(elemType1,))

myPart.seedPart(size=mesh_size, deviationFactor=0.1, minSizeFactor=0.1)

myPart.generateMesh()

# Node coordinates

Coor=[]

for i in myPart.nodes:

coor=i.coordinates

Coor.append(coor)

Coor=array(Coor)

## solve generalised eigenvalue problem

def geneigsympos(A,B):

#fixme: input checks on the matrices

LI = asmatrix(linalg.cholesky(B)).I

C = LI * asmatrix(A) * LI.T

evals, evecs = linalg.eigh(C)

if type(A) == type(asarray(A)): output = "array"

# A was passed as numpy-array

else: output = "matrix"

#but the evecs need to be transformed back:

evecs = LI.T * asmatrix(evecs)

if output == "array": return evals, asarray(evecs)

else: return asmatrix(evals), evecs

## discretisation

def disc():

global c

#

c=[]

L_set=round(float(Coor[0][2]),3)

for i in range(0,len(Coor)):

for j in range(0,3):

Coor[i][j]=round(float(Coor[i][j]),3)

if float(Coor[i][2])==L_set:

c.append([Coor[i][0],Coor[i][1],0])

c=array(c)

#

center=[(min(c[:,0])+max(c[:,0]))/2,(min(c[:,1])+max(c[:,1]))/2]

angle=zeros(len(c))

for i in range(0,len(c)):

dist=sqrt((c[i][0]-center[0])**2+(c[i][1]-center[1])**2)

sin=(c[i][1]-center[1])/dist

cos=(c[i][0]-center[0])/dist

if cos>=0:

angle[i]=arcsin(sin)

else:

angle[i]=pi-arcsin(sin)

c=c[argsort(angle)]

# node type

# 0 sub

# 1 internal main

# 2 external main

c[0][2]=2

c[len(c)-1][2]=2

for i in range(0,len(c)):

if abs(c[i,0])==max(c[:,0]) and abs(c[i,1]-center[1])==max(c[:,1])/2:

c[i][2]=1

return c

## matrices

def Matrices():

# initialisation

K=zeros([4*len(c),4*len(c)])

G=zeros([4*len(c),4*len(c)])

#

for i in range(0,len(c)-1):

# initialisation

K11=zeros([4,4])

K22=zeros([4,4])

# Calculating width of element, bel

bel=sqrt((c[i+1][0]-c[i][0])**2+(c[i+1][1]-c[i][1])**2)

# Calculating K11

S11=1/(1-v**2)*(t**3/12)*Eel*(pi**4*bel/L**3)

S12=1/(1-v**2)*(t**3/12)*Eel*v*(pi**2/(bel*L))

S21=S12

S22=1/(1-v**2)*(t**3/12)*Eel*(L/bel**3)

S33=(t**3/12)*Eel/2/(1+v)*(pi**2/(bel*L))

K11[0][0]=S11/2

K11[0][1]=S11/4

K11[0][2]=S11/6-S12

K11[0][3]=S11/8-3*S12/2

K11[1][0]=S11/4

K11[1][1]=S11/6+2*S33

K11[1][2]=S11/8-S12/2+2*S33

K11[1][3]=S11/10-S12+2*S33

K11[2][0]=S11/6-S21

K11[2][1]=S11/8-S21/2+2*S33

K11[2][2]=S11/10-S21/3-S12/3+2*S22+8*S33/3

K11[2][3]=S11/12-S21/4-3*S12/4+3*S22+3*S33

K11[3][0]=S11/8-3*S21/2

K11[3][1]=S11/10-S21+2*S33

K11[3][2]=S11/12-3*S21/4-S12/4+3*S22+3*S33

K11[3][3]=S11/14-3*S21/5-3*S12/5+6*S22+18*S33/5

# Calculating K22

F11=1/(1-v**2)*Eel*(pi**2*bel*t/L)

F12=v/(1-v**2)*Eel*pi*t

F21=F12

F22=1/(1-v**2)*Eel*(L*t/bel)

F33=Eel/2/(1+v)*(pi**2*bel*t/L)

K22[0][0]=F33/2

K22[0][1]=F33/4

K22[0][2]=0

K22[0][3]=F33*L/(2*bel*pi)

K22[1][0]=F33/4

K22[1][1]=F22/2+F33/6

K22[1][2]=-F21/2

K22[1][3]=F33*L/(4*bel*pi)-F21/4

K22[2][0]=0

K22[2][1]=-F12/2

K22[2][2]=F11/2

K22[2][3]=F11/4

K22[3][0]=F33*L/(2*bel*pi)

K22[3][1]=F33*L/(4*bel*pi)-F12/4

K22[3][2]=F11/4

K22[3][3]=F11/6+F33*L**2/(2*bel**2*pi**2)

# Calculating K element matrix

C=matrix([[1,0,0,0,0,0,0,0],[0,bel,0,0,0,0,0,0],[-3,-2*bel,3,-bel,0,0,0,0],[2,bel,-2,bel,0,0,0,0],[0,0,0,0,1,0,0,0],[0,0,0,0,-1,0,1,0],[0,0,0,0,0,1,0,0],[0,0,0,0,0,-1,0,1]])

R=matrix([[0,-(c[i+1][1]-c[i][1])/bel,(c[i+1][0]-c[i][0])/bel,0,0,0,0,0],[0,0,0,1,0,0,0,0],[0,0,0,0,0,-(c[i+1][1]-c[i][1])/bel,(c[i+1][0]-c[i][0])/bel,0],[0,0,0,0,0,0,0,1],[0,(c[i+1][0]-c[i][0])/bel,(c[i+1][1]-c[i][1])/bel,0,0,0,0,0],[1,0,0,0,0,0,0,0],[0,0,0,0,0,(c[i+1][0]-c[i][0])/bel,(c[i+1][1]-c[i][1])/bel,0],[0,0,0,0,1,0,0,0]])

Kel_core=zeros([8,8])

Kel_core[0:4,0:4]=K11

Kel_core[4:8,4:8]=K22

Kel=R.T*C.T*Kel_core*C*R

# Calculating G element matrix

H=(bel*t*pi**2/(2*L))*matrix([[1.0/1,1.0/2,1.0/3,1.0/4,0,0,0,0],[1.0/2,1.0/3,1.0/4,1.0/5,0,0,0,0],[1.0/3,1.0/4,1.0/5,1.0/6,0,0,0,0],[1.0/4,1.0/5,1.0/6,1.0/7,0,0,0,0],[0,0,0,0,1.0/1,1.0/2,0,0],[0,0,0,0,1.0/2,1.0/3,0,0],[0,0,0,0,0,0,1.0/1,1.0/2],[0,0,0,0,0,0,1.0/2,1.0/3]])

Gel=R.T*C.T*H*C*R

# Assembling K Global matrix

K[(4*i):(4*i+8),(4*i):(4*i+8)]+=Kel

# Assembling K Global matrix

G[(4*i):(4*i+8),(4*i):(4*i+8)]+=Gel

return K,G

## modal decomposition

def MD():

#

row=len(c)

NDF=4*row

# labels of external main and sub nodes

EndSubNode=[]

for i in range(0,row):

if c[i][2]==0 or c[i][2]==2:

EndSubNode.append(i)

EndSubNode=array(EndSubNode)

# number of local modes

num_sub_node=0

num_in_main_node=0

num_ex_main_node=0

for i in range(0,row):

if c[i][2]==0:

num_sub_node+=1

elif c[i][2]==1:

num_in_main_node+=1

elif c[i][2]==2:

num_ex_main_node+=1

num_local=2*(num_sub_node+num_ex_main_node)+num_in_main_node

# plate angles

CosSin=[]

for i in range(0,len(EndSubNode)):

if i==len(EndSubNode)-1:

node=EndSubNode[i]-1

else:

node=EndSubNode[i]

bel=sqrt((c[node+1][0]-c[node][0])**2+(c[node+1][1]-c[node][1])**2)

Cos=(c[node+1][0]-c[node][0])/bel

Sin=(c[node+1][1]-c[node][1])/bel

CosSin.append([Cos,Sin])

#

K,G=Matrices()

# local modes

DOFl=concatenate([arange(4,NDF+1,4),4*EndSubNode+2,4*EndSubNode+3])

NumDOFl=len(DOFl)

DOFk=concatenate([DOFl,arange(1,NDF+1,4)])

NumDOFk=len(DOFk)

Rk=zeros([NumDOFk,num_local])

Rk[0:row,0:row]=eye(row)

for i in range(0,len(EndSubNode)):

Rk[NDF/4+i][NDF/4+i]=-CosSin[i][1]

Rk[NDF/4+len(EndSubNode)+i][NDF/4+i]=CosSin[i][0]

Kkk=K[DOFk-1,:][:,DOFk-1]

Vk=linalg.solve(Kkk,Rk)

Vl=zeros([NDF,num_local])

Vl[DOFk-1,:]=Vk

# sort local

Vl=matrix(Vl)

Sl,Al=geneigsympos(array(Vl.T*K*Vl),array(Vl.T*G*Vl))

Vl=Vl*Al

# assumptions

# null warping nodal forces

Trans=eye(NDF)

Trans=delete(Trans,arange(1,NDF+1,4)-1,1)

# null transverse stress

Cond1=zeros([row-1,NDF])

for i in range(0,row-1):

b=sqrt((c[i+1][0]-c[i][0])**2+(c[i+1][1]-c[i][1])**2)

Cos=(c[i+1][0]-c[i][0])/b

Sin=(c[i+1][1]-c[i][1])/b

Cond1[i][4*i+1]=Cos/b

Cond1[i][4*i+2]=Sin/b

Cond1[i][4*(i+1)+1]=-Cos/b

Cond1[i][4*(i+1)+2]=-Sin/b

Cond1[i][4*i]=pi/L/2*v

Cond1[i][4*(i+1)]=pi/L/2*v

Cond1=matrix(Cond1)

Cond1=Cond1*linalg.inv(K)

# equilibrium of nodal forces

Cond2=zeros([3,NDF])

Cond2[0,arange(2,NDF+1,4)-1]=1

Cond2[1,arange(3,NDF+1,4)-1]=1

for i in range(0,NDF/4):

Cond2[2][4*i+1]=-c[i][1]

Cond2[2][4*i+2]=c[i][0]

# distortional modes

Vd=linalg.solve(K,Trans*nullspace(concatenate([Cond1,Cond2,Vl.T],axis=0)*Trans))

# sort distortional

Vd=matrix(Vd)

Sd,Ad=geneigsympos(array(Vd.T*K*Vd),array(Vd.T*G*Vd))

Vd=Vd*Ad

# global modes

Vg=linalg.solve(K,Trans*nullspace(concatenate([Cond1,Vl.T,Vd.T],axis=0)*Trans))

# sort global

Vg=matrix(Vg)

Sg,Ag=geneigsympos(array(Vg.T*K*Vg),array(Vg.T*G*Vg))

Vg=Vg*Ag

# select modal shape

if Type=='L':

V_type=Vl

elif Type=='D':

V_type=Vd

elif Type=='G':

V_type=Vg

V=V_type[:,mode_sel-1]

# normalisation

V=V/sqrt(V.T*K*V)

# nodal force

F=K*V

return F

## match FE and FS nodes

def MN():

global POS

POS=zeros([len(Coor),2])

for i in range(0,len(Coor)):

x_match=[j for j,x in enumerate(c[:,0]) if x==Coor[i][0]]

y_match=[j for j,y in enumerate(c[:,1]) if y==Coor[i][1]]

POS[i,0]=[val for val in x_match if val in y_match][0]

POS[i,1]=Coor[i][2]

## transform FS to FE nodal forces

def Trans():

F_FE=zeros([len(POS),4])

lon_coord=sort(list(set(POS[:,1])))

mesh_lon_size=lon_coord[1]-lon_coord[0]

for i in range(0,len(POS)):

node=POS[i][0]

Z=POS[i][1]

U=F[4*node+0]

V=F[4*node+1]

W=F[4*node+2]

Sita=F[4*node+3]

# calculating nodal force by integrating force

# notations:

# Z11: left element left node

# Z12: left element right node

# Z21: right element left node

# Z22: right element right node

if Z==0:

Z21=0

Z22=mesh_lon_size

# right element contribution

IntU=(-cos(num_hw*pi/L_t*Z22)+cos(num_hw*pi/L_t*Z21)-num_hw*pi/L_t*(Z22-Z21)*sin(num_hw*pi/L_t*Z21))/(num_hw*pi/L_t)**2/(Z22-Z21)**2

Int=(sin(num_hw*pi/L_t*Z21)-sin(num_hw*pi/L_t*Z22)+num_hw*pi/L_t*(Z22-Z21)*cos(num_hw*pi/L_t*Z21))/(num_hw*pi/L_t)**2/(Z22-Z21)**2

elif Z==L_t:

Z11=L_t-mesh_lon_size

Z12=L_t

# left element contribution

IntU=(cos(num_hw*pi/L_t*Z12)-cos(num_hw*pi/L_t*Z11)+num_hw*pi/L_t*(Z12-Z11)*sin(num_hw*pi/L_t*Z12))/(num_hw*pi/L_t)**2/(Z12-Z11)**2

Int=(-sin(num_hw*pi/L_t*Z11)+sin(num_hw*pi/L_t*Z12)-num_hw*pi/L_t*(Z12-Z11)*cos(num_hw*pi/L_t*Z12))/(num_hw*pi/L_t)**2/(Z12-Z11)**2

else:

Z11=Z-mesh_lon_size

Z12=Z

Z21=Z

Z22=Z+mesh_lon_size

# left element contribution

IntU1=(cos(num_hw*pi/L_t*Z12)-cos(num_hw*pi/L_t*Z11)+num_hw*pi/L_t*(Z12-Z11)*sin(num_hw*pi/L_t*Z12))/(num_hw*pi/L_t)**2/(Z12-Z11)**2

Int1=(-sin(num_hw*pi/L_t*Z11)+sin(num_hw*pi/L_t*Z12)-num_hw*pi/L_t*(Z12-Z11)*cos(num_hw*pi/L_t*Z12))/(num_hw*pi/L_t)**2/(Z12-Z11)**2

# right element contribution

IntU2=(-cos(num_hw*pi/L_t*Z22)+cos(num_hw*pi/L_t*Z21)-num_hw*pi/L_t*(Z22-Z21)*sin(num_hw*pi/L_t*Z21))/(num_hw*pi/L_t)**2/(Z22-Z21)**2

Int2=(sin(num_hw*pi/L_t*Z21)-sin(num_hw*pi/L_t*Z22)+num_hw*pi/L_t*(Z22-Z21)*cos(num_hw*pi/L_t*Z21))/(num_hw*pi/L_t)**2/(Z22-Z21)**2

# sum

IntU=IntU1+IntU2

Int=Int1+Int2

#

F_FE[i][0]=U*IntU;

F_FE[i][1]=V*Int;

F_FE[i][2]=W*Int;

F_FE[i][3]=Sita*Int;

#

return F_FE

## write FE nodal force

def WNF():

trans_dof=[3,1,2,6]

load_text=''

for i in range(0,len(Coor)):

for j in range(0,4):

mag=F_FE[i][j]

dof=trans_dof[j]

load_text+="""Assembly."Column Instance"."""+str(i+1)+','+str(dof)+','+str(mag)+'\n'

return load_text

## create model in abaqus part 2

def Model_2():

# Load

myModel.keywordBlock.synchVersions(storeNodesAndElements=False)

myModel.keywordBlock.insert(32,"""*Cload\n"""+load_text)

myModel.keywordBlock.insert(37,"""*NODE PRINT\nU""")

# Job

import job

mdb.Job(name='mode', model=new_name)

mdb.jobs['mode'].submit()

mdb.jobs['mode'].waitForCompletion()

## output FE nodal forces

def output_NF():

if os.path.isdir('results') == False:

os.mkdir('results')

with open('results/nodal forces '+new_name+'.txt','w') as f:

f.write('X\tY\tZ\tRX\tRY\tRZ\n')

for i in range(0,len(F_FE)):

f.write(str(F_FE[i][1])+'\t'+str(F_FE[i][2])+'\t'+str(F_FE[i][0])+'\t0\t0\t'+str(F_FE[i][3])+'\n')

## output FE nodal displacements

def output_ND():

with open('mode.dat') as f:

flag=0

for i, line in enumerate(f, 0):

if line.find("N O D E   O U T P U T")!=-1:

start_line=i+10

flag=1

elif flag==1 and line.find("MAXIMUM")!=-1:

end_line=i-2

break

with open('mode.dat') as f:

s=f.readlines()

# eliminate overall translation along Z-axis

minU=0

maxU=0

for i in range(start_line,end_line+1):

line_content=s[i].split()

if float(line_content[3])>maxU:

maxU=float(line_content[3])

if float(line_content[3])<minU:

minU=float(line_content[3])

averU=(minU+maxU)/2

if os.path.isdir('results') == False:

os.mkdir('results')

with open('results/nodal displacements '+new_name+'.txt','w') as f:

f.write('X\tY\tZ\tRX\tRY\tRZ\n')

for i in range(start_line,end_line+1):

line_content=s[i].split()

f.write(line_content[1]+'\t'+line_content[2]+'\t'+str(float(line_content[3])-averU)+'\t'+line_content[4]+'\t'+line_content[5]+'\t'+line_content[6]+'\n')

#################### Main #####################

Model_1()

disc()

F=MD()

MN()

F_FE=Trans()

load_text=WNF()

Model_2()

output_NF()

output_ND()

# APPENDIX B. Imperfection modal magnitudes

The imperfection modal magnitudes of section T13014, T15414 (L=1800, L=2500), T7914 (L=1800, L=2500), S11012 are shown in Fig. 1, Fig. 2, Fig. 3, Fig. 4, Fig. 5 and Fig. 6 respectively.

Fig. 1 Imperfection modal magnitudes of section T13014

Fig. 2 Imperfection modal magnitudes of section T15414 (L=1800)

Fig. 3 Imperfection modal magnitudes of section T15414 (L=2500)

Fig. 4 Imperfection modal magnitudes of section T7914 (L=1800)

Fig. 5 Imperfection modal magnitudes of section T7914 (L=2500)

Fig. 6 Imperfection modal magnitudes of section S11012

# APPENDIX C. Total potential energy expression for the initial post-buckling analysis

The expression of the total potential energy in Eq. (8.37) is expanded as shown in Table 1, where only terms associated with second- or fourth-order s are considered, including λQiQj, λRnRp, QiQj, RnRp, RnQiQj and QiQjQkQl , as shown in Eq. (8.66).

Table 1 Expansion of the expression of the total potential energy

| I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | 11. |  | 12. |
| 2 |  |  | 21. |  |  |
| 3 |  |  | 31. |  |  |
| 4 |  |  | 41. |  | 42. |
|  |  |  | 43. |  | 44. |
| 5 |  |  | 51. |  | 52. |
|  |  |  | 53. |  | 54. |
|  |  |  | 55. |  | 56. |
| 6 |  |  | 61. |  |  |
| II |  |  |  |  |  |
| 1 |  |  | 11. |  | 12. |
| 2 |  |  | 21. |  |  |
| 3 |  |  | 31. |  | 32. |
|  |  |  | 33. |  | 34. |
| III |  |  |  |  |  |
| 1 |  |  | 11. |  | 12. |
| 2 |  |  | 21. |  | 22. |
|  |  |  | 23. |  | 24. |
| 3 |  |  | 31. |  | 32. |
|  |  |  | 33. |  | 34. |
| 4 |  |  | 41. |  |  |
| 5 |  |  | 51. |  | 52. |
|  |  |  | 53. |  | 54. |
|  |  |  | 55. |  | 56. |
| 6 |  |  | 61. |  |  |
| IV |  |  |  |  |  |
| 1 |  |  | 11. |  | 12. |
| 2 |  |  | 21. |  | 22. |
| 3 |  |  | 31. |  |  |
| 4 |  |  | 41. |  | 42. |
| 5 |  |  | 51. |  | 52. |
|  |  |  | 53. |  | 54. |
|  |  |  | 55. |  |  |
| 6 |  |  | 61. |  | 62. |
|  |  |  | 63. |  | 64. |
|  |  |  | 65. |  |  |
| V |  |  |  |  |  |
| 1 |  | 11. |  | 12. |  |
| 2 |  | 21. |  | 22. |  |
| 3 |  | 31. |  | 32. |  |
| 4 |  | 41. |  | 42. |  |

Note:

- Terms involved in both local and distortional buckling analysis are coloured in red, while terms involved only in distortional buckling analysis are coloured in black.

- The underlined terms are associated with ∂u0/∂y, and are thus only involved in cases other than ‘uniform compression’ (compression applied by ‘load through centroid’ or bending).

- The crossed out terms are associated with ∂v0/∂x, and thus vanish since ∂v0/∂x = 0.

We define

The terms involved in the ‘uniform compression’ case (neither underlined nor crossed out terms) are calculated and tabulated in Table 2 and Table 3.

Table 2 Multipliers for ‘uniform compression’ terms

|  |  | Coef. 1 | Coef. 2 |  | Int. x | Int. y |
| --- | --- | --- | --- | --- | --- | --- |
| I | 11 | 1 | 1 |  |  |  |
|  | 12 | 1 | 1 |  |  |  |
|  | 21 | 1 | 1/4 |  |  |  |
|  | 31 | 1 | 1/4 |  |  |  |
|  | 41 | 1 | 1 | -1 |  |  |
|  | 42 | 1 | 1 | -1 |  |  |
|  | 43 | 1 | 1 |  |  |  |
|  | 44 | 1 | 2 |  |  |  |
|  | 51 | 1 | 1 | -1 |  |  |
|  | 52 | 1 | 1 | -1 |  |  |
|  | 53 | 1 | 1 |  |  |  |
|  | 56 | 1 | 2 |  |  |  |
|  | 61 | 1 | 1/2 |  |  |  |
| II | 11 | 1 | 1 |  |  |  |
|  | 12 | 1 | 1 |  |  |  |
|  | 21 | 1 | 1 |  |  |  |
|  | 31 | 1 | 1 | ν |  |  |
|  | 32 | 1 | 1 | ν |  |  |
|  | 33 | 1 | 1 |  |  |  |
|  | 34 | 1 | 2 |  |  |  |
| III | 11 | 1 | 2ν |  |  |  |
|  | 12 | 1 | 2ν |  |  |  |
|  | 21 | 1 | 2ν(1/2) | -1 |  |  |
|  | 22 | 1 | 2ν(1/2) | -1 |  |  |
|  | 23 | 1 | 2ν(1/2) |  |  |  |
|  | 24 | 1 | 2ν |  |  |  |
|  | 31 | 1 | 2ν(1/2) | ν |  |  |
|  | 32 | 1 | 2ν(1/2) | ν |  |  |
|  | 33 | 1 | 2ν(1/2) |  |  |  |
|  | 34 | 1 | 2ν |  |  |  |
|  | 41 | 1 | 2ν(1/4) |  |  |  |
|  | 51 | 1 | 2ν(1/2) | ν |  |  |
|  | 52 | 1 | 2ν(1/2) | ν |  |  |
|  | 53 | 1 | 2ν(1/2) |  |  |  |
|  | 56 | 1 | 2ν |  |  |  |
|  | 61 | 1 | 2ν(1/4) |  |  |  |
| IV | 11 | 1 | (1/2)(1- ν) |  |  |  |
|  | 12 | 1 | (1/2)(1- ν) |  |  |  |
|  | 21 | 1 | (1/2)(1- ν) |  |  |  |
|  | 22 | 1 | (1/2)(1- ν) |  |  |  |
|  | 31 | 1 | (1/2)(1- ν) |  |  |  |
|  | 41 | 1 | 2[(1/2)(1- ν)] |  |  |  |
|  | 42 | 1 | 2[(1/2)(1- ν)] |  |  |  |
|  | 53 | 1 | 2[(1/2)(1- ν)] |  |  |  |
|  | 54 | 1 | 2[(1/2)(1- ν)] |  |  |  |
|  | 55 | 1 | 2[(1/2)(1- ν)] |  |  |  |
|  | 63 | 1 | 2[(1/2)(1- ν)] |  |  |  |
|  | 64 | 1 | 2[(1/2)(1- ν)] |  |  |  |
|  | 65 | 1 | 2[(1/2)(1- ν)] |  |  |  |
| V | 11 | t2/12 |  |  |  |  |
|  | 12 | t2/12 |  |  |  |  |
|  | 21 | t2/12 |  |  |  |  |
|  | 22 | t2/12 |  |  |  |  |
|  | 31 | t2/12 | 2ν |  |  |  |
|  | 32 | t2/12 | 2ν |  |  |  |
|  | 41 | t2/12 | 2(1- ν) |  |  |  |
|  | 42 | t2/12 | 2(1- ν) |  |  |  |

We define

Table 3 Results for ‘uniform compression’ terms

|  |  |  | I 11 | IV 11 |
| --- | --- | --- | --- | --- |
|  |  |  | II 11 | IV 21 |
|  |  |  | III 11 | IV 41 |
|  |  |  | V 11 | V 21 |
|  |  |  | V 31 | V 41 |
|  |  |  | III 51 | I 51 |
|  |  |  | I 41 | II 31 |
|  |  |  | III 21 | III 31 |
|  |  |  | V 12 | V 22 |
|  |  |  | V 32 | V 42 |
|  |  |  | I 12 | IV 12 |
|  |  |  | II 12 | IV 22 |
|  |  |  | III 12 | IV 42 |
|  |  |  | I 42 | II 32 |
|  |  |  | III 22 | III 32 |
|  |  |  | I 52 | III 52 |
|  |  |  | I 53 |  |
|  |  |  | III 53 | III 56 |
|  |  |  | I 56 |  |
|  |  |  | I 44 | III 24 |
|  |  |  | IV 54 | IV 55 |
|  |  |  | II 34 | III 34 |
|  |  |  | IV 64 | IV 65 |
|  |  |  | I 43 | III 23 |
|  |  |  | IV 53 |  |
|  |  |  | II 33 | III 33 |
|  |  |  | IV 63 |  |
|  |  |  | I 31 |  |
|  |  |  | III 61 | I 61 |
|  |  |  | I 21 | II 21 |
|  |  |  | III 41 | IV 31 |

# APPENDIX D. Modal shapes

Table 1 Buckling modal shapes

| bw, bf, tf (mm) | tw (mm) | L1 | L2 |
| --- | --- | --- | --- |
| bw = 100, bf = 110, tf = 1 | 1 |  |  |
|  | 1.2 |  |  |
|  | 1.4 |  |  |
|  | 1.6 |  |  |
|  | 1.8 |  |  |
|  | 2 |  |  |
| bw = 100, bf = 120, tf = 1 | 1 |  |  |
|  | 1.2 |  |  |
|  | 1.4 |  |  |
|  | 1.6 |  |  |
|  | 1.8 |  |  |
|  | 2.0 |  |  |
| bw = 100, bf = 130, tf = 1 | 1 |  |  |
|  | 1.2 |  |  |
|  | 1.4 |  |  |
|  | 1.6 |  |  |
|  | 1.8 |  |  |
|  | 2 |  |  |
| bw = 100, bf = 140, tf = 1 | 1 |  |  |
|  | 1.2 |  |  |
|  | 1.4 |  |  |
|  | 1.6 |  |  |
|  | 1.8 |  |  |
|  | 2 |  |  |
| bw = 100, bf = 150, tf = 1 | 1 |  |  |
|  | 1.2 |  |  |
|  | 1.4 |  |  |
|  | 1.6 |  |  |
|  | 1.8 |  |  |
|  | 2 |  |  |

Table 2 Buckling modal shapes

| section | D | L1 | L2 |
| --- | --- | --- | --- |
| CH1-7 |  |  |  |
| CH2-8 |  |  | None |
| CH2-10 |  |  | None |
| CH2-12 |  |  | None |
| CH2-14 |  |  | None |
| H5-2 |  |  |  |
| H5-3 |  |  |  |
| H11-1 |  |  |  |
| H11-2 |  |  |  |
| H11-3 |  |  |  |
| H12-1 |  |  |  |
| H12-2 |  |  |  |
| H12-3 |  |  |  |
| H13-1 |  |  |  |
| H13-2 |  |  |  |
| H13-3 |  |  |  |
| Z2-2 |  |  |  |
| Z2-3 |  |  |  |
| Z5-1 |  |  |  |
| Z5-2 |  |  |  |
| Z5-3 |  |  |  |
| Z8-1 |  |  |  |
| Z8-2 |  |  |  |
| Z8-3 |  |  |  |
| Z11-1 |  |  |  |
| Z11-2 |  |  |  |
| Z11-3 |  |  |  |
| Z12-1 |  |  |  |
| Z12-2 |  |  |  |
| Z12-3 |  |  |  |
| Z13-1 |  |  |  |
| Z13-2 |  |  |  |
| Z13-3 |  |  |  |
| R11-1 |  |  |  |
| R11-2 |  |  |  |
| R11-3 |  |  |  |
| R13-1 |  |  |  |
| R13-2 |  |  |  |
| R13-3 |  |  |  |

# APPENDIX E. Verification of the multi-linear model

Table 1 Verification of the multi-linear model

| bw = 100 mm, bf = 110 mm, tf = 1 mm |  |
| --- | --- |
| tw = 1 mm, L = 316 mm | tw = 1 mm, L = 300 mm |
| tw = 1.4 mm, L = 276 mm | tw = 1.6 mm, L = 254 mm |
| tw = 1.8 mm, L = 241 mm | tw = 2 mm, L = 233 mm |
| bw = 100 mm, bf = 120 mm, tf = 1 mm |  |
| tw = 1 mm, L = 333 | tw = 1.2 mm, L = 314 mm |
| tw = 1.4 mm, L = 292 mm | tw = 1.6 mm, L = 273 mm |
| tw = 1.8 mm, L = 262 mm | tw = 2 mm, L = 254 mm |
| bw = 100 mm, bf = 130 mm, tf = 1 mm |  |
| tw = 1 mm, L = 351 mm | tw = 1.2 mm, L = 331 mm |
| tw = 1.4 mm, L = 309 mm | tw = 1.6 mm, L = 293 mm |
| tw = 1.8 mm, L = 282 mm | tw = 2 mm, L = 275 mm |
| bw = 100 mm, bf = 140 mm, tf = 1 mm |  |
| tw = 1 mm, L = 370 mm | tw = 1.2 mm, L = 349 mm |
| tw = 1.4 mm, L = 329 mm | tw = 1.6 mm, L = 313 mm |
| tw = 1.8 mm, L = 303 mm | tw = 2 mm, L = 296 mm |
| bw = 100 mm, bf = 150 mm, tf = 1 mm |  |
| tw = 1 mm, L = 391 mm | tw = 1.2 mm, L = 368 mm |
| tw = 1.4 mm, L = 348 mm | tw = 1.6 mm, L = 333 mm |
| tw = 1.8 mm, L = 323 mm | tw = 2, L = 316 mm |
| Multi-linear model ABAQUS analysis |  |

Table 2 Verification of the multi-linear model

| CH1-7-400 | CH1-7-600 |
| --- | --- |
| CH1-7-800 | CH2-8-1000 |
| CH2-10-1000 | CH2-12-1000 |
| CH2-14-1000 | H5-2 |
| H5-3 | H8-1 (imp = 0.5t) |
| H8-2 (imp = t) | H8-3 (imp = 0.5t) |
| H11-1 | H11-2 (imp = 0.5t) |
| H11-3 | H12-1 |
| H12-2 | H12-3 |
| H13-1 | H13-2 |
| H13-3 | Z2-2 |
| Z2-3 | Z5-1 |
| Z5-2 | Z5-3 |
| Z8-1 | Z8-2 (imp = 0.5t) |
| Z8-3 | Z11-1 |
| Z11-2 | Z11-3 |
| Z12-1 | Z12-2 |
| Z12-3 | Z13-1 |
| Z13-2 | Z13-3 |
| R11-1 (imp = 0.5t) | R11-2 (imp = 0.5t) |
| R11-3 | R13-1 |
| R13-2 (imp = t) | R13-3 |
| Multi-linear model ABAQUS analysis |  |